3GPP TSG-RAN WG4 Meeting #114	                                                                                            R4-2500641

Athens, GR, 17 Feb –21 Feb, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT CW RF requirements

Agenda item:	7.24.2.3

Document for:	Discussion

- Introduction

A new WI “Solutions for Ambient IoT (Internet of Things) in NR” [1] is approved in RAN#106. The scope of the WI is limited that only device 1 is studied. This contribution provides our views for AIoT CW RF requirements based on the WI scope and the agreements in SI phase [2].

- Discussion

The agreements in SI phase are as following [2],

Issue 2-6: CW for D1T1

Agreement:

- Whether requirements potentially needed or not is summarized in the following table:

| RF Requirement for CW for D1T1 | Requirements potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | TBD in WI phase |
| Frequency error | YES |
| Phase noise | No for inside topology TBD in WI phase for outside topology |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | TBD in WI phase |

In TR 38.769, the potential requirements are categorized as three RF requirements, i.e. output power/Output power dynamic range, Transmit signal quality and RF spectrum emission/Spurious emissions/Phase noise. It seems detail discussion is needed for a clear view. According to the approved WID, carrier wave transmission is for waveform 1 only, without hopping. In TR 38.769, waveform 1 is clarified as a single-tone unmodulated sinusoid, also referred to as 'a single tone'. In the following analysis, the requirements are discussed according to the guidelines in TR 38.769.

2.1 Output power

The exact level for output power requirement should be decided in WI phase. For D1T1-B case 1-4, CW is outside topology. In the co-existence study, the CW output power is assumed as 23 dBm for UL spectrum. Following the co-existence study, it’s reasonable to define the CW output power as 23 dBm. Otherwise, the co-existence simulation needs to be re-conducted.

Proposal 1: The output power for CW output power is defined 23 dBm for R19 WI.

For ON/OFF power, we’re not sure the necessity of this requirement. For the off power, it originally comes from the concerns of the off power noise contribution to the system. In R19 WI, the deployment scenario is indoor then the off power contribution to other system is not severe. On the other hand, if the CW is in off mode, most of the Tx components are switched off, the estimated output power is very low which is far from current BS/UE off power requirement. Considering transient period, currently it applies to BS/UE TDD mode. Some time slot is needed to protect the Rx chain when Tx chain is transmitting power even the output power in the ramping up or ramping down status. For CW node, the motivation to protect Rx chain doesn’t exist. From the timing aspect, it seems RAN1 may not discuss timing relationship of R2D and CW signals. From the above analysis, we didn’t find strong motivation to define ON/OFF power for CW node.

Observation 1: No strong motivation to define ON/OFF power requirement is found for R19 AIoT CW node.

Proposal 2: ON/OFF power or transient period RF requirement for CW mode is not defined.

2.2 Frequency error

CW frequency error impacts the device backscattering signal quality, it should be defined. The exact requirement should consider the impact to the system. Considering the R19 AIoT WI is planed to be closed in 3 RAN group meetings, it may not have time to study the impact to system performance. Following LA BS can be a candidate considering the similar deployment area and the LA BS implementation cost.

Proposal 3: CW frequency error requirements can consider to reuse NR LA BS requirement, i.e. 0.1 ppm.

2.3 Emission/phase noise requirement

The emission requirement usually is defined to protect other system. For R19 AIoT, CW is transmitted on UL spectrum, so the UE emission requirement can be considered. NR UE emission requirements includes SEM, ACLR, spurious emission and transmit intermodulation. For CW node, our understanding is that it’s not necessary to define the out of band emission including SEM, ACLR because the CW signal is single tone. For the general emission requirement, it may be necessary in case any large harmonic interfere other systems. For the spurious emission requirement, NR UE requirement can be considered. The OOB boundary needs further discussion, for example, if the boundary is based on the device channel edge or the system BW channel edge, what’s the separation between the channel edge and the beginning of the spurious emission requirement, etc.

Proposal 4: NR UE spurious emission requirement can be considered for R19 AIoT CW node, the frequency boundary for the emission requirements FFS.

For the phase noise requirement, we didn’t find the strong motivation to define this requirement. Phase noise causes the constellation to rotate about the origin. Phase noise has a significant impact to the high order modulations but R19 AIoT uses OOK or BPSK modulation. In our understanding, even some simulation or analysis can be done to derive phase noise requirement for OOK and BPSK, the requirement should be very relaxed that not necessary to use it to guide the implementation. So our preference is not defining phase noise requirement for CW node at least for R19.

Observation 2: No strong motivation to define CW phase noise requirement is found for R19 AIoT CW node.

- Conclusion

This contribution provides our views for CW node RF requirements. We have the following proposals and observations.

For output power,

Proposal 1: The output power for CW output power is defined 23 dBm for R19 WI.

Observation 1: No strong motivation to define ON/OFF power requirement is found for R19 AIoT CW node.

Proposal 2: ON/OFF power or transient period RF requirement for CW mode is not defined.

For frequency error,

Proposal 3: CW frequency error requirements can consider to reuse NR LA BS requirement, i.e. 0.1 ppm.

For emission/phase noise,

Proposal 4: NR UE spurious emission requirement can be considered for R19 AIoT CW node, the frequency boundary for the emission requirements FFS.

Observation 2: No strong motivation to define CW phase noise requirement is found for R19 AIoT CW node.

- References

[1] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106

[2] R4-2420470, “WF on RF requirements for A-IoT”, RAN4#113, Huawei

[3] TR 38.769