# 3GPP TSG-RAN WG4 Meeting #114	R4-2500642

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.11.3

Source: 	Xiaomi

Title: 	System parameters for Ku band

Document for:	Discussion

1		Introduction

For Rel-19 NTN Ku Band introduction, the suitable numerologies e.g., FR1 like or FR2 like are key issue. During previous RAN4 discussion, companies realized both FR1 numerologies and FR2 numerologies have pros and cons in terms of spectrum usage efficiency, UL timing synchronization, phase noise, beam hopping granularity and link budget UL/DL.  It’s hard to make decision on numerologies selection based on above decision criteria. Both FR1 and FR2 numerologies are feasible for the deployment.

In RAN4#113, RAN4 failed to meet the RAN-P demand to make down-selection between FR1 like or FR2 like numerologies due to the diverse demand from NTN operators for the deployed bandwidths with the range from 30MHz to 250MHz which are across FR1 channel bandwidth set and FR2 channel bandwidth set. RAN4 make following recommendation to RAN-P in WF [2], with corresponding LS to RAN1/RAN2 cc RAN-P in [3] for the potential impact to RAN1/RAN2:

# WF for NTN Ku band System parameters - Sub-topic 3-9: FR1 FR2 down selection Overall observation is that both FR1 numerology and FR2 numerology are feasible. There are use cases that favor either FR1 numerology or FR2 numerology Some of the disadvantages of numerology combinations can be mitigated with implementation choices WF on numerology: Agree to define two sets of bands, one for FR1 numerology and the other for FR2 numerology. RAN4 will further discuss SCS options.

In RAN#106, the situation maintained same as RAN4 discussion with different use cases and different demand from satellite industry on Ku band usage which captured in WF [4]. It’s agreed to revise the WID including both numerologies with two sets of bands meanwhile also pending on the feedback from RAN1/RAN2 with check point in RAN#107:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

On the band definition, in last RAN4 meeting, two candidate bands are agreed as following:

Table X: Band definitions for Ku band

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| nX(Note 1) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| nY(Note 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25. |  |  |  |

In this contribution, we discussed the system parameters for both FR1 and FR2 numerologies with two sets of bands following RAN-P decision.

2		Discussion

Band definition and frequency range

Target frequency range on Ku bands as following from 10.7GHz to 14.5GHz

- For ITU regions 1 and 3:

  - Downlink 10.70 – 12.75 GHz

  - Uplink 13.75-14.5 GHz

- For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision):

  - Downlink 10.70 – 12.70 GHz

  - Uplink 13.75-14.5 GHz

- For ITU regions 1 and 3

  - Downlink 10.70 – 12.75 GHz

  - Uplink 12.75 - 13.25 GHz

- For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision):

  - Downlink 10.70 – 12.70 GHz

  - Uplink 12.70 - 13.25 GHz

Refer to TS 38.101-5, existing NTN-FR1 and NTN-FR2 are specified as following:

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 7125 MHz |
| FR2-NTN (Note 2) | 17300 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications. |  |

The target frequency range of Ku band(s) is out of existing FR1-NTN and FR2-NTN frequency ranges. In existing RAN1/RAN2 specifications, there are many places refer to the reference as FR1 and FR2-1 bands, or TDD-FR1 bands, TDD-FR2-1 bands and TDD-FR2-2.  In order to avoid the impact to other WGs (RAN1 and RAN2), RAN4 shall introduce Ku bands in existing frequency range designation instead of introducing new frequency ranges.

In order to cover Ku bands without introducing new frequency ranges, the frequency range of existing FR1-NTN and FR2-NTN need to be extended with overlapping frequency ranges.

To avoid confusion and impact to other WGs on these overlapping frequency between two frequency range designation, additional notes need to be added to clarify NTN bands with band numbering below or equal to 256 treated as FR1 band, and NTN bands with band numbering between 257 to 512 as FR2 bands.

Proposal 1: Extending existing NTN-FR1 and NTN-FR2 frequency ranges to cover Ku bands with overlapping frequency ranges.

Proposal 2: Define separate bands/band numbers for the same frequency ranges of Ku bands as NTN-FR1 band with associated FR1 numerology and NTN-FR2 bands with FR2 numerology.

Proposal 3: Clarify NTN bands with band numbering below or equal to 256 treated as FR1 band, and NTN bands with band numbering between 257 to 512 as FR2 bands. (Detailed proposed changes as following)

Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range with band numbering below or equal to 256 are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range with band numbering from 257 to 512 are regarded as a FR2-1 band when references from other specifications. |  |

CHBW, SCS and spectrum utilization

According to the agreed WID, RAN4 target to define two sets of bands with FR1-numberologies and FR2-numoerlogies as following:

- Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing

- Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing.

We believe above two sets of CHBW/SCS already meet the demand from NTN satellite operators. Some companies mentioned to introduce additional 15kHz SCS (FR1) and 60kHz SCS (FR2) to allow some flexibility.

For existing commercial FR1 TN deployment, 30kHz is widely deployed for above 2.6GHz FR1 frequency ranges. Furthermore, 30kHz SCS is more robust to PN noise and high doppler shift scenario (considering satellite motion and mobile VSAT scenario).

For FR2 numerology, 120kHz SCS is widely deployed and commercialized which also bring benefit to avoid mixed numerologies on SSB and data transmission. On the concern on UL timing synchronization issue, we believe it’s already resolved in previous RAN4 agreements on updating Te_NTN value from 10*64*Tc to  7.5*64*Tc (CR R4-2420236).

RAN4#113 RRM session agreements: Agreement: Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR2-NTN; 120; 60; 13*64*Tc. 120; 7.5*64*Tc. 240; 60; 13*64*Tc. 120; 7.5*64*Tc. Note 1: Tc is the basic timing unit defined in TS 38.211 [6] Note 2: The value of Te_NTN for 120 kHz uplink SCS assumes LOS, and GNSS signal power level above [TBD] dBm [and max acceleration of [TBD] m/s2], FFS other side conditions if needed.

With above analysis, we propose to stick to RAN-P agreement without introducing additional SCSs given no technical benefits foreseen and simplification of numerologies have benefits to reduce RAN4 workload and test effort.

Proposal 4: Limit to 30kHz SCS (data and SSB) for FR1 Ku band(s) and 120kHz SCS (data and SSB) for FR2 Ku band(s).

Proposal 5: On detailed channel bandwidth sets including spectrum utilization, guard band reusing existing values from TN specifications for following channel bandwidths:

- FR1 NTN Ku Band(s)- 30kHz: 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz, and 100 MHz

- FR2 NTN Ku Band(s)- 120kHz: 50 MHz, 100 MHz, 200 MHz, 400 MHz

Sync raster and channel raster

On channel raster, we believe existing SCS based channel raster can be reused on FR1 Ku band (s) and FR2 Ku band (s).

Proposal 6: For FR1 NTN Ku Band(s), 30kHz channel raster applied and for FR2 NTN Ku Band(s), 120kHz channel raster applied.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| nX | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| nY | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1246666 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

Table 5.4.2.3-3: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2084999 | 1553336 – <8> – 1746664 |
| nX | 120 | 1333334 – <8> – 1366666 | 1113334 – <8> – 1250000 |
| nY | 120 | 1333334 – <8> – 1366666 | 1113334 – <8> – 1246666 |

On sync raster, we believe that introducing 120kHz SCS SSB on FR2 and 15kHz SCS SSB for FR1 is sufficient.

Proposal 7: For FR1 NTN Ku Band(s), 30kHz SSB applied and for FR2 NTN Ku Band(s), 120kHz SSB applied.

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| nX | 30 kHz | Case C | 12849 – <1> – 14267 |
| nY | 30 kHz | Case C | 12849 – <1> – 14232 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| nX | 120 kHz | Case D | 12857 – <12> – 14259 |
| nY | 120 kHz | Case D | 12857 – <12> – 14225 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

3		Conclusion

In this contribution, views on system parameters for Ku band were provided.

Proposal 1: Extending existing NTN-FR1 and NTN-FR2 frequency ranges to cover Ku bands with overlapping frequency ranges.

Proposal 2: Define separate bands/band numbers for the same frequency ranges of Ku bands as NTN-FR1 band with associated FR1 numerology and NTN-FR2 bands with FR2 numerology.

Proposal 3: Clarify NTN bands with band numbering below or equal to 256 treated as FR1 band, and NTN bands with band numbering between 257 to 512 as FR2 bands. (Detailed proposed changes as following)

Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN (Note 1) | 410 MHz – 14500MHz |
| FR2-NTN (Note 2) | 10700 MHz – 30000 MHz |
| NOTE 1: NTN bands within this frequency range with band numbering below or equal to 256 are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range with band numbering from 257 to 512 are regarded as a FR2-1 band when references from other specifications. |  |

Proposal 4: Limit to 30kHz SCS (data and SSB) for FR1 Ku band(s) and 120kHz SCS (data and SSB) for FR2 Ku band(s).

Proposal 5: On detailed channel bandwidth sets including spectrum utilization, guard band reusing existing values from TN specifications for following channel bandwidths:

- FR1 NTN Ku Band(s)- 30kHz: 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz, and 100 MHz

- FR2 NTN Ku Band(s)- 120kHz: 50 MHz, 100 MHz, 200 MHz, 400 MHz

Proposal 6: For FR1 NTN Ku Band(s), 30kHz channel raster applied and for FR2 NTN Ku Band(s), 120kHz channel raster applied.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| nX | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1250000 |
| nY | 30 | 1333334 – <2> – 1366666 | 1113334 – <2> – 1246666 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

Proposal 7: For FR1 NTN Ku Band(s), 30kHz SSB applied and for FR2 NTN Ku Band(s), 120kHz SSB applied.

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| nX | 30 kHz | Case C | 12849 – <1> – 14267 |
| nY | 30 kHz | Case C | 12849 – <1> – 14232 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| nX | 120 kHz | Case D | 12857 – <12> – 14259 |
| nY | 120 kHz | Case D | 12857 – <12> – 14225 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

Reference

[1] RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

[2] R4-2419909, Way Forward for [112][314] NR_NTN_Ku_Band_General, Eutelsat

[3] R4-2419902, LS on Ku band numerology, Eutelsat, TO: RAN1, RAN2, CC: RAN-P

[4] RP-243301, Way Forward on Ku band numerology, Intelsat, Thales, Eutelsat Group, European Space Agency, FirstNet, Bosch, Novamint, SyncTechno Inc, Fraunhofer HHI, Fraunhofer IIS, BMW, Gilat, CHTTL, SES, Airbus, EDF, Sharp, Hispasat, Lockheed Martin, UIC