# 3GPP TSG-RAN WG4 Meeting #114	R4-2500644

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.8.2

Source: 	Xiaomi

Title: 	LB (FDD) – LB (SDL) CA with switching

Document for:	Discussion

- Introduction

In last RAN-P, a new WI was agreed to enable LB-LB CA with swicthing in [1]. The motivication of this WI was to efficiently use the SDL spectrum together with low FDD band via CA by swicthing to eliminate UE implemenation channllenge  of supporting co-current transmission/reception between these two carriers due to antenna performance with wide frequnecy separation (fractional bandwith), and or duplexer design w/o sufficient frequency gap between DL and UL.

Detailed WID is reproduced here for reference:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In this contribution, we analyze the impact to UE RF requirements to enable LB-LB CA via switching.

- Discussion

Based on the agreed WID, it’s agreed to focus on the scope as following:

- Focus on FDD-SDL band with semi-static switching pattern

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

  - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier

  - Focus on collocated scenario with single TAG and aligned SCS 15kHz only

- Example band combination: band n5 + SDL band n29

RAN4 target to specify time mask and required switching delay for the transition between two carriers as case 1 to case 2 or vice versa w/o simultaneous Tx/Rx between the PCell and the SCell.

The required transient period from UE transceiver chain switching between carrier 1 and carrier is depending on UE architecture which can be categorized as following:

- Partially shared receiver chain: UE share RF front-end including antenna and LNA with separate LO/Mixer, LBF between two carriers

  - UE needs to switch duplex in front-end between transient period with necessary RF retuning as well

- Fully shared receiver chain: UE share whole receiver chain from antenna to ADC

  - UE needs to switch duplex in front-end between transient period with necessary RF retuning for mixer/LO and other analog components as well

- Separate receiver chain: for band combination of band n5 + SDL band n29, if separate receiver chain including antenna used than co-current reception between two carriers can be deployed which is already specified in existing specification as normal band combination CA_n5A-n29A.

Proposal 1: Only consider partially shared receiver chain and fully shared receiver chain as reference architectures to decide switching delay and time mask for carrier switching on example band combos n5_n29.

For the required time on RF returning among two carriers and location of switching period, existing switching period are specified for UL Tx switching can be reused as starting point. In Rel-16 UL Tx switching between two carriers for inter-band UL CA, the required switching period is specified as UE capability with candidate values {35us, 140us, 210us} to accommodate different UE implementation.

Proposal 2: Introduce UE capability with multiple values for switching period to accommodate different UE implementation.

- Reusing existing values from UL Tx switching {35us, 140us, 210us} as baseline assumption

Regarding detailed time mask, in existing specification RAN4 specify time mask for uplink carrier switching mask for different cases. The switching command can be RRC based, MAC-CE or DCI based as specified in TS 38.214 for uplink Tx switching and switching location are indicated by RRC uplinkTxSwitchingPeriodLocation. The switching mask requirements for inter-band UL CA with 2 carriers are described in Figure 6.3A.3.3.2-1a and Figure 6.3A.3.3.2-1b either on carrier 1 or carrier 2 based on RRC indication.

Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1 Figure 6.3A.3.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

During the transient period, RF returning will interrupt both downlink reception and uplink transmission across two carriers which corresponding to RAN4 uplink transmission time mask requirements in RF specification TS 38.101-1 and DL interruption requirements specified in RRM specification TS 38.133.

For time mask requirements, we can leverage the work from Rel-16 UL Tx switching as much as possible meanwhile some differences are highlight below:

- On FDD-SDL band combination, no uplink transmission on SDL carrier: the switching happed between carrier 1 (Pcell, FDD carrier Tx/Rx) and carrier 2 (Scell, SDL carrier RX only)

- No con-current transmission and reception between these 2 carriers (for UL Tx switching, simultaneous reception and/or transmission across two carriers allowed pending on band combinations)

- Switching scheme: In Rel-19 only focus on semi-static switching pattern based on RRC configuration

For the analysis simplicity, here we take carrier 1 as FDD carrier (Pcell) and carrier 2 as SDL carrier (Scell). Depending on switching time location on carrier 1 or carrier 2, the corresponding impact to time mask and DL interruption requirements is different (the analysis on interruption impact in companion contribution [2]):

- If switching time always out of Pcell transmission slots, then general time mask with some editorial update can be applied as described in figure below

Figure 1: Time mask for LB(FDD)-LB(SDL) switching on Pcell/carrier 1 with switching period out of UL carrier 1/Pcell

- If switching time always located on Pcell/UL carrier than the switching period will bring impact to Pcell transmission slots, then general time mask with some update can be applied as described in figure below

Figure 2: Time mask for LB(FDD)-LB(SDL) switching on Pcell/carrier 1 with switching period on UL carrier1/Pcell

Proposal 3: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL)

Proposal 4: Specify separate time mask based on switching period location out of Pcell or on Pcell as described in figure 1 and figure 2.

- Conclusion

In this contribution, we analyze the impact to UE RF requirements to enable LB-LB CA via switching.

Proposal 1: Only consider partially shared receiver chain and fully shared receiver chain as reference architectures to decide switching delay and time mask for carrier switching on example band combos n5_n29.

Proposal 2: Introduce UE capability with multiple values for switching period to accommodate different UE implementation.

- Reusing existing values from UL Tx switching {35us, 140us, 210us} as baseline assumption

Proposal 3: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL)

Proposal 4: Specify separate time mask based on switching period location out of Pcell or on Pcell as described in figure 1 and figure 2.

Figure 1: Time mask for LB(FDD)-LB(SDL) switching on Pcell/carrier 1 with switching period out of UL carrier 1/Pcell

Figure 2: Time mask for LB(FDD)-LB(SDL) switching on Pcell/carrier 1 with switching period on UL carrier1/Pcell

Reference

[1] RP-243317, New WID on low band carrier aggregation via switching, AT&T TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE

[2] R4-2500645, RRM requirements on LB-LB CA via switching