# 3GPP TSG-RAN WG4 Meeting #114	R4-2500645

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.8.2

Source: 	Xiaomi

Title: 	LB (FDD) – LB (SDL) CA with switching

Document for:	Discussion

- Introduction

In last RAN-P, a new WI was agreed to enable LB-LB CA with swicthing in [1]. The motivication of this WI was to efficiently use the SDL spectrum together with low FDD band via CA by swicthing to eliminate UE implemenation channllenge  of supporting co-current transmission/reception between these two carriers due to antenna performance with wide frequnecy separation (fractional bandwith), and or duplexer design w/o sufficient frequency gap between DL and UL.

Detailed WID is reproduced here for reference:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In this contribution, we analyze the impact to DL interruption time to enable LB-LB CA via switching.

- Discussion

Based on the agreed WID, it’s agreed to focus on the scope as following:

- Focus on FDD-SDL band with semi-static switching pattern

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

  - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier

  - Focus on collocated scenario with single TAG and aligned SCS 15kHz only

- Example band combination: band n5 + SDL band n29

RAN4 target to specify time mask and required switching delay for the transition between two carriers as case 1 to case 2 or vice versa w/o simultaneous Tx/Rx between the PCell and the SCell.

For the required time on RF returning among two carriers and location of switching period, existing switching period are specified for UL Tx switching can be reused as starting point. In Rel-16 UL Tx switching between two carriers for inter-band UL CA, the required switching period is specified as UE capability with candidate values {35us, 140us, 210us} to accommodate different UE implementation.

During the transient period, RF returning will interrupt both downlink reception and uplink transmission across two carriers which corresponding to RAN4 uplink transmission time mask requirements in RF specification TS 38.101-1 and DL interruption requirements specified in RRM specification TS 38.133.

Proposal-1: The switching period X us pending UE RF session decision.

For DL interruption requirements, we can leverage the work from Rel-16 UL Tx switching as much as possible meanwhile some differences are highlight below:

- On FDD-SDL band combination, no uplink transmission on SDL carrier: the switching happed between carrier 1 (Pcell, FDD carrier Tx/Rx) and carrier 2 (Scell, SDL carrier RX only)

- No con-current transmission and reception between these 2 carriers (for UL Tx switching, simultaneous reception and/or transmission across two carriers allowed pending on band combinations)

- Switching scheme: In Rel-19 only focus on semi-static switching pattern based on RRC configuration

For the analysis simplicity, here we take carrier 1 as FDD carrier (Pcell) and carrier 2 as SDL carrier (Scell). Depending on switching time location on carrier 1 or carrier 2, the corresponding impact to time mask and DL interruption requirements is different (the analysis on time mask impact in companion contribution [2]). The expected interruption on two carriers as described in below figures:

Refer to Rel-16 UL transmission discussion, the interruption length is specified following:

- Consider the interruption uncertainty due to MRTD and TA adjustment accuracy, where MRTD=3us.

- Interruption length is expressed by: ceil(switching period+2*TA adjustment uncertainty+2*MRTD-CP length)/symbol duration)+1

- 1 additional symbol was added due to timing offset between UL carrier and DL carrier

As explained above, in LB (FDD) -LB CA(SDL) via switching scenario, only Pcell has UL carrier with single TAG and co-located assumption.

Then above Rel-16 UL transmission DL interruption equation can be further simplified as following:

Proposal 2: Interruption length of LB(FDD)-LB(SDL) via switching can be specified based on following equation

- Interruption length of LB(FDD)-LB(SDL) via switching = ceil((switching period+TA adjustment uncertainty+ MRTD-CP length)/symbol duration)+1

- TA adjustment uncertainty = 130ns, for collocated scenario MRTD = 3us

Proposal 3: The DL interruption starts from the first OFDM symbol which fully or partially overlap with the UL switching period

Assuming the switching period same as Rel-16 UL Tx switching, then expected interruption as shown in below table for example.

DL interruption length on NR carrier(s) in the unit of OFDM symbols (X) for switching between two carriers (LB (FDD) – LB(SDL))

|  | NR Slot length (ms) | Carrier switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35µs | 140µs | 210µs |
| 0 | 1 | 2 | 3 | 4 |

- Conclusion

In this contribution, we analyze the impact to DL interruption time to enable LB-LB CA via switching.

Proposal-1: The switching period X us pending UE RF session decision.

Proposal 2: Interruption length of LB(FDD)-LB(SDL) via switching can be specified based on following equation

- Interruption length of LB(FDD)-LB(SDL) via switching = ceil((switching period+TA adjustment uncertainty+ MRTD-CP length)/symbol duration)+1

- TA adjustment uncertainty = 130ns, for collocated scenario MRTD = 3us

Proposal 3: The DL interruption starts from the first OFDM symbol which fully or partially overlap with the UL switching period.

Reference

[1] RP-243317, New WID on low band carrier aggregation via switching, AT&T TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE

[2] R4-2500644, RF requirements on LB-LB CA via switching, Samsung

[3] R4-200863, WF on RRM requirements for Tx switching between two uplink carriers, Huawei