# 3GPP TSG-RAN WG4 Meeting #114	R4-2500646

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.1.1.2.2

Source: 	Xiaomi

Title: 	MPR applicability for FR1 intra-band UL CA

Document for:	Discussion

- Introduction

RAN4 discussed MPR applicability for FR1 intra-band UL CA including non-contiguous and contiguous; which almost resolved all open issue with concrete conclusion in last RAN4 meeting as following [1]:

# Topic #2: MPR applicability for FR1 intra-band UL CA ### Sub-topic 2-1: Intra-band contiguous UL CA Sub-topic description Open issues and candidate options before meeting: #### Issue 2-1-1: Applicable emission requirements for PC2/PC3 intra-band contiguous CA with single activated cell Agreement in main session: - ULCA emission requirements (SEM/ACLR/SE) based on the aggregated BW should apply, independent of the dualPA IE indication. #### Issue 2-1-2: Applicable emission requirements for PC1.5 intra-band contiguous CA with single activated cell Agreement - Drop the discussion, as it was already discussed and agreed in last meeting that to further discuss handling of PC1.5 once the UL CA requirements are available ### Sub-topic 2-2: Intra-band non-contiguous CA #### Issue 2-2-1: Applicable MPR and corresponding spectrum requirements for PC2/PC3 intra-band NC CA Agreement in main session: - No change of applicable MPR requirement in current spec but with clarification that applicable emission requirements are composite based regardless of dualPA IE indication. - FFS on whether update MPR for Single CC Fallback from Non-contiguous ULCA without dual PA support depending on UE capability. - Note: Composite emissions means combination of individual sub-block emissions.

In this controbution, views on remaining open issues are provided.

2		Discussion

Sub-topic 2-1: Intra-band contiguous UL CA

In last RAN4 meeting, companies also proposed to consider additional scenarios as captured in moderator summary in issue 2-1-3.

#### Issue 2-1-3: Others - Proposals - Proposal 1: The granularity of Rel-19 UE capability on FR1 intra-band UL CA MPR improvement should be per BC. Whether separate capabilities for contiguous and non-contiguous case can be FFS. (Huawei) - Proposal 2: MPR applicability and enhancement in the cases of more than one active cell with time non-overlapping UL transmission should be considered also in Rel-19 if time allows. (Nokia)

In our view, considering other scenarios besides of single activated cell case belongs to up scoping. The scenario with time non-overlapping UL transmission (non co-current UL transmission) is valid scenario which pending on network scheduling i.e., NW can schedule UL transmission for different carriers with time shift under same TDD pattern and this can be dynamic changed as per TTI basis.  From UE processing perspective, the scenario is different compared to cell active/deactivate case. During Cell status transition from active to deactivate or vise versa, UE always needs some interruption time for RF returning, AGC resetting and baseband update.  UE cannot adjust RF front-end based UL transmission scheduling if both cells activated as the UL scheduling can be per TTI basis. With above consideration, the conclusion for MPR applicability and enhancement in the case with single activated cell can’t directly apply to non-co-current UL transmission scenario with multiple active cells.

Proposal 1: Focus on single active cell case only for MPR applicability for FR1 intra-band UL CA in Rel-19 UE RF WI.

For UE capability, we believe one additional UE capability/feature shall be introduced on MPR enhancement for Intra-band contiguous UL CA given we change the MPR applicability rule, and this capability shall be per BC basis as optional feature.

For intra-band non-contiguous CA, FFS whether new UE capability/feature shall be introduced or not. In our view if no new MPR applicable rule introduced, then there is no need to introduce new UE capability/feature.

Proposal 2:  Introduce new UE capability/feature for MPR applicability for FR1 intra-band UL contiguous CA as optional feature as per BC basis.

Proposal 3: No need to introduce new UE capability/feature for FR1 intra-band UL non-contiguous CA if no change on existing MPR applicability.

Sub-topic 2-2: Intra-band non-contiguous CA

For intra-band non-contiguous CA, it’s already agreed that no change on existing MPR applicability given it’s already referred to single CC MPR with single activated UL cell.

Some companies proposed to optimize MPR for single CC Fallback from Non-contiguous ULCA with finer granularity of B. In our view, this will be time consuming with measurement/simulation campaign and the expected work is not trivial.

According to the WID as reproduced below, RAN4 target to discuss MPR applicability for FR1 intra-band UL CA.  In our view, update/replace existing MPR values not belongs to current Rel-19 UE RF WI scope.

- Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated

Proposal 4: Not consider to update MPR for Single CC Fallback from Non-contiguous ULCA in Rel-19 UE RF WI.

3		Conclusion

In this controbution, views on remaining open issues are provided.

Proposal 1: Focus on single active cell case only for MPR applicability for FR1 intra-band UL CA in Rel-19 UE RF WI.

Proposal 2:  Introduce new UE capability/feature for MPR applicability for FR1 intra-band UL contiguous CA as optional feature as per BC basis.

Proposal 3: No need to introduce new UE capability/feature for FR1 intra-band UL non-contiguous CA if no change on existing MPR applicability.

Proposal 4: Not consider to update MPR for Single CC Fallback from Non-contiguous ULCA  in Rel-19 UE RF WI.

Reference

[1] RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei

[2] R4-2418138, Topic summary for [113][116] NR_ENDC_RF_Ph4_part1, Moderator (Huawei)

[3] R4-2420527, WF on Power domain enhancement, Huawei