# 3GPP TSG-RAN WG4 Meeting #114	R4-2500648

# Athens, GR, 17th – 21st February, 2025

Agenda Item:			7.10.2.31

Source: 	Xiaomi

Title: 						RF requirements for IoT-NTN

Document for:	Discussion

- Introduction

Rel-19 NTN agreed to introduce HPUE for NR-NTN/Iot-NTN operation:

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, views on remaining open issues were provided.

- Discussion

ACLR requirements

Issue 1-1-2: PC1/2 ACLR should not below PC3 requirements, based on the assumption that PC3 requirement can be relaxed if necessary. - Proposals - Option 1: Yes. - Option 2: No - Option 3: Others Agreement: - HPUE ACLR should not be relaxed than that for PC3 for NB-IoT NTN.

In last RAN4 meeting, initial conclusion was reached for co-existence study as following:

Issue 1-8 to Issue 1-10: Required ACIR/ACLR for from co-ex study Agreement: Based on the results collected till RAN4#113, observations as following for the required ACLR from co-existence study evaluation perspective: For IoT-NTN PC2: - Less than 21.1 dB with 1.5km isolation distance assumed; - Less than [20.3] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For IoT-NTN PC1: - Less than 21.9 dB with 1.5km isolation distance assumed; - Less than [24.2] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case

On isolation distance, we prefer to align with previous assumption the deployment scenario and restriction shall be consistency independent to device power classes which belongs to UE capability.

On NTN operation, the critical issue is coverage due to long distance between satellite over the space to terminal. With reasonable ACLR, it will bring benefits to improve uplink coverage with less MPR and or A-MPR.

Based on above consideration, we suggest to specify following ACLR values:

Proposal 1: Specify IoT -NTN HPUE ACLR as following (relaxing PC3 requirements if necessary)

- PC2: 23 dBc

- PC1: 23 dBc

Issue 1-1-4: SEM relaxation for the case UE CBW is much narrower compared to BS CBW - Proposals - When UE PC2 ACLR is decided, regarding SEM for IoT-NTN HPUE, if no specific concern, consider relaxing UE SEM requirements within SAN CBW but still meet ITU SEM regulatory requirements. - WF - FFS whether MPR reduction technique currently discussed in Rel-19 can be considered here

In our view, it’s worth to study other candidate power domain enhancement e.g.  emission requirements in additional to boost the transmission power with advanced PA meanwhile we also realize there is a parallel discussion in Rel-19 UE RF WI for the power domain enhancement with emission requirements enhancement under TN operation including NR and IoT.  Clearly, it’s out of scope of existing Rel-19 NTN WI, and we can wait for the conclusion in TN and consider to leverage relative work to NTN operation in later release.

Proposal 2: Not consider SEM relaxation for the case UE CBW is much narrower compared to BS CBW as it’s out of the existing WI scope and which can be further considered in future release pending on the conclusion from TN operation.

Target device types and device type assumption

Issue 1-2-1: Discuss the unsettled requirements applicability for Iot-NTN: - Agreement (Ad Hoc): - No 2Tx PC1/PC2 requirement will be specified for IoT NTN in Rel-19; - Only consider non-handheld for PC1 1Tx eMTC based with full duplex Iot-NTN - FFS whether requirements should be defined for PC1 1Tx handheld UE for NB-Iot based Iot-NTN Updated Status: (For Information) Table: Current status for requirements consideration and feasibility for Iot-NTN Power Class; Antenna Numbers; Handheld and/or non-handheld. NB-Iot based; PC1; 1Tx; FFS. PC2; 1Tx; Both. eMTC based with half duplex; PC1; 1Tx; FFS. PC2; 1Tx; Both. eMTC based with full duplex; PC1; 1Tx; Non-handheld. PC2; 1Tx; Both.

In general, we prefer to leverage existing TN network design on FDD bands as much as possible with following thinking:

- Harmonize the common design between TN and NTN operation for echo-system

- Shared RF chain between NTN and TN operation on similar frequency range for handheld device for cost and form factor limitations

- Reduce RAN4 workload on leverage the conclusion from TN to NTN

Some companies proposed to consider GSM PA for supporting PC1. However, IoT devices may not support GSM operation and using GSM PA with bring additional cost for IoT device types.  We suggest to focus on non-hand held device types on supporting PC1 on IoT NTN operation.

Proposal 3: Only focus on non hand-held device type for PC1 IoT NTN including NB-IoT, eMTC with half-duplex, and eMTC with full duplex in Rel-19

- Conclusion

In this contribution, views on remaining open issues were provided.

Proposal 1: Specify IoT -NTN HPUE ACLR as following (relaxing PC3 requirements if necessary)

- PC2: 23 dBc

- PC1: 23 dBc

Proposal 2: Not consider SEM relaxation for the case UE CBW is much narrower compared to BS CBW as it’s out of the existing WI scope and which can be further considered in future release pending on the conclusion from TN operation.

Proposal 3: Only focus on non-hand-held device type for PC1 IoT NTN including NB-IoT, eMTC with half-duplex, and eMTC with full duplex in Rel-19.

Reference

[1] R4-2420403, WF on NR NTN HPUE, Samsung