3GPP TSG-RAN WG4 Meeting#114		R4-2500651

Athens, GR, Feb 17th -21st, 2025

Agenda item:	7.26.2.1

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on systems parameters for LP-WUR

Document for:	Discussion

Introduction

In RAN4#113meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

Channel raster for LP-WUR

Agreements: - No new channel raster is introduced dedicated for LP-WUS/WUR. - FFS on whether there is need of the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost

According to the RAN1’s agreements, in RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. According to the RAN2’s LS [2], many companies raised concerns that if LP-Receivers support only a subset of the bands supported by the main receiver and if LP-WUS, Paging and RACH are executed in the LP-WUS bands, it might lead to overload of these bands if the NW set the LP-WUS frequency as a higher priority. At the same time, at the RAN#106 meeting, RAN plenary to task RAN1/2/4 to continue discussion to address the operator concerns associated with the UE not supporting LP-WUS reception on all the bands supported by the MR, and related solutions have been mentioned. Based on the above analysis, from the perspective of RAN4, there is no need for the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost. If LR supports the same bands as MR, there is no need to consider the above solutions. If the LR supports limited frequencies, it still has related solutions to make sure LP-WUS functions. In our understanding, it is up to UE implementation, there is no need for the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost.

Proposal 1: It is up to UE implementation, there is no need for the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost.

Conclusions

In this paper, we provide our views on system parameters for LP-WUR, we have made the following proposals and observations.

Proposal 1: It is up to UE implementation, there is no need for the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost.

- Reference

- R4-2420528, WF on LP-WUS UE RF requirements, vivo, TSG-RAN WG4 113meeting

- R2-2410957,Relpy LS to R2-2407921/R1-2407559, TSG –RAN WG2 128meeting

- RP-242656, On LR and MR operating in different bands in release 19 , Vodafone, vivo and Huawei, HiSilicon TSG RAN meeting #106