3GPP TSG-RAN WG4 Meeting # 114                                    R4-2500698

Athens, GR, 17th – 21st February, 2025

Agenda item:	 7.4.3

Source:        ZTE Corporation, Sanechips

Title:           Discussion on RRM impacts for less than 5MHz BW

Document for:  Approval

Introduction

The work item on NR support for dedicated spectrum less than 5MHz for FR1 was approved at RAN#103. according to the WID, RAN4 shoud define RRM requirements for inter-band CA and DC for combinations introduced in RF part and main RAN4 scopes are summarized as following:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

In this contribution, we will provide our views on RRM impacts for less than 5MHz BW.

Discussion

As per the WID in Rel-19, CA and DC are supported and the RRM requirements maybe impact due to introduction of NR in less than 5 MHz bandwidth. RANP agreed to use a PBCH transmission bandwidth is 20 PRBs for 5MHz channel bandwidth and a PBCH transmission bandwidth is 12 PRBs for 3MHz channel bandwidth. For the case that CBW equals to 3MHz, the transmission bandwidth for PBCH is smaller and RAN4 should further discuss how to update relative requirements.In last meeting, we have reached agreements on many issues[1].

  - SCell configured with less than 5MHz bandwidth

In RAN4#112bis meeting, we have discussed the RRM scope for SCell configured with less than 5MHz bandwidth. And there is a remain issue on whether to consider intra-band CA in R19, copy as follows.

Issue 1-4-1: EMR requirements: - Proposals - Option 1: RAN4 to define EMR requirements for target cell with 12RB PBCH. Delay requirements for SSB index reading in EMR is extended by [3] samples. - Option 2: No changes on top of rel-18 EMR measurement reporting are needed. Agreement: - RAN4 to define EMR requirements for target cell with 12RB PBCH, and the delay requirements for SSB index reading in EMR is extended by [3] more samples.

According to the WID, RAN4 should define RRM requirements for inter-band CA and DC for combinations introduced in RF part. As to EMR requirements for target cell with 12RB PBCH, RAN4 needs to specify SSB index identification delay requirements for UE, taking into consideration the impacts from PBCH puncturing.

- In Rel-18, RAN4 has discussed SSB index identification delay for intra-frequency measurement and inter-frequency measurement. Finially, RAN4 has reached concensus that 3+4 samples for intra-frequency measurement with Es/Iot≥-6 dB, and 3+3 samples for inter-frequency measurement with Es/Iot≥-4 dB to acquire the index of the SSB being measured. The definiation of time period for time index detection for a UE operating on a target cell with 12 PRB SSB has been captured in clause 9.2.5.1, 9.2.6.2, 9.3.4, and 9.3.9, applicable to FR1 intra-f without gap, FR1 intra-f with gap, FR1 inter-f with gap and FR1 inter-f without gap, respectively.

Observation 1: In Rel-18, SSB index identification delay

  - Unknown intra-frequency cell (Es/Iot≥-6 dB): 3+4 samples

  - Unknown inter-frequency cell (Es/Iot≥-4 dB): 3+3 samples

- In our understanding, same requirements can be reused for EMR requirements. In current spec., time period for time index detection for deactivated SCell as follow.

TSSB_index,NR_Inter is the additional time period used to acquire the index of the SSB being measured as defined in table 4.4.2.2-1. Table 4.4.2.2-1: TSSB_index,NR_Inter DRX cycle length [s]; Scaling Factor (N1); TSSB_index,NR_Inter [s] (number of DRX cycles). FR1; FR2Note1. 0.32; 1; 8; N2 x 1.28 x N1 x 1.5 (N2 x 4 x N1 x 1.5). 0.64; 5; N2 x 1.28 x N1 (N2 x 2 x N1). 1.28; 4; N2 x 1.28 x N1 (N2 x 1 x N1). 2.56; 3; N2 x 2.56 x N1 (N2 x 1 x N1). NOTE 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. NOTE 2: N2 = 3 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1, and N2 = 5 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR2..

Therefore, for TSSB_index,NR_Inter, we prefer to reuse R18 requirements to compensate the losses caused by punching.

Proposal 1: Remove the bracket and the delay requirements for SSB index reading in EMR is extended by 3 more samples.

2.2PSCell configured with less than 5MHz bandwidth

For NR DC, we have definied the requirements for

- PSCell addition, conditional PSCell addition, Subsequent Conditional PSCell addition delay requirements

- PSCell change, conditional PSCell change, Subsequent Conditional PSCell change delay requirements

- Handover with PSCell, conditional handover including target MCG and SCG

- Known and unknown SCG activation delay requirements

- Measurement requirements for deactivated SCC

For the third bullet, we discussed Handover with PSCell (clause 6.1.5) and conditional handover including target MCG and SCG (clause 6.1.6) in R19, and updated the change delay requirements of PSCell. Meanwhile, NR Conditional Handover including target MCG and candidate SCG in clause 6.1.7 maybe also need updataed, which involves PSCell change delay. We understand that this should also be within the scope of this topic.

##### 6.1.7.1.2 PSCell conditional change delay DCHOwithCPC_PSCell = TRRC_delay + TEvent_DU + max (Tmeasure_PCell, Tmeasure_PSCell) + TUE_preparation + Tprocessing + T∆_PSCell + TPSCell_DU + 2 ms - TRRC_delay is the RRC procedure delay defined in clause 12 in TS 38.331 [2] for processing the RRC command. - TEvent_DU in PCell and PSCell handover delay requirement is the delay uncertainty which is the time from when the UE successfully decodes the RRC command for CHO including target MCG and candidate SCG for CPC until conditions exist at the measurement reference point which will trigger the CHO and CPC. - Tmeasure_PCell is the time for the UE to measure and realize the condition for CHO is fulfilled. Tmeasure_PCell equals to the measurements time stated in clause 6.1.7.1.1.1. - Tmeasure_PSCell is the time for the UE to measure and realize the condition for conditional PSCell change is fulfilled. Tmeasure_PSCell equals to the measurements time stated in clause 6.1.7.1.2.1. - TUE_preparation is the UE preparation time for conditional handover with conditional PSCell change, and starts at simultaneous execution of PCell handover and PSCell change to a target PCell and a target PSCell respectively after UE realizes the conditions of conditional PCell handover and conditional PSCell change are met and identities of the target PCell and PSCell are determined. TUE_preparation is up to 10 ms. - Tprocessing is as specified in clause 6.1.7.1.1. - T∆_PSCell is time for fine time tracking and acquiring full timing information of the target PSCell. T∆_PSCell = 1*Trs ms. - Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms. - TPSCell_DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in table 8.1-1 of TS 38.213 [3].

Proposal 2: SSB index acquisition of less than 5MHz cell should be extended in legacy requirement for Conditional Handover including target MCG and candidate SCG. And T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

Conclusions

In this contribution, we provide our views on the RRRM impacts for less than 5MHz BW and the proposals are:

Proposal 1: Remove the bracket and the delay requirements for SSB index reading in EMR is extended by 3 more samples.

Proposal 2: SSB index acquisition of less than 5MHz cell should be extended in legacy requirement for Conditional Handover including target MCG and candidate SCG. And T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

References

- R4-2420089, WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2, Intel