3GPP TSG-RAN WG4 Meeting # 114                                    R4-2500699

Athens, GR, 17th – 21st February, 2025

Agenda item:	 7.4.4

Source:        ZTE Corporation, Sanechips

Title:           Discussion on performance for less than 5MHz BW

Document for:  Approval

Introduction

The work item on NR support for dedicated spectrum less than 5MHz for FR1 was approved at RAN#103. According to the WID, RAN4 shoud define RRM requirements for inter-band CA and DC for combinations introduced in RF part .Meanwhile, The performance part for FR1 less than 5 MHz should be started from the RAN4#114 meeting. Hence, this contribution paper provides the views on which test shall be specified.

Discussion

As per the WID in Rel-19, CA and DC are supported and the RRM requirements maybe impact due to introduction of NR in less than 5 MHz bandwidth. RANP agreed to use a PBCH transmission bandwidth is 20 PRBs for 5MHz channel bandwidth and a PBCH transmission bandwidth is 12 PRBs for 3MHz channel bandwidth. For the case that CBW equals to 3MHz, the transmission bandwidth for PBCH is smaller and RAN4 should discuss how to update relative requirements.We have discussed three meetings and reached many agreements [1]. Specifically, our requirement modifications for the core part include the following aspects.

| SCell configured with less than 5MHz bandwidth | SCell activation delay requirements Measurement requirements for deactivated SCC |
| --- | --- |
| PSCell configured with less than 5MHz bandwidth | PSCell addition delay requirements Conditional PSCell addition delay requirements Subsequent Conditional PSCell addition delay requirements PSCell change delay requirements Conditional PSCell change delay requirements Subsequent Conditional PSCell change delay requirements Handover with PSCell NR Conditional Handover including target MCG and target SCG SCG activation delay requirements Measurement requirements for deactivated PSCell |
| Other RRM impacts | EMR requirements |

For the UE operating with 5 MHz bandwidth, the defined requirements are the same as that for the existing NR UE and there is no need to define additional test cases to cover the UE operating with 5 MHz. For the UE operating with 3 MHz bandwidth, the main changes from the core requirements are captured in SSB index reading. Therefore, the test cases should be specified to cover these new requirements.

Proposal 1: For the UE operating with 3 MHz bandwidth, the main changes from the core requirements are captured and the test cases should be specified to cover these new requirements.

Roughly browsing through the test cases of the current spec in A.6, at least the following parts involving FR1 SCell and PSCell need to be updated to adapt to PBCH punching:

A.6.3.3 Conditional handover A.6.3.1.17 Handover with PSCell change delay from NR-DC (FR1-FR1) to NR-DC (FR1-FR1) A.6.3.3.3 NR conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC A.6.5.3 SCell Activation and Deactivation Delay A.6.5.10 Conditional PSCell addition and release delay (FR1 NR-DC) A.6.5.11 PSCell addition and release delay A.6.5.12 Subsequent conditional PSCell addition/change A.6.5.12.1 Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC A.6.5.12.2 Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC A.6.6.9 Idle Mode CA/DC Measurements

Proposal 2: RAN4 should specify the following test cases and discuss whether/how to make down-selection:

- Handover with PSCell change delay

- NR conditional handover including target MCG and target SCG

- SCell Activation and deactivation of SCell

- Conditional PSCell addition and release delay

- PSCell addition and release delay

- Subsequent conditional PSCell addition/change

- Idle Mode CA/DC Measurements

- Other cases are not precluded.

Similar to the discussion on test cases of R18 PCell configured with less than 5MHz bandwidth, testing configuration can follow the conclusion of R18. As for the specific test cases, we can refer to the test steps of normal NR UE. Meanwhile, to avoid duplicate descriptions, we can use a simplified test case writing approach that proposed by companies in R18 to reduce the overhead caused by the test case parameters. And the main difference between traditional approach and simplified test approach is that only changed parameters are specified when compared to the baseline table.

The test procedure in clause A.6.6.1.5.2 applies for this test. Supported test configurations are specified in table A.6.6.1.12.2-1. General test parameters as specified in table A.6.6.1.5.2-2 with config 1 apply to this test, except those specified in table A.6.6.1.12.2-2. Cell specific test parameters as specified in table A.6.6.1.5.2-3 with config 1 apply to this test, except those specified in table A.6.6.1.122-3.

Proposal 3: Align the description of test case with PCell, use simplified approach for less than 5MHz test cases.

Conclusions

In this contribution, we provide our views on the performance part for less than 5MHz BW and the proposals are:

Proposal 1: For the UE operating with 3 MHz bandwidth, the main changes from the core requirements are captured and the test cases should be specified to cover these new requirements.

Proposal 2: RAN4 should specify the following test cases and discuss whether/how to make down-selection:

- Handover with PSCell change delay

- NR conditional handover including target MCG and target SCG

- SCell Activation and deactivation of SCell

- Conditional PSCell addition and release delay

- PSCell addition and release delay

- Subsequent conditional PSCell addition/change

- Idle Mode CA/DC Measurements

- Other cases are not precluded.

Proposal 3: Align the description of test case with PCell, use simplified approach for less than 5MHz test cases.

References

- R4-2416849, WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2, Intel