3GPP TSG-RAN WG4 Meeting # 114                                    R4-2500704

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.27.2.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Conditional Intra-CU LTM

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to L3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

Discussion

Compared to R18 LTM, conditional Intra-CU LTM should be supported in R19.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

LTM as introduced in Rel-18 offers short interruption time but not with the same level of robustness as the conditional L3 mobility procedures. In Rel-19, enhancements should be specified so that the system can benefit from both the high robustness and short interruption. It’s beneficial to combine LTM and conditional L3 mobility procedures and RAN4 needs to define requirements based on RAN2’s design.

2.1 General

In R18 mobility, RAN4 has agreed that the cell switch delay requirement is applicable for the case without L1 measurement and report under certain conditions for FR1. meanwhile, for FR2, the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured.

RAN4#113 Issue 1-1-2: Measurement requirements on conditional LTM < Way forward > FFS the following proposal: - Support L3 measurement based conditional LTM for FR1 from RAN4 perspective. - Note: QC need further check. Continue to discuss in the next meeting. RAN2#128 The triggering condition of conditional LTM can be based on L3 measurement.

That is to say, at least for FR1, LTM is decoupled from L1 measurement and can complete cell switching based on L3 measurement. In R19, we need to discuss conditional cell switching. If the switching here can only be based on L1 measurements, then the protocol appears incomplete. To maintain alignment with R18 LTM, we tend to support L3 measurement. In addition, RAN2 also discussed this issue in the last meeting and reached a positive conclusion. Therefore, from the perspective of RAN4, conditional LTM based on L3 measurement should be supported, at least for FR1.

Proposal 1: Support L3 measurement based conditional LTM for FR1 from RAN4 perspective.

RAN4#113 For UE based TA measurement: < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK)

Regarding the issue of UE based TA measurement, RAN4 has also discussed the same issue in R18 mobility, but due to time constraints, we did not define dedicated requirements. In the discussion of R19 conditional LTM, RAN2 agreed to consider two TA acquisition methods at the RAN2#127bis meeting, including UE based TA measurement and PDCCH ordered early TA acquisition. At the same time, signaling details were discussed in RAN2#128 meeting. So, from the perspective of RAN4, we should define the relevant requirements. Meanwhile, the dedicated requirements for UE based TA measurement can also be used in R18 LTM.

RAN2#127bis - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. RAN2#128 - The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. - The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. - Candidate cell TA is maintained by a new timer.

Proposal 2: RAN4 should define RRM requirements for UE based TA measurement.

2.2 Conditional LTM delay

From the objective, UE evaluated conditions will be specified by other groups to trigger LTM. RAN4 should define conditional cell switch delay requirement, both intra-frequency and inter-frequency need be considered. Meanwhile, the requirements are applicable to conditional cell switch to change the NR PCell to another NR cell. The relevant requirements can refer to the conditional handover of L3 in section 6.1.4.

6.1.4.2 NR FR1 – NR FR1 conditional handover The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handover from NR FR1 cell to NR FR1 cell. ##### 6.1.4.2.1 Handover delay Procedure delays for all procedures that can command a conditional handover are specified in TS 38.331 [2]. When the UE receives a RRC message implying conditional handover the UE shall be ready to start the transmission of the new uplink PRACH channel within DCHO seconds from the end of the last TTI containing the RRC command. DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution Where: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2]. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover Tmeasure is the measurements time stated in clause 6.1.4.2.2. TCHO_execution is the conditional execution preparation time in clause 6.1.4.2.3. Tinterrupt is the interruption time stated in clause 6.1.4.2.4.

Proposal 3: For conditional LTM delay, CHO requirements framework can be used as baseline.

In previous meeting, RAN2 has reached consensus that source cell sends the conditional LTM configuration via RRCReconfiguration to UE. In legacy, the starting point of conditional handover delay is the end of the last TTI containing the RRC command. As to the starting point of conditional LTM delay, we prefer to reuse the definition in conditional handover. That is, the starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM. In the last meeting, there were four proposals. In our understanding, the difference between proposal1 and proposal2 is whether to consider RRC procedure delay. The key issue is whether to use the same requirements for conditional LTM and subsequent conditional LTM.

Issue 1-3-2: The starting point of conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM. - Proposal 2: The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration. - Proposal 3: Similar as event-triggered L1 reporting. The CondEvent is met over the air. - Proposal 4: Wait for more RAN2 agreements.

Proposal 4: before defining the starting point of conditional LTM delay, RAN4 should discuss whether to use the same requirements for conditional LTM and subsequent conditional LTM.

- If yes, The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration.

- If no, the starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM.

In last meeting, RAN4 has reached agreement that The ending point of conditional LTM is the time the UE transmits the first UL transmission on the target cell, which can be used for both rach-based and rach-less procedure. As to the starting point of conditional LTM delay, the difference is whether to consider TRRC as a component of conditional LTM delay.

Issue 1-3-4: The components of conditional LTM delay < Way forward > FFS the following proposals: - Proposal 1: DCLTM = TRRC + TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process. - Proposal 1a: TEarly_process is the time for early candidate TCI State activation/deactivation and early TA acquisition. The time for early TA acquisition could be as the time for UE based TA measurement or PDCCH ordered early TA acquisition further. - Proposal 2: DCLTM = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. - Proposal 3: DCLTM = TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution. - Proposal 4: DC-LTM = TRRC + TEvent_DU + Tmeasure + TCHO_execution + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. - Proposal 4a: TLTM-processing, Tfirst-RS, TRS-proc, TLTM-IU which are defined in R18 LTM cell switch delay can be reused. - Proposal 5: The delay shall include the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) and interruption time (Tinterrupt) at least. Others are waiting for further progress in other groups. - Proposal 6: TEvent_DU and Tmeasure like components are likely needed, exact components are needed to wait for more RAN2 agreements. - Proposal 7: Wait for RAN2 progress.

Proposal 5: Use DCLTM = TRRC + TEvent_DU +Tmeasure + TCLTM_execution + TLTM-interrupt. as baseline and consider whether/how to capture the time for early candidate TCI State activation/deactivation and early TA acquisition.

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM

Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes LTM cell switch to a target cell and interruption time starts.

TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch.

2.3 Subsequent Conditional LTM delay

as we mentioned before, the difference between conditional LTM delay and subsequent conditional LTM delay is whether to consider RRC procedure. Therefore, Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Meanwhile, the definition of TEvent_DU need to be updated, which is similar as subsequent CPAC in R18.

RAN4#106bis Issue 1-1: starting point of subsequent CPC in RRM requirements Starting point of subsequent CPC in RRM requirements is the time when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change. Issue 1-2: ending point of subsequent CPC in RRM requirements Ending point of subsequent CPC in RRM requirements is the transmission of PRACH preamble towards the target PSCell. Issue 1-3: subsequent CPC delay requirements For subsequent CPC delay requirements: if starting point is the time when UE completes the previous CPC/CPA, Tconfig_PSCell_Subsequent_Change_Conditional = TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms and the definition of each component is the same as 8.11B.2 in TS38.133 except that TEvent_DU needs to be updated. Issue 1-4: TEvent_DU in subsequent CPC delay requirements TEvent_DU is the delay uncertainty which is the time from [when UE transmits SN RRCReconfigurationcomplete message for the previous PSCell addition or change] until a condition exists at the measurement reference point which will trigger the subsequent conditional PSCell change.

Proposal 6: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Similar as SCPAC, TRRC will not be considered in subsequent conditional LTM delay and TEvent_DU needs to be updated.

Proposal 7: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

Conclusions

In this contribution, we put forward the following proposals on conditional LTM.

Proposal 1: Support L3 measurement based conditional LTM for FR1 from RAN4 perspective.

Proposal 2: RAN4 should define RRM requirements for UE based TA measurement.

Proposal 3: For conditional LTM delay, CHO requirements framework can be used as baseline.

Proposal 4: before defining the starting point of conditional LTM delay, RAN4 should discuss whether to use the same requirements for conditional LTM and subsequent conditional LTM.

- If yes, The starting point is the time when UE transmits an RRCReconfigurationComplete message that is for confirmation of the LTM configuration.

- If no, the starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM.

Proposal 5: Use DCLTM = TRRC + TEvent_DU +Tmeasure + TCLTM_execution + TLTM-interrupt. as baseline and consider whether/how to capture the time for early candidate TCI State activation/deactivation and early TA acquisition.

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM

Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes LTM cell switch to a target cell and interruption time starts.

TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch.

Proposal 6: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay. Similar as SCPAC, TRRC will not be considered in subsequent conditional LTM delay and TEvent_DU needs to be updated.

Proposal 7: The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2420102, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom