3GPP TSG-RAN4 Meeting #114	R4-2500727

Athens, GR, February 17 – 21, 2025

Agenda item:			7.6.3

Source:	vivo

Title:	Impacts on UE RF requirements and DL performance

Document for:	Discussion

- Introduction

In RAN4#113 meeting, the WF[1] has been agreed as follows, and this contribution will discuss about the remaining open issues.

#### Issue 2-1: Discussion on baseband assumptions Agreement: - Consider only same SCS for both CCs for one Rx RF chain architecture in this SI - 15KHz SCS is used as the starting point - Single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the TR. Issue 2-5: Triggering condition to enable ”One RF Rx chain” RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled - Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled - Other determining factors are not precluded to be studied - Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2 Issue 3-3: Discussion of impacts on DL performance for UE enabling fragmented carrier mode - Companies are encouraged to bring analysis with their own assumptions following the table format - A separate table would be needed for Tx leakage + blocker and individual CA channel bandwidth combination in Table 7.3A.2.2-1 of TS 38.101-1 #### Issue 3-4: Discussion on in-gap requirements - RAN4 evaluate performance impact on the cases: - image of in-gap interference overlaps with wanted carrier - For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC - image of in-gap blocking interference does not overlap with wanted carrier #### Issue 2-4: Signaling related Companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting. #### Issue 3-1: Discussion on evaluation priority of example bands It would be up to companies’ preference and contribution driven on the example bands evaluation. The study needs to cover evaluation on both FDD and TDD bands and focus on general requirements. Band specific requirements can be left for WI phase. Open issues (for information) #### Issue 2-7: Others RAN4 to clarify whether either of the two fragmented carriers shall be a PCELL or if fragmented carriers can be defined purely as DL SCELL’s #### Issue 3-2: Discussion on maximum power level of in-gap interference For one Rx RF chain, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm, specific value is FFS #### Issue 3-5: Discussion on DL fragmented carriers in TDD bands #### Background: To consider transition time if there’s one LO for the TDD band and study the feasibility Discuss following options for the number LO’s needed for fragmented carrier operation in TDD with one of the carriers having UL allocation:  Option 1: One LO will be enough for both UL and DL.  Option 2: Separate LOs will be needed for UL and DL.

- Discussion

## 2.1. DL performance test of CA_n25(2A)

### General idea on testing

Testing should be a very useful way to verify the current performance of the RF chain. However, the difficulty is that the current UE implementation doesn’t have the “sharing” assumptions. For the UE actually implemented intra-band NC CA, the separate RF chains would be used, thus we cannot verify a UE that actually support CA_n25(2A).

However, we can use the single carrier configuration that configures a large channel bandwidth to mimic the “sharing condition”. In this situation, a large bandwidth can be configured, while only a part of RBs were actually scheduled, to mimic the NC CA situation. By comparison, the performance of the small channel bandwidth can be used to mimic the separated RF chain implementation.

In addition, the constraints of system simulators have to be taken into account. Currently, there are no non-contiguous allocation test for demodulation, and system simulators also do not support such non-contiguous configuration, though the physical layer spec support. In order to overcame this difficulty, separate testing of two different blocks (which is used to mimic two difference CCs in NC CA) is used.

In this way, the whole tests became feasible with a UE supporting n25 single CC operation and a normal system simulator.

Observation 1: Single carrier configuration with two blocks used respectively can be used as a way to verify the Fragmented carrier performance for sharing RF chain for NC CA.

### Bandwidth Configuration Description

Among the several example bands: n2, n25, n3, n7, n66, n41, n39, and n26, in this paper CA_n25(2A) is chosen as the main case to test the DL performance with RX chain sharing between the two DL CCs. Table1 shows the specific configuration: Two DL signals have the same receiving bandwidth of 5MHz, which is the simplest configuration considering the impact of UL and blocker on the DL performance. When the bandwidth of the two DL CCs is asymmetric the impact evaluation would be more complex, which will be studied in the future.

As can be seen from the yellow row marked under the CA_n25(2A) configuration in Table 1, the entire DL bandwidth in sharing state is 40MHz (SCC:5M+Wgap:30M+PCC:5M). Therefore, a 40M wideband single-carrier DL signal is set to represent the total sharing RX CBW, where the RB configuration in the upper or lower edge of the signal represents the location of the DL CC in the sharing RX CBW. Only one of the PCC and SCC is configured at a time, and the DL performance of the two DL CCS is observed alternately.

Also, for the uplink a 40M broadband signal with a full RB bandwidth of 5M is configured to simulate the actual 5M UL CC, with 80M frequency separation between the center of UL PCC RB allocation and the DL PCC RB allocation. UL RBs are allocated at the lower edge of the broadband 40M signal to keep the image product as far away from the DL signal as possible. It is a simplified configuration setup scheme from the test point of view, which will inevitably introduce some errors. All the reference sensitivity results are based on single antenna measurement. The following test results are all based on this configuration assumption and provide corresponding observations just from our point of view.

Fig1. configuration of CA_n25(2A) in testing

Table1. Power class 3 intra-band non-contiguous CA reference sensitivity with one uplink

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55 | 325 | 0.0 | FDD |
|  |  |  | Wgap = 30 | 505 | 0.0 |  |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |

### Test connection schematics

Fig2. connection schematic diagram of testing

As can be seen in the connection schematic diagram, the directional property of the directional coupler is used to prevent the incoming blocker signal from entering the Radio Communication Tester, and the duplexer is set to prevent TX high-power signals from entering into the signal source and causing alterations to the signal. The attenuation of the inserted devices has been taken into account.

### Test Case Description & Results & Analysis

As shown in ANNEX, there are 7 test cases in total, of which cases 1 to 6 have one PCC and one SCC test result each, while test case 7 has only PCC test results. Each test case is divided into two sub-test cases, i.e., the UL signal is set to the highest and lowest power to test the demodulation performance of DL CC respectively. The results and configurations are also copied below for easier reference.

Fig 3. Test configuration & REFSENS results of CA_n25(2A) in ANNEX

Case 1: Baseline, only Tx leakage effects are considered. The UL CC is also configured with full RB of 5MHz: 25RB inside 5MHz.

  - Compared to separate RX chain: With the highest UL output power, the REFSENS of PCC and SCC is deteriorated by 8.9dB and 8.8dB separately. With the lowest UL output power, the REFSENS of PCC and SCC is deteriorated by 8.8dB and 8.7dB separately.

  - When the RX chain is switched from separate to shared, the DL's suppression of UL signals decreases and the deterioration of DL performance by UL interference becomes more severe.

Case 2: Only Tx leakage effects are considered, the UL CC is 10RB configured on the highest side of 5MHz.

  - Compared to separate RX chain: With the highest UL output power, the REFSENS of PCC and SCC is deteriorated by 9.2dB and 8.9dB separately. With the lowest UL output power, the REFSENS of PCC and SCC is deteriorated by 8.8dB and 8.7dB separately.

  - Comparing case 2 to case 1, the deterioration of the DL demodulation performance is slightly increased when UL transmits at maximum power, because the UL power concentrates on the side of the allocated bandwidth close to the DL.

  - Generally, the impact on SCC is similar to PCC, because the frequency separation between UL CC and DL SCC is as far as 40M which is 8 times the UL CC’s BW, so for PCC the TX leakage would not be of big difference. What needs to be clarified is that since the data jitter introduced by the error of the test environment is about 0.5dB, the actual REFSENS of PCC in the table should be slightly better than that of SCC as it is further from the UL signal.

  - From the simulation results in section 2, it could be seen that the interference caused by the leakage of UL signals into the PCC and SCC frequency range is not significant, so it is reasonable to deduce that the interference of the uplink signal to the downlink signal mainly stems from the effect of TX leakage on the dynamic range of the ADC and the larger channel bandwidth introduced more noise for shared RF chain.

Fig4. configuration of case 1&2

Observation 2: For CA_n25(2A) with configuration of 5M+30M+5M: With only TX interference, REFSENS deteriorates by about 9 dB for the shared RX chain compared to the separate RX chain, and the deterioration is not much different between PCC and SCC.

Proposal 1: The large performance degradation should be considered for sperate RX chain and shared RF chain without considering blocking signal due to different RX channel bandwidth.

Case 3: Both TX leakage and blocker are considered, the blocker is configured as 5MHz beside the DL PCC, the power of the blocker is set as -65dBm (REFSENS in spec+31.5).

  - Compared to separate RX chain: With the highest UL output power, the REFSENS of PCC and SCC is deteriorated by 9.2dB and 8.8dB separately. With the lowest UL output power, the REFSENS of PCC and SCC is deteriorated by 9.1dB and 8.5dB separately.

  - The deterioration of REFSNS for both PCC and SCC is within 1 dB compared to the state with only UL signal interference.

Case4: Both TX leakage and blocker are considered, the blocker is configured as 5MHz beside the DL PCC, the power of the blocker is set as -81.5dBm (REFSENS +15).

  - Compared to separate RX chain: With the highest UL output power, the REFSENS of PCC and SCC is deteriorated by 8.8dB and 8.8dB separately. With the lowest UL output power, the REFSENS of PCC and SCC is deteriorated by 8.6dB and 8.6dB separately.

  - The deterioration of REFSNS for both PCC and SCC is within 1 dB compared to the state with only UL signal interference.

  - In cases 3 and 4, the REFSENS of SCC remains almost unchanged compared to case 1&2 due to the position of the blocker about 25M away from the SCC and its relatively low power.

Observation 3: For CA_n25(2A) with configuration of 5M+30M+5M: When the power of the blocker is relatively low (e.g., about 35 dB higher than the actual tested REFSENS and about 31.5 dB higher than the spec REFSENS), the deterioration generated by the blocker is not noticeable and much smaller than that by UL signal when the RX chain is switched from separate to shared.

Case5: Both TX leakage and blocker are considered, the blocker is configured as 5MHz beside the DL PCC, the power of the blocker is set as -25dBm (highest power).

  - Compared to separate RX chain: With the highest UL output power, the REFSENS of PCC and SCC is deteriorated by 8.2dB and 34.8dB separately. With the lowest UL output power, the REFSENS of PCC and SCC is deteriorated by 10.6dB and 36.6dB separately.

    - The deterioration of the demodulation performance of PCC is less obvious than that of SCC compared to the separate state, because the blocker is located in the adjacent channel of PCC and the power is very high at -25dBm, so the interference power leaking into the PCC is so high even in the independent chain (REFSENS deteriorates by about 29dB compared to the state with only UL signal interference, i.e., case1), so the deterioration is relatively less obvious when switching to the shared link.

    - For SCC, its own narrow-band filter will have a better suppression effect on the blocker which is far away when it receives on the independent chain (REFSENS only deteriorates by about 3dB compared to the state with only UL signal interference, i.e., case1), but when it switches to the shared chain, the blocker is located in the passband of the wide-band filter, so it can't be filtered out effectively, therefore, when the blocker is in the high power state, the interference to SCC will be especially obvious.

  - The sensitivity fluctuates greatly at minimum UL power, and if the received signal power decreases by 0.1dB at maximum UL power, then normal demodulation is not possible and it will be close to blocking.

Fig5. Configuration of case 3~5

Observation 4: For CA_n25(2A) with configuration of 5M+30M+5M: With the highest UL output power (-25dBm), the REFSENS of PCC and SCC deteriorates by 8~11dB and 34~37dB separately when the RX chain is switched from separate to shared.

Observation 5: When the blocker power increases to a certain level (e.g., -25dBm), the deterioration of the shared RX chain state compared to the separate RX chain state will be more pronounced in the SCC than in the PCC even when the blocker is allocated adjacent to PCC.

Case6: Both TX leakage and blocker are considered, the blocker is configured as 5MHz beside the DL SCC, the power of the blocker is set as -65dBm (REFSENS +31.5).

  - With the highest UL output power, the REFSENS of PCC and SCC is deteriorated by 8.7dB and 9dB separately. With the lowest UL output power, the REFSENS of PCC and SCC is deteriorated by 8.4dB and 9.1dB separately.

  - Compared with case3, the power of the blocker remains unchanged but its position is moved from next to the PCC to next to the SCC, so the combined effect of the UL signal and the blocker signal, which are both relatively close to the SCC, results in a deterioration that is more severe than in case 3. For PCC, the blocker is further away and the deterioration decreases compared to case3.

Fig6. Configuration of case 6

Case7: Both TX leakage and blocker are considered, the blocker is configured at 5MHz at the DL SCC position to simulate a scenario where its image overlaps the DL PCC just exactly, and the power is set to -65dBm (REFSENS +31.5) or -25dBm (highest power). Only the demodulation performance of PCC is observed in the two sub cases.

  - With the highest UL output power, the REFSENS of PCC is deteriorated by 40.6dB. With the lowest UL output power, the REFSENS of PCC is deteriorated by 41.5dB.

  - When the blocker’s power = -25dBm, PCC can't demodulate properly and the connection is hard.

  - When the image of the blocker overlaps the DL CC just exactly, the maximum power of the blocker that can be accepted to ensure proper demodulation of the DL signal is about -35 dBm.

Fig7. Configuration of case 7

Observation 6: When the image of the blocker overlaps the DL CC just exactly, the maximum power of the blocker that can be accepted to ensure proper demodulation of the DL signal is about -35 dBm.

Proposal 2: If only specifying one set of DL requirements with RX chain sharing, it should be based on the worst case where the blocker's image overlaps the DL CC completely.

Proposal 3: When the image of the blocker overlaps the DL CC just exactly, the maximum power of the blocker should be limited not higher than -35 dBm to ensure the proper demodulation of the DL signals.

## 2.2. Simulation results with only UL interference

The following figures are simulation results of CA_n7(2A), CA_n25(2A) and CA_n26(2A), only the interference of the UL signal’s leakage falling to exactly two DL CCs’ frequency range is considered, for this part of the interference, the filter could not provide sufficient attenuation. The red line of baseline represents the ideal throughput-SNR curve without UL leakage interference, and the green line represents the throughput-SNR curve with UL leakage interference. The DL chain is in a shared state, as can be seen from the six following figures, since the UL signal is closer to SCC, SCC is affected more severely.

For CA_n25(2A) in Fig8, the deterioration of both PCC and SCC is insignificant, especially for PCC. For SCC, the SNR deterioration due to TX leakage is about 2 dB. Combined with the results of the previous tests, it can be seen that for this configuration, the interference of the uplink signal to the downlink signal mainly stems from the effect of TX leakage on the dynamic range of the ADC.

Observation 7: For CA_n25(2A), the interference of the uplink signal to the downlink signal mainly stems from the effect of TX leakage on the dynamic range of the ADC.

For CA_n26(2A) compared to CA_n25(2A), although the power of the UL is relatively low, the UL PCC is much closer (only 10 M to DL SCC). Hence, the SNR of the SCC deteriorates significantly by about 10 dB. However for PCC, taking into account the effect of Wgap, the deterioration is relatively mild, about 2 dB.

For CA_n7(2A), it is similar to CA_n25(2A) as the power setting and frequency gap is similar. The deterioration of both PCC and SCC is insignificant, especially for PCC. For SCC, the SNR deterioration due to TX leakage is about 2 dB.

Table2. RB configuration & power settings of simulation

|  | UL configuration | Gap between UL PCC and DL SCC | DL configuration | PCC_power (Normalization) | SCC_power (Normalization) | UL_power (Normalization) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7(2A) | 50RB1, 10M | 75M | 10M+30M(Wgap)+5M | 0 | -2.2 | 76.8 |
| CA_n25(2A) | 25RB1, 5M | 40M | 5M+30M(Wgap)+5M | 0 | 0 | 78.5 |
| CA_n26(2A) | 5RB74, 15M | 10M | 15M+10M(Wgap)+10M | 0 | 23.4 | 74.7 |

Fig8. DL performance of CA_n7(2A)

Fig9. DL performance of CA_n25(2A)

Fig10. DL performance of CA_n26(2A)

- Conclusion

Observation 1: Single carrier configuration with two blocks used respectively can be used as a way to verify the Fragmented carrier performance for sharing RF chain for NC CA.

Observation 2: For CA_n25(2A) with configuration of 5M+30M+5M: With only TX interference, REFSENS deteriorates by about 9 dB for the shared RX chain compared to the separate RX chain, and the deterioration is not much different between PCC and SCC.

Proposal 1: The large performance degradation should be considered for sperate RX chain and shared RF chain without considering blocking signal due to different RX channel bandwidth.

Observation 3: For CA_n25(2A) with configuration of 5M+30M+5M: When the power of the blocker is relatively low (e.g., about 35 dB higher than the actual tested REFSENS and about 31.5 dB higher than the spec REFSENS), the deterioration generated by the blocker is not noticeable and much smaller than that by UL signal when the RX chain is switched from separate to shared.

Observation 4: For CA_n25(2A) with configuration of 5M+30M+5M: With the highest UL output power (-25dBm), the REFSENS of PCC and SCC deteriorates by 8~11dB and 34~37dB separately when the RX chain is switched from separate to shared.

Observation 5: When the blocker power increases to a certain level (e.g., -25dBm), the deterioration of the shared RX chain state compared to the separate RX chain state will be more pronounced in the SCC than in the PCC even when the blocker is allocated adjacent to PCC.

Observation 6: When the image of the blocker overlaps the DL CC just exactly, the maximum power of the blocker that can be accepted to ensure proper demodulation of the DL signal is about -35 dBm.

Proposal 2: If only specifying one set of DL requirements with RX chain sharing, it should be based on the worst case where the blocker's image overlaps the DL CC completely.

Proposal 3: When the image of the blocker overlaps the DL CC just exactly, the maximum power of the blocker should be limited not higher than [-35 dBm] to ensure the proper demodulation of the DL signals.

Observation 7: For CA_n25(2A), the interference of the uplink signal to the downlink signal mainly stems from the effect of TX leakage on the dynamic range of the ADC.

- References

- R4-2420337, WF on NR FR1 DL fragmented carriers study, MediaTek Inc. RAN4#113.

# ANNEX: Test configuration & REFSENS results of CA_n25(2A)

Table 2. Test configuration & REFSENS results of CA_n25(2A)

| ID | DL Allocation |  |  |  |  |  |  |  |  | Interference |  |  |  |  |  |  | Output |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Configuration / CBW |  |  |  |  |  |  |  |  | UL Allocation |  | blocker |  |  |  |  | sharing RX chain |  | separate RX chain (baseline) |  | deterioration value of shared chain compared to separate chain |  |
|  | Equivalent Single Carrier ( Fc_DL = 1950 MHz, Fc_UL = 1870 MHz, BW = 40 MHz ) |  |  |  |  | Block1 Equivalent BW (MHz) | Equivalent Wgap (MHz) | Block2 Equivalent BW (MHz) | SCS (kHz) | PCC |  | Allocation |  |  | P_blocker |  |  |  |  |  |  |  |
|  | Band | Block1 (Equivalent PCC) |  | Block2 (Equivalent SCC) |  |  |  |  |  | RB allocation (LCRB @ RBstart) | Range | Equivalent Range | BW (MHz) | Channel Center Frequency (MHz) | P_blocker=PCC_REFSENS+∆P | P_blocker=SCC_REFSENS +∆P | REFSENS of Block1 with max/min UL power (Equivalent PCC) | REFSENS of Block2 with max/min UL power (Equivalent SCC) | REFSENS of Block1 with max/min UL power (Equivalent PCC) | REFSENS of Block2 with max/min UL power (Equivalent SCC) | △REFSENS of Block1 with max/min UL power (Equivalent PCC) | △REFSENS of Block2 with max/min UL power (Equivalent SCC) |
|  |  | RB allocation | Fc(MHz) | RB allocation | Fc(MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for a CA_n25(2A) Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 | Note: Center approximate 1887.5 for equivalent 5MHz allocation | - | - | - | - | - | -92.9/-93.8 | - | -101.8/-102.6 | - | 8.9/8.8 |  |
| 1.2 | n25 | - | - | 25@0 | Note:Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -93.1/-93.7 | - | -101.9/-102.4 |  | 8.8/8.7 |
| 2.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 10@206 |  | - | - | - | - | - | -92.6/-93.8 | - | -101.8/-102.6 | - | 9.2/8.8 |  |
| 2.2 | n25 | - | - | 25@0 | Note: Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -93/-93.7 | - | -101.9/-102.4 |  | 8.9/8.7 |
| 3.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 |  | Note: Adjacent to block1； Close to 25@163 | 5 | 1962.2 | -65 （-96.5+31.5） | - | -92.3/-93.1 | - | -101.5/-102.2 | - | 9.2/9.1 |  |
| 3.2 | n25 | - | - | 25@0 | Note: Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -92.9/-93.6 | - | -101.7/-102.1 |  | 8.8/8.5 |
| 4.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 |  | Note: Adjacent to block1； Close to 25@163 | 5 | 1962.2 | -81.5 (-96.5+15) | - | -92.7/-93.7 | - | -101.5/-102.3 | - | 8.8/8.6 |  |
| 4.2 | n25 | - | - | 25@0 | Note: Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -92.9/-93.6 | - | -101.7/-102.2 |  | 8.8/8.6 |
| 5.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 |  | Note: Adjacent to block1； Close to 25@163 | 5 | 1962.2 | -25 (maximum power) | - | -64.9/-63 (Note1) | - | -73.1/-73.6 (Note1) | - | 8.2/10.6 |  |
| 5.2 | n25 | - | - | 25@0 | Note: Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -64.5/-63 | - | -99.3/-99.6 |  | 34.8/36.6 |
| 6.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 |  | Note: Adjacent to block2； Close to 25@25 | 5 | 1937.8 | - | -65 （-96.5+31.5） | -92.8/-93.7 | - | -101.5/-102.1 | - | 8.7/8.4 |  |
| 6.2 | n25 | - | - | 25@0 | Note: Center approximate 1932.5 | - | 30 | 5 | 15 |  |  |  |  |  |  |  | - | -92.5/-92.9 | - | -101.5/-102 |  | 9/9.1 |
| 7.1 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 | 25@191 |  | Note: Same position of previous block2, Image to block1；Close to 25@0 | 5 | 1932.8 | - | -65 （-96.5+31.5） | -61/-60.8 | - | -101.6/-102.3 | - | 40.6/41.5 |  |
| 7.2 | n25 | 25@191 | Note: Center approximate 1967.5 | - | - | 5 | 30 | - | 15 |  |  |  |  |  |  | -25 (maximum power) | Interference is too high to connect | - | -99.2/-99.7 | - |  |  |
| Note1: Sensitivity fluctuates greatly at minimum UL power, demodulation is not possible at maximum UL power if the target CC’s power drops by 0.1dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |