3GPP TSG-RAN4 Meeting #114	R4-2500730

# Athens, GR, February 17 – 21, 2025

Agenda item:			7.1.1.2.3

Source:	vivo

Title:	Discussion on MPR applicability for FR2

Document for:	Discussion

- Introduction

In RAN#113, the WF[1] has been agreed, and the remaining issues for FR2 are as follows:

# Topic #3: MPR applicability for FR2 ### Sub-topic 3-1: CC activation based enhancement for FR2 #### Issue 3-1-1: Conditions of CC activation based enhancement Agreement in main session and WF: - For the Rel-19 activation based enhancement, FR2 UE CA MPR is based on the activated subset of the CCs in the CA configuration where all the activated UL CCs forms a contiguous block. - FFS emission requirements are activated UL CCs BW based or Cumulative aggregated channel bandwidth (CABW) based #### Issue 3-1-2: Optional UE capability Agreement in main session and WF: - For Rel-19 FR2 UE MPR, define distinct enhancement capabilities for configuration-based enhancements and activation-based enhancement. - FFS if the capabilities should be mutually exclusive ### Sub-topic 3-2: Others - Proposals in R4-2418337 (Samsung) - Proposal 1: Highlight 800MHz upper limit for the enhanced MPR feature when applying MPR values in the tables. - Proposal 2: RAN4 to further discuss the MPR applicability when bandwidth basis = 400MHz or 800MHz. - WF - FFS proposal 2 with the identified discrepancy of CA BW class and MPR table - FFS if 800MHz is explicitly referenced as an upper limit

- Discussion

In the WF of #113, it has been agreed that for Rel-19 FR2 UE MPR, distinct enhancement capabilities should be defined for configuration-based enhancement and activation-based enhancement. During the #113 meeting, there was some offline discussion about the application details of the two UE capabilities initiated by QC, which should be clarified in this meeting:

When UE only supports configuration-based enhancement capability [fr2-MPR-ImprovementUplinkDependent-r19], which means that configured UL BWchannel_CA could be used as the bandwidth basis for MPR, otherwise Cumulative aggregated channel bandwidth (CABW) is the bandwidth basis. Also based on this capability, if only one UL CC is configured then MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2, i.e., the MPR of single carrier is used.

When UE only supports activation-based enhancement capability [fr2-MPR-ImprovementActivatedUplinkDependent-r19], which means that the activated CABW could be used as the bandwidth basis for MPR, otherwise Cumulative aggregated channel bandwidth (CABW) is the bandwidth basis. Also based on this capability, if only one UL CC and one DL CC are activated then MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2, i.e., the MPR of single carrier is used.

When UE supports both activation-based and configuration-based enhancement capabilities [fr2-MPR-ImprovementUplinkDependent-r19] & [fr2-MPR-ImprovementActivatedUplinkDependent-r19], which means that activated UL BWchannel_CA could be used as bandwidth basis for MPR. The activated UL BWchannel_CA is the bandwidth of the part of CCs that is activated from the configured UL BWchannel_CA. Based on the two capabilities, if only one UL CC is configured or if only one UL CC from the configured UL CCs is activated then MPRWT_C_CA shall be determined from Tables 6.2.2.1-1 and 6.2.2.1-2, i.e., the MPR of single carrier is used.

From the above discussion, it is clear that the two capabilities should be independent as they represent different transmission bandwidths applied for MPR respectively. However, they should not be mutually exclusive as one UE could support one of them or both of them. Furthermore, when the two capabilities are both supported by one UE, it could transmit uplink signals with the narrowest bandwidth to get maximum MPR reduction.

At the same time, in addition to the two capabilities discussed above, what should be discussed is whether the UE also needs to support the DC location reporting capability to perform MPR reduction as the change in bandwidth would also result in the change in DC location.

| extendedDC-LocationReport-r17 Indicates whether the UE supports extended DC location reporting (based on indicated default DC location) for at least 2 UL CCs in one band. A UE that supports this feature also supports extended DC location reporting for 1 UL CC in one band. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 1: The enhancement capabilities for configuration-based and activation-based should be independent but not mutually exclusive.

Proposal 2: In order to perform MPR reduction, whether extendedDC-LocationReport-r17 must be supported together with the two capabilities needs discussion.

- Conclusion

Proposal 1: The enhancement capabilities for configuration-based and activation-based should be independent but not mutually exclusive.

Proposal 2: In order to perform MPR reduction, whether extendedDC-LocationReport-r17 must be supported together with the two capabilities needs discussion.

- References

- R4-2420527, WF on power domain enhancement, Huawei, HiSilicon, RAN4 #113.