# 3GPP TSG-RAN WG4 Meeting #114	R4-2500733

# Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Updated simulation results of MIMO layer evaluation for 6Rx UE

Agenda Item:	7.1.1.3.2

Document for:	Approval

Introduction

In the past several meetings, we discussed the feasibility of 6MIMO layer however there was no consensus [1][2][3][4][5]. For 6Rx, we tried to align simulation methodology and parameters for 4&6 MIMO layer performance comparison, however some parameters were still open. We still have disputes over the feasibility for 6Rx-6Layer, not to mention that we cannot reach consensus on many key simulation parameters. We have encountered great resistance in the project progress. In this contribution, we provide our views on the feasibility for 6Rx-6Layer and give updated simulation results for the comparisons between 4 and 6 MIMO layers based on 32T6R, based on that, our views on 6 MIMO layer support are provided.

Discussion

For the meaning of feasibility study, we discussed several meetings. And the issue was captured in RAN4#113 meeting minutes.

Chair: the following is for information and encourage companies to provide the thoughts in the next meeting. Issue 2-1-7: Views on Feasibility - RAN4 to further discuss the meaning of feasibility for the introduction of features, including: - How does RAN4 collectively agree on company views (do all companies require to agree a feature is feasible)? - Does feasibility include gain over a legacy feature (in this case 6-layer vs 4-layer)? - Does the definition of feasibility correspond to feasibility in one case, a majority of cases, or all cases?

From our understanding, the meaning of feasibility depends on the context of a specific WID or SID. It can be different for different WID or SID. For 6Rx-6layer feasibility case, we think it is quite clear. According to the WID for 6Rx, the evaluation criteria for 6R-6L feasibility are:

- 6Rx-6L outperforms 6R-4L

- Based on realistic simulation assumptions

6Rx is targeted for Handheld UE, instead of FWA UE. If we cannot reach consensus on this issue, there will be no basis for further discussion.

One thing we want to highlight is what ‘realistic’ means for 6Rx-6Layer. In this case, we think ‘realistic’ means technology already deployed in the field, requirements in the specification, or assumptions based on measurements. We cannot use some fabricated simulation assumptions combinations to simulate gain for 6Rx-6Layer, saying that this feature is feasible. In RAN4, we suggest to continue the discussion on realistic simulation assumptions for 6Rx-6Layer. We suggest to further discuss and make a compromise on these simulation assumptions, e.g.

- BS Tx EVM & BS correlation matrix

- PDSCH DMRS configuration

- Rx EVM

Then, the group can collect companies’ simulation results and try to conclude on the feasibility for 6Rx-6Layer.

For 6 MIMO layer evaluation, we took link level simulation as baseline, using TDL channel models with antenna correlation matrix. We conducted performance simulation for 4&6 MIMO layer and the detailed simulation assumptions were listed in the Annex.

Simulation assumptions

We encountered a lot of controversy when discussing simulation parameters in the past meetings. One thing each company needs to be reminded is that the feasibility study for 6R-6L should be based on realistic assumptions. From our perspective, we encountered many problems for simulation assumptions for 6R-6L feasibility. Some companies choose very optimistic assumptions for the study, e.g. 1% for BS Tx EVM, ideal BS correlation matrix. Even if there is simulation gain in the simulation, it will weaken the justification for the feasibility for 6L-6L. If the situation in the field is worse than the assumption, 6L-6L may be not beneficial. Moreover, features without commercial use cannot be used as a simulation baseline, e.g. R18 eDMRS configuration. It is not persuasive to say one feature is feasible based on another feature which is not used in the field. Anyway, we should keep in mind what ‘realistic’ means when evaluating 6R-6L feasibility.

If we check the simulation parameters in the Annex, there are many items with more than one option. In this case, there are many combinations of simulation cases if there is no down selection. In our simulation, we choose two typical sets of simulation assumptions:

Priority 1：TDLA30-10+DMRS 1+1 for 4layer and 2+2 for 6layer + UE Correlation matrix: Option 3

Priority 2：TDLA30-10+DMRS 1+1 for 4layer and 2+0 for 6layer+ UE Correlation matrix: Option 1

UE antenna correlation

Antenna correlation is an important aspect for designing MIMO antenna systems. With lower antenna correlation, it indicates better MIMO performance that can be achieved with the underlying antenna system. The antenna correlation coefficient is mostly given as the range of 0.1~0.5 between any two antennas, which is reasonable considering the actual antenna performances. The UE antenna parameter is given in Table 1 in the annex. In the last meeting, Option 3 and Option 4 were added based on real measurements and they are realistic UE antenna correlation matrix. In our simulation, we picked Option 3 as Priority 1 case. For comparison, we also used Option 1 as Priority 2 case.

BS Tx EVM and correlation matrix

For BS antenna number, we assumed 8Tx as a baseline in our simulation.

BS Tx EVM and correlation matrix are two controversial simulation assumptions for 4&6 MIMO layer simulation. For Tx EVM, RAN4 agreed not to tighten this requirement, which confirmed the rationality of minimum requirement. Unless BS venders challenge this requirement, we suggest to use this requirement as a starting point. In the field, in order to past the test for Tx EVM, we agreed the BS performance can be better than the minimum requirement and Option 3 is reasonable to consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM, which is commonly used in demod session. Option 1 is too Optimistic and far away from the minimum requirement. We doubt than whether 1% Tx EVM can be achieved for 256QAM. We suggest to take Option 3 as baseline.

For BS correlation matrix, we think ideal non-correlation matrix is too optimistic. We suggest BS vendors can come up realistic correlation matrix. In our simulation, we considered both cases in the simulation table, that is, 3GPP MIMO correlation matrix with Alpha = 0.1/0 to compare the performance.

PDSCH DMRS configuration

For PDSCH DMRS configuration, we use DMRS Type 1 in Rel-15, and DMRS 1+1 for 4layer and 2+2 for 6layer as baseline in our simulation.

Simulation results

### Simulation results for Priority 1

Priority 1：TDLA30-10+DMRS 1+1 for 4layer and 2+2 for 6layer + UE Correlation matrix: Option 3

| a) UE correlation: option 3, BS correlation: option 1 | b) UE correlation: option 3, BS correlation: option 2 |
| --- | --- |

Figure 1. Performance comparison of 4&6 layer for Priority 1 Case

Figure 1 shows the SNR-Throughput curves of 4&6 layer performance for Priority 1 Case. Figure 1a uses the assumption of 3GPP MIMO correlation matrix with Alpha = 0.1. Figure 1b uses ideal BS correlation matrix with Alpha = 0. With the introduction of BS correlation Option 1, large throughput degradation can be observed compared with ideal BS correlation. For example, 4 rank with SNR equals 40dB, the maximum throughput is 753.61Mbps with ideal BS correlation and 327.87 Mbps with BS correlation with Alpha=0.1. The throughput degradation is 56% in this case with the introduction with realistic BS correlation matrix. Large throughput degradation can be observed for 6 rank under the same SNR condition with the introduction of BS correlation: option 2.

Observation 1: Under fixed rank and same SNR condition, large throughput degradation can be observed with BS correlation (Alpha = 0.1), compared with ideal BS correlation (Alpha = 0).

From our simulation, the BS correlation matrix will have a huge impact on the performance for 4 rank and 6 rank. Ideal BS correlation matrix is too optimistic. Low BS correlation with Alpha=0.1 seems more reasonable. It should be considered in the feasibility study.

For the comparison of 4&6 layer performance, we see no gain under the SNR up to 40dB for 6 layer. With or without introduction of BS correlation matrix with Alpha=0.1, 4 layer performance always out performs 6 layer. Under ideal BS correlation matrix, the performance of 6 rank will be close with 4 rank under SNR=40. With SNR higher than 40, 6 rank performance may out perform 4 layer, however it is not realistic to achieve such high SNR condition. Under BS correlation matrix with Alpha = 0.1, the performance of 6 layer is far behind 4 layer.

Observation 2: With or without introduction of BS correlation matrix with Alpha=0.1, 4 layer performance always out performs 6 layer under SNR less than 40dB.

Priority 2：TDLA30-10+DMRS 1+1 for 4layer and 2+2 for 6layer+ UE Correlation matrix: Option 1

| a) UE correlation: option 1, BS correlation: option 1 | b) UE correlation: option 1, BS correlation: option 2 |
| --- | --- |

Figure 2. Performance comparison of 4&6 layer for Priority 2 Case

For Priority Case 2, the only difference is UE correlation matrix compared with Priority Case 1. UE correlation Option 1 reflects another typical UE antenna design for 6Rx, which is also considered in our simulation. Figure 2 shows the SNR-Throughput curves of 4&6 layer performance for Priority 2 Case. Figure 2a uses the assumption of 3GPP MIMO correlation matrix with Alpha = 0.1. Figure 2b uses ideal BS correlation matrix with Alpha = 0. From Figure 2, it can be observed that it shares similar trend for the SNR-Throughput cures for 4 and 6 MIMO layers with different UE Rx correlation. With or without introduction of BS correlation matrix with Alpha=0.1, 4 layer performance always out performs 6 layer.

Observation 3: For different Rx correlation, it shares similar trend for the SNR-Throughput cures for 4 and 6 MIMO layers.

Observed from our simulation results Figure 1~2, it is not feasible for 6 layer since it doesn’t have gain compared with 4 layer. We suggest not to introduce this feature since it has no gain.

In this meeting, we add some new simulation cases proposed by our opponent:

- 32T6R+Rel-15 2+2(4L), Rel-15 2+2 (6L)+UE correlation matrix: Option 1+BS Tx EVM: 1% for all modulation

The simulation results under different BS correlation matrix are given in Figure 3 a) ~ c):

| a) BS correlation Matrix with alpha=0 |  | b) BS correlation Matrix with alpha=0.01 |
| --- | --- | --- |
| c) BS correlation Matrix with alpha=0.05 |  |  |

Figure 3. Performance comparison of 4&6 layer for Case 3

Figure 3 a)~c) show the simulation results for performance comparison of 4&6layer for case 3 under BS correlation with alpha equal to 0, 0.01, 0.05. For BS correlation matrix with alpha equals to 0, we tried to replicate the results by the opponent. 6R-6Layer shows gain compared 6Rx-4layer, as depicted in Figure 3a. However, we don’t think these results are convincing 6R-6L is feasible since the simulation assumptions are too optimistic. We added low BS correlation matrix with alpha equals to 0.01 and 0.05 as shown in Figure b) and c). The results show that 6R-4L out-perform 6R-6L under the SNR scale to 30dB. There is no performance gain for 6R-6Layer. We expect the results for 6Rx-6Layer to be worse if more realistic assumptions are added to the simulation, for example, if the BS Tx EVM are worse than 1% for higher modulation, or antenna efficiency, Rx EVM are considered in the simulation.

Observation 4: Considering low BS correlation matrix with alpha equal to 0.01 or 0.05, 6Rx-6Layer cannot outperform 6Rx-4Layer for Case 3.

Proposal 1: At least for handheld UE, do not support 6 MIMO layer for 6Rx.

Conclusion

This contribution discusses 6 MIMO-layer support for 6Rx. The following observations and proposal are made:

Observation 1: Under fixed rank and same SNR condition, large throughput degradation can be observed with BS correlation (Alpha = 0.1), compared with ideal BS correlation (Alpha = 0).

Observation 2: With or without introduction of BS correlation matrix with Alpha=0.1, 4 layer performance always out performs 6 layer under SNR less than 40dB.

Observation 3: For different Rx correlation, it shares similar trend for the SNR-Throughput cures for 4 and 6 MIMO layers.

Observation 4: Considering low BS correlation matrix with alpha equal to 0.01 or 0.05, 6Rx-6Layer cannot outperform 6Rx-4Layer for Case 3.

Proposal 1: At least for handheld UE, do not support 6 MIMO layer for 6Rx.

- References

[1] R4-2406585, WF on requirements for 6Rx, AT&T, RAN4#110-bis.

[2] R4-2410751, WF on requirements for 6Rx, AT&T, RAN4#111.

[3] R4-2414285, WF on 6Rx UE requirements, AT&T, RAN4#112.

[4] R4-2417209, WF on 6Rx UE requirements, AT&T, RAN4#112bis.

[5] R4-2420376, WF on 6Rx requirements, AT&T, RAN4#113.

# Annex

Table 1 Feasibility Study Link-Level Simulation Assumptions

| Parameters |  | Values |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHZ |
| SCS |  | 30 |
| Antenna configuration |  | {8Tx, 6Rx} as baseline Interested companies can submit results with {32Tx, 6Rx} and specify the precoding type |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank {4,6}, interested companies can bring {5} |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 Interested companies can bring MCS Table 1 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx; companies are encouraged to provide details on the SVD update periodicity (5ms) |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) Option 2: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) Option 4: Rel-15 2+2(4L), Rel-15 2+2(6L) [To be further discussed online] |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx, companies are encouraged to provide details on the SVD update periodicity |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Option 4: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 Option 5: 3GPP MIMO correlation matrix with beta=0.00718 (Option 5 is parametric and to be justified since Options 1-4 are based on measurements) Note: For the feasibility analysis, antenna efficiency can be considered. Companies are encouraged to provide the antenna efficiency numbers. |
| BS correlation matrix |  | Option 1: 3GPP MIMO correlation matrix with Alpha = 0.1 Option 2: 3GPP MIMO correlation matrix with Alpha = 0.0 Other options are not precluded. Matrices requested from interested BS vendors for RAN4#113 |
| BS Tx EVM |  | 3% for 256QAM 6% for 64QAM |