# 3GPP TSG-RAN WG4 Meeting #114	R4-2500734

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Further discussion on SRS antenna switching requirements

Agenda Item:	7.1.1.3.3

Document for:	Approval

Introduction

SRS antenna switching requirements are one of important aspects for 6Rx requirements. In the last meeting, we discussed ΔTRxSRS values and whether to consider a subset of available patterns for 3TR6 AS-SRS. The WFs are captured in [1]. In this contribution, we will further discuss the remaining issues with the addition of t3r6.

Discussion

The WF [1] for SRS antenna switching and ΔTRxSRS of last meeting was captured as follows:

Issue 1-1-1: Whether to consider a subset of available patterns for 3T6R AS-SRS Way Forward: RAN4 to further discuss the following options. - Option 1: Consider a subset of available patterns for 3T6R AS-SRS. FFS on the subset details. - Option 2: Consider all available patterns for 3T6R AS-SRS. Issue 1-2-1: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 Way Forward: RAN4 to check whether the ∆TRxSRS values in the table below could be agreed. The average values from company proposals, as well as the existing values for other AS-SRS configurations, were considered as the starting point and were rounded to align with 0.5dB increments to keep consistency with the existing specification approach. Table: ΔTRxSRS values Operating Bands; ΔTRxSRS t3r6 (dB); ΔTRxSRS t1r6-t3r6 (dB); ΔTRxSRS t2r6-t3r6 (dB); ΔTRxSRS t1r6-t2r6-t3r6 (dB). Band n41, n77, n78; 3.0; 4.5; 4.0; 5.5. Band n79; 4.5; 6.0; 5.5; 7.0. Band n104; 5.0; 7.0; 6.0; 8.0.

For Issue 1-1-1, we tried to understand the intention for this issue. If it is only for the simplicity of the SRS antenna switching requirements, it is not necessary to consider a subset of available patterns for 3T6R AS-SRS only. For t1t6 and t2r6, we already considered all available patterns. For 3T6R, we should do the same. Furthermore, most companies already proposed ΔTRxSRS for all available patterns for 3T6R AS-SRS. There is no point in further downselection.

Proposal 1: Consider all available patterns for 3T6R AS-SRS, that is, t3r6, t1r6-t3r6, t2r6-t3r6, t1r6-t2r6-t3r6.

For SRS antenna switching requirements, we need to study the ΔTRxSRS requirements based on possible RF architectures for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6. This value is the additional insertion loss when UE transmits SRS on Rx antenna port and it is highly related to the RF architecture for SRS antenna switching. Figure 1~4 shows the example RF architectures for supporting t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6.

Figure 1. Example RFFE for t3r6

Figure 2. Example RFFE for t1r6-t3r6

Figure 3. Example RFFE for t2r6-t3r6

Figure 4. Example RFFE for t1r6-t2r6-t3r6

In all, the value of ΔTRxSRS is highly related to the RF architecture for SRS antenna switching, including the insertion loss of RF switch (e.g. SP4T, SP2T) and additional SRS trace. For the insertion loss, we give the numbers for each item:

- SP2T: 0.5/0.6dB

- SP3T: 0.8/1dB

- SP4T: 1/1.3dB

- DP3T: 1.1/1.4dB

- The IL of additional SRS Trace: 1.8/2.4 dB

The number before the dash is the insertion loss for band n41, n77, n78, and the number after the dash is for band n79.

Take t3r6 as an example, the insertion loss for Ant 2 is assumed as the worst, which is 2.8 dB (0.5+0.5+1.8=2.8dB) for band n41, n77, n78. The insertion loss for Ant 1 is TRx path with minimum insertion loss, which is 0.5 dB. Then the ΔTRxSRS value is 2.3dB for t1r6 for band n41, n77, n78.

Based on our calculation, we derive the ΔTRxSRS values for t1r6, t2r6 and t1r6-t2r6.

| Operating bands | ΔTRxSRS requirement for t3r6 | ΔTRxSRS requirement for t1r6-t3r6 | ΔTRxSRS requirement for t2r6-t3r6 | ΔTRxSRS requirement for t1r6-t2r6-t3r6 |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 0.5+0.5+1.8-0.5=2.3dB | 1+0.5+0.5+1.8-1=2.8dB | 0.8+0.8+1.8-0.8=2.4dB | 1+1.1+0.8+1.8-1=3.7dB |
| Band n79 | 0.6+0.6+2.4-0.6=3dB | 1.3+0.6+0.6+2.4-1.3=3.6dB | 1+1+2.4-1=3.4dB | 1.3+1.4+1+2.4-1.3=4.8dB |

The actual RFFE is more complex than the above example RF architectures considering supporting multiple bands and other factors. It is reasonable to consider some implementation margins for ΔTRxSRS requirements for 6Rx.

Furthermore, we agreed to introduce additional breakpoint for band n104. The IL loss difference is 0.5dB for t1r6, 0.5dB for t2r6 and 1dB for t1r6-t2r6 due to the frequency difference between band n79 and n104. We suggest to adopt the same margin, 0.5dB for t3r6, 1dB for t1r6-t3r6 and t2r6-t3r6, 1.5dB for t1r6-t2r6-t3r6 for band n104.

Thus our proposal for ΔTRxSRS values of t3r6, t1r6-t3r6, t2r6-t3r6, t1r6-t2r6-t3r6 for 6Rx is as follows:

| Operating bands | ΔTRxSRS requirement for t3r6 | ΔTRxSRS requirement for t1r6-t3r6 | ΔTRxSRS requirement for t2r6-t3r6 | ΔTRxSRS requirement for t1r6-t2r6-t3r6 |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.5dB | 5dB | 4dB | 5.5dB |
| Band n79 | 4dB | 6 dB | 5.5dB | 6.5dB |
| Band n104 | 4.5dB | 7dB | 6.5dB | 8 dB |

In the last meeting, our proposal was averaged with other companies’ proposals. It can be observed that our proposal and the averaged value are very close. The difference between average value and our proposal is within 0.5 dB. The average method is generally OK for us.

Proposal 2：The average value can be adopted for t3r6, t1r6-t3r6, t2r6-t3r6, t1r6-t2r6-t3r6:

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

Conclusion

This contribution discusses ΔTRxSRS requirements for 6Rx considering SRS antenna switching configurations t3r6, t1r6-t3r6, t2r6-t3r6 and t1r6-t2r6-t3r6. The following proposals are made:

Proposal 1: Consider all available patterns for 3T6R AS-SRS, that is, t3r6, t1r6-t3r6, t2r6-t3r6, t1r6-t2r6-t3r6.

Proposal 2: The average value can be adopted for t3r6, t1r6-t3r6, t2r6-t3r6, t1r6-t2r6-t3r6:

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

- References

[1] R4-2420376, WF on 6Rx requirements, AT&T, RAN4#113.