3GPP TSG-RAN WG4 Meeting # 114 	R4-2500741

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on the remaining issue of FR2 OTA

Agenda Item:	7.15.2

Document for:	Approval

- Introduction

In the past meetings, the test method for sTxMP was almost finished and in this contribution, we provide our view on remaining issues.

- Discussion

2.1 Alternative method for AoA selection

Whether a test direction other than the beam peak direction of single CC operation without STxMP can be used for first AoA

- Agreements:

  - Alternative test directions rather than the default AoA1 to be declared by UE.

    - The declaration of AoA1 direction can be any direction. If UE doesn’t declare AoA1, the default AoA1, i.e., the beam peak direction of single CC operation without STxMP, will be applied.

    - AoA2 direction is to use AoA1 + or - UE declared offset from set of {30deg, 60deg, 90deg, 120deg, 150deg} in xz plane of test system

    - AoA1+ UE declared offset or AoA1 - UE declared offset needs to be declared by UE

In [1], a method for alternative of AoA selection of sTxMP was agreed:

This alternative AoA is totally decided by UE declaration, but it is not possible to allow UE declare an AoA with arbitrary granularity and some restriction seems to be needed. Since another option for AoA selection is beam peak of single CC if UE not declare any AoA，we prefer use same test grid for both of them, i.e., the declared AoA should align with the test grid of beam peak search.

Proposal 1: The UE declared AoA should align with the test grid of beam peak search of single CC.

2.2 MU for sTxMP testing

Another pending issue is the MU of sTxMP testing. In our understanding, one that may introduce additional uncertainty is when TE distinguish the power of beams. It is not clear how real TE to do this but one possible way to perform measurement of each beam is measure the EPRE of DMRS and then estimate the power of each layer, and such estimation may introduce uncertainty since the power of PUSCH may not be flat in time domain or frequency domain. However, more information may need to be provided by TE vendor if the MU need to be evaluated.

Observation 1: It is better for TE vendor to provide information on whether distinguishing the power of different beams will introduce additional uncertainty.

- Conclusion

In this contribution, we provide our views on MU and remaining issue in test method

Proposal 1: The UE declared AoA should align with the test grid of beam peak search of single CC.

Observation 1: It is better for TE vendor to provide information on whether distinguishing the power of different beams will introduce additional uncertainty.

- References

- R4-2419810, Way Forward for [113][327] NR_FR2_OTA, Qualcomm Incorporated