3GPP TSG-RAN WG4 Meeting #114 	R4-2500746

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Discussion on the RF requirement for CW

Agenda Item:	7.24.2.3

Document for:	Approval

- Introduction

RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

  - Use band n8 as an example band

In [1], the WID for R19 AIoT was approved:

In this contribution, we provide our views on the RF requirement for CW.

- Discussion

- Frequency of CW

During SI stage, we provided the analysis for the impact from CW to legacy NR, and the published paper shows that if the CW is orthogonal with NR, the influence can be minimized [2]. From this perspective, it is better to force the CW frequency to align with the NR channel raster, especially for in-band operation.

Proposal 1: For in-band operation, the frequency of CW should align with the NR channel raster.

Another transmit quality requirement is frequency error. In [3], the frequency error for RFID reader is ±10ppm, which can be considered as the starting point

The frequency error is the the difference between the measured operating frequency and the nominal operating

frequency. relative to the nominal operating frequency of each of the applicable channels. The frequency error shall not

exceed ±10 ppm [3]

Proposal 2: Take ±10ppm as the starting point for CW frequency error.

- Maximum output power

In [1], the spectrum usage in R19 is clearly stated:

FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

For the CW in UL spectrum, the output power is assumed as 23 dBm, which is same as PC3 UE. The 23 dBm output power take the co-existence into consideration and also avoid the SAR issue. We prefer only consider 23 dBm as maximum output power for CW due to the limited meeting cycle in this release.

Proposal 3: The maximum output power of CW is restricted to 23 dBm in R19.

- Unwanted emission/phase noise

In SI, both unwanted emission and phase noise are discussed as RF requirement. The unwanted emission is usually defined based on regulation and the phase noise mainly consider the impact to D2R reception. However, from requirement design perspective, it is not necessary to define them separately, and one composite emission mask should be enough.

Proposal 4: The phase noise of CW can be implicitly defined as part of CW unwanted emission mask, and no need to define phase noise as an individual requirement.

- Conclusion

In this contribution, we provide our preliminary views on CW RF requirement.

Proposal 1: For in-band operation, the frequency of CW should align with the NR channel raster.

Proposal 2: Take ±10ppm as the starting point for CW frequency error.

Proposal 3: The maximum output power of CW is restricted to 23 dBm in R19.

Proposal 4: The phase noise of CW can be implicitly defined as part of CW unwanted emission, and no need to define phase noise as an individual requirement.

- References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei)

- 3GPP TR 38.769

- ETSI EN 302 208