3GPP TSG-RAN WG4 Meeting # 114	R4-2500764

Athens, GR, 17th – 21st February, 2025

Source: 	vivo, SmarterMicro

Title: 	Iot NTN HPUE Tx Requirements and feasibility analysis

Agenda Item:	7.10.2.3.1

Document for:	Approval

Introduction

In RAN4#112, the HPUE for NTN [1] was started. A summary can be referenced in [2] and a WF [3] has been agreed. A paper [4] was submitted including an initial feasibility study and analysis for both NR-based and Iot-based NTN HPUE Tx requirements.

In RAN4#112bis, a comprehensive discussion paper including measurement results for 2 PAs are provided for Iot-NTN in [5]. A summary is in [6] and WF in [7] including the duty cycle related discussion.

In RAN4#113, a discussion paper including analysis on NB-Iot based NTN ACLR/SEM/MPR/P-MPR has been submitted in [8], and more measurement results were also submitted. A summary and WF was included in [9] and [10] in which more agreements has been included. The duty cycle based SAR control scheme was also attached.

In general, progress has been made in Tx requirements discussion, while duty cycle related discussion is still not diverse. In addition, considerable progress has been made for the requirements applicability part in which 2Tx requirements were precluded, and the requirements needs to be study is greatly simplified.

In this discussion paper, discussion is made in three areas:

- Remaining Tx requirements

- UE Architecture Analysis for NB-Iot based Iot-NTN (Feasibility)

- SAR and Duty cycle discussion

HPUE Impact for Tx Requirements

General

In last meeting, a list of agreements on “General & Feasibility” has been agreed in [10]. In order to make it more clear and provide a baseline for next stage discussion, the following table is provided to reflect the current status of what scenario and UE type has been considered in the requirements, and this will serve as a basic reference and starting point for further requirements discussion.

Updated Status: (For Information)

Table: Current status for requirements consideration and feasibility for Iot-NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

NB-Iot & eMTC based

MOP Tolerance

In last meeting, there were proposals on MOP tolerance for Iot-NTN to use LTE band 14 PC2 case, which is +2/-2dB. However, according to our observation this may not be quite typical case. The main problem is that although HPUE was defined in LTE, it is not well commercialized, even for TDD cases which introduced the HPUE earlier than LTE band 14.

It is only in NR, the power amplifiers for HPUE have been widely used in the market. In current implementation, one typical implementation is to reusing the NR RF chain for default power class for Iot-NTN, and it is likely that the PC2 would also do so. Although one can argue that the designed bandwidth may not be quite aligned, still we think that there is a benefit to fully exploit the NR components would have a benefit.

In addition, the NR-NTN also already have agreement of +2/-3dB tolerance for PC2, and using another tolerance in Iot-NTN seems not needed.

Observation 1: For PC2, a typical implementation of Iot-NTN is reusing NR Tx Chain to do transmission, thus there is benefit to align Iot-NTN MOP tolerance with NR HPUE.

Based on the observation, there is the following proposal:

Proposal 1a: Using +2/-3dB as MOP tolerance for Iot-NTN for PC2, which is aligned with NR and NR-NTN.

For power class 1, although the implementation might be more complicated, the tolerance currently for both LTE and NR is +2/-3dB. It is natural to using this as a starting point for Iot-NTN.

Proposal 1b: Using +2/-3dB as MOP tolerance for Iot-NTN for PC1 as starting point.

There were also discussion on +0.8dB upper tolerance for HD-FDD for FD-FDD UE, however, no conclusion was made. There is also a difficulty that this topic seems discussed in multiple WIs, and this has bring some difficulty in treating this topic. Thus it is proposed to have some coordination to treat this topic on only one thread.

Proposal 1c: Treat the topic of adding of upper tolerance for HD-FDD for FD-FDD UE only in one thread.

NB-Iot Based

ACLR

In last meeting, considering GSM adjacent deployment is not typical in Iot-NTN, and occupied channel bandwidth is already more stringent, the following agreement was made:

Agreement:

- Remove GSM_ACLR for NB-IoT based NTN HPUE.

For Rel-18, maintenance CR [11] was submitted to reflect this revision. For Rel-19, this could be reflected when requirements introduced. It is admitted this may be a maintenance issue now and the real discussion can be done in other agenda.

Observation 2: The removal of GSM_ACLR was also submitted in maintenance agenda to be considered in Rel-18.

SEM

In last meeting, a comprehensive analysis and tests were done for SEM in [8] and the following agreements were made in [10]

Issue 1-1-3: For NB-Iot based NTN, considering to extend the SEM for PC2 and PC1 as following

Agreement: For NB-Iot based NTN, consider PC3 SEM as a starting point. More evaluation can be considered.

Based on the measurements results in [8] based on GSM PA, the current SEM is the major bottleneck for higher power, and there are some reasons behind a more relaxed value for SEM. This is also beneficial for UE to reach higher value at least for some implementations.

However, there is no precedency or relaxed SEM for higher power class up till now, and it is doubtful on whether relaxation is suitable simply to accommodate some implementation. Currently we lack more detailed evaluation such as in co-existence to study how much real performance impact would be if more relaxed SEM is introduced.

In addition, if more adjustments would be considered, what about the other regional requirements? We generally do not support to consider more regional requirements in SEM since RAN4 already have mature procedure of A-MPR to consider them. However, if SEM is adjusted for other reason, may be the regional requirements might be also raised. This might add more complexity.

Proposal 2: Study more on the effect of SEM before conclude on whether reusing PC3 SEM for PC2/PC1 or not for NB-Iot based Iot-NTN.

MPR

MPR is generally should be studied after ACLR and SEM confirmed. The method of simulation and testing can be considered. In last meeting, only a very preliminary agreement was reached on the reference point of MPR as following:

Agreement:

- As the starting point, consider the center single tone allocation as MPR=0 reference point, and introduce MPR for edge single tone allocation.

We can use this as a starting point, and try to do test verification and/or simulation to study the MPR needed for various power classes.

During offline discussion, there are further concerns on the reference architecture need to be considered. E.g, what kind of RF chains and components would be typical in the future to carry NB-Iot based Iot NTN.

Proposal 3: Study the reference architectures more to have a more reasonable and comprehensive baseline for minimum MPR requirements.

P-MPR

In RAN4#113, based on the analysis in [8], the following agreement is reached in the WF[10]:

Issue 1-1-6: NB-Iot based Iot NTN,  P-MPR has not been defined.

- Proposals

  - Revise the configured transmitted Power section for NB-Iot Based Iot-NTN in 36.102 to incorporate P-MPR. Details FFS.

Agreement:

- Revise the configured transmitted Power section for NB-Iot Based Iot-NTN in 36.102 to incorporate P-MPR.

  - Details are FFS.

- Other solutions for SAR are not precluded.

In this meeting we provided a TP for this in 36.102 as following and also attached in Annex part:

The thinking is tried to minimise the revision and keep the redundancy to a minimum value.

In addition, it is believed that P-MPR as a basic SAR solution can apply to all the power classes. To keep better alignment, it is also suggested to revise Rel-18 specification on this point.

Proposal 4: For NB-Iot based Iot-NTN, discuss the proposal, confirm it is applicable for all the power classes, and discuss whether it can be defined from Rel-18.

eMTC-Based

As discussed in [8], for eMTC based Iot-NTN, the requirements are basically reused from eMTC, which is quite aligned with LTE in many basic requirements. So as a general guideline, legacy eMTC/LTE requirements can be reused for HPUE as starting point, except some special cases.

Although this is not officially set as an agreement, we can have some more proposals based on this principle. E.g.

Proposal 5: Reuse LTE SEM for eMTC-based Iot-NTN PC2 and PC1.

For ACLR, further guidelines are still under discussion. The MPR is also likely to be impacted.

In addition, the requirements applicability of eMTC based with half duplex for PC1 is still not confirmed yet for handheld/non-handheld. If only non-handheld is considered, which currently seems have more chance since this is anyway a wideband operation and challenging for PA. Then the reusing LTE PC1 requirements would be natural.

Observation 3: The ACLR and MPR for eMTC based Iot-NTN can be discussed after the set of ACLR guidelines. Requirements for eMTC based with half duplex for PC1 may also need further discussion.

Architecture Analysis for NB-Iot Based Iot-NTN

General

Current Status

In last meeting, a list of agreements on “General & Feasibility” has been agreed in [10]. The following agreements are reached:

- Agreement (Ad Hoc):

  - No 2Tx PC1/PC2 requirement will be specified for IoT NTN in Rel-19;

  - Only consider non-handheld for PC1 1Tx eMTC based with full duplex Iot-NTN

  - FFS whether requirements should be defined for PC1 1Tx handheld UE for NB-Iot based Iot-NTN

Based on the agreement, the following updated Status was also included in [10] for Information.

Table: Current status for requirements consideration and feasibility for Iot-NTN

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with half duplex | PC1 | 1Tx | FFS |
|  | PC2 | 1Tx | Both |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

It can be seen that currently, it is still not decided yet the PC1 requirements for NB-Iot based and eMTC based with half duplex Iot-NTN would be applicable for handheld or not. The non-handheld requirements has been agreed to be applied.

Currently, the main interest from industry focused on NB-Iot based Iot-NTN, and eMTC based did not get too much attention.

Section Introduction

In this section, a comprehensive analysis of possible implementation architectures are provided for NB-Iot based Iot-NTN. Here are some Main characteristics for NB-Iot based Iot-NTN:

- Narrowband (180KHz maximum, 15KHz/3.75Khz single-tone)

- Half-duplex only, no duplexer needed

- Suitable particularly for GEO, with more need for power.

Those characteristics are actually quite different with both NR/LTE/eMTC, and this is a unique case for communication. Based on extensive study and survey, there are currently three baseline considerations for implementation architecture:

- 1. GSM PA based

  - Reusing GSM Tx Chain including PA

- 2. NR PA based

  - Reusing NR Tx Chain including PA

- 3. Satellite communication proprietary PA based

  - Dedicated RF chain using PA designed specific for satellite communication

This section would make a comprehensive study based on Architecture al analysis and measurements. The data here were copied and referenced from earlier contributions.

GSM based (Arch-1)

GSM Characteristics

The GSM has already been used for more than two decades and still typical implementation in most current commercial mobile phones. Its basic features are very similar to NB-Iot. E.g.

- Narrow bandwidth (180Khz)

- TDD system, which is similar to half-duplex for NB-Iot

This gives it potential to have higher power than NR/LTE. Actually, GSM has a typical maximum output power (e.g. 33dBm) that is even higher compared to PC1. This provides a very good base for NB-Iot based Iot NTN re-use this RF chain. We have already submitted a number of contributions based on this previously.

However, this does not mean that the GSM chain can already fully ready to satisfy the most demanding PC1 requirements. Firstly, the OFDM used in NB-Iot has a higher PAPR than GMSK in GSM. Secondly, the tuned frequency for current PA are also not aligned with NTN usage.

Up to now, GSM might be most economical and widely available option to be reused, and may even be higher than NR in current stage. One potential problem is the usage of GSM is declining from operator side, and some vendors may in future choose to reduce GSM implementation. If this process is quick, reusing this link may bring further redundancy in some time in the future.

Observation 4a: Reusing GSM is a low cost and low implementation difficulty option in current stage for NB-Iot based Iot-NTN. The continual availability of GSM may have an impact in the future.

Measurements

A lot of measurements have been conducted in [5][8] based on one commercial Integrated LB/MB GSM PA module with antenna switch. Here we briefly summarized the key measurement results in them and calculate a power budget.

A simplified architecture is copied below:

For this module, approximate 1~2dB IL between the GSM PA and the test output port already considered, and a typical extra IL needed between test output port and antenna port is within 1dB.

The Test configurations are from 36.521-4, which is for conformance testing for NB-Iot based Iot-NTN. We used full tone(#10) by default and edge singe tone if explicitly stated. A SCS 15KHz is used in all the cases. Referenced from [5].

The UTRA ACLR (full tone) measurements is in [5] and copied below.

Considering the current PC3 UTRA_ACLR = 37dB and co-existence need is even smaller. The ACLR would not be limiting factor and the MOP of this module is 31dBm.

The SEM measurements were done in [8] with reference to PC3 SEM:

.

The measurement results are as below from [8] (Only a value closest to SEM mask is recorded):

The results show that the MOP of this module is 30/31dBm (depending on band), and Power backoff to 29dBm/28dBm needed for 255/256 to satisfy PC3 SEM for this module.

Power budget

Based on previous measurements, it can be seen this PA capability is well beyond PC2, but PC1 is still challenging. In order to study what a PC1 UE requirements would be, we made a table here to consider various factors. In it, we use a [3]dB MPR which is 1dB higher than current PC3 as an example.

|  | PC1 | Note |
| --- | --- | --- |
| Nominal MOP | [31] dBm |  |
| Tolerance | [+2/-3] dB |  |
| MPR (full tone) | [3] dB | Considering 2dB MPR in PC3, and different MPR = 0 point |
| IL margin | [1] dB | Between the test port and antenna port |
| Required Power range (full tone) = Nominal MOP + Tolerance – MPR + IL margin | [26 ~ 34] dBm |  |

According to the simple calculation, a PC1 UE need the PA module MOP to be within [26~34]dBm, and this range already considers the power tolerance.

Consider the PA measurements already done, the current achievable full tone MOP in the antenna port is 28dBm for 255 and 27dBm for 256 in the antenna port. It is noted that the 1dB difference compared to SEM measurements results from assumption that 1dB extra IL after test output port.

In addition, it should also be noted that this full tone is a worst case scenario that has at least 2~3dB MPR expected, it means that for zero MPR reference signal (e.g. center single tone), the achievable power would likely be 30~31dB , which already reached the nominal MOP for PC1, and well inside the required power range.

As another proof, it can be seen that for 255 edge single tone configuration, the measured SEM results already passed the requirement with 31dBm output power. If 1dB MPR could be assumed for this case to balance the 1dB IL, the 30dBm antenna port output power would means that a 31dBm MOP is already achieved for MPR = 0 case.

In all, the measured GSM PA already satisfy tentative required power range for PC1 in full tone, and can almost achieve nominal PC1 MOP for single tone case in certain band.

Based on this measurement results and budget analysis, the following observation can be made.

Observation 4b: Based on GSM implementation, PC1 is achievable, though power back off (e.g. MPR) is still needed. The power significantly higher than PC2 expected.

Admittedly, from a mass production point of view, this 0~1dB margin is not satisfactory, but potential optimization can be foreseen after PC1 defined. This performance is still significantly better compared to PC2.

In addition, optimization of PA can be expected with moderate cost increase. It should be noted that only after the specification exists, further optimization can be expected for components.

That is reason that we propose that PC1 should be applicable for handheld even if the current implementation

Observation 4c: With 3GPP requirements defined, optimization of GSM PA can be expected with moderate cost increase.

Proposal 6: GSM based solution is one possible implementation for NB-Iot based Iot-NTN for up to PC1.

NR based (Arch-2)

NR based solution for NB-Iot based Iot NTN is already being implemented for PC3. Further extend this architecture to PC2 or even higher power classes is another natural evolvement of implementation.

In addition, since NR is still at the very beginning of its life cycle, this implementation do not have the potential risk of being obsolete in the UE in the future like GSM have. Thus it is also being another preferred implementation architecture for NB-Iot based Iot NTN.

Currently PC2 PA for FDD bands are still not widely available yet. Since it is only in Rel-18 more FDD bands are defined for PC2. However, the PC2 HPUE for TDD had been commercialized for a few years, and based on preliminary analysis, it should not be too difficult to migrate some to NTN FDD bands and still keep similar power level. In this sense, it seems no serious difficulty would be foreseen for this implementation to achieve PC2.

Observation 5a: Reusing NR Tx Chain is one typical implementation for NB-Iot based Iot-NTN, and no major difficulty for PC2 is foreseen.

However, implementation of PC1 may still challenging for this implementation, though also potentially feasible.

In current MPR evaluation, the PC2 PA calibration point is 30dB output power, considering a 4dB post PA loss. This 30dB PA output power itself is already close to what a GSM can achieve. In addition, considering the fact that PC2 PA calibration point is a wideband OFDM signal, it is reasonable to believe for narrow band NB-Iot signal, there could be some more margin for power boosting. So it seems that a PC2 NR PA itself can potentially reach nominal PC1 power even with some margin.

However, the high insertion loss for NR is still an obstacle. Some dedicated implementations have to be made to bypass the duplexer and other components, to reduce the insertion loss, and reduce the difficulty of duplexer implementation. Theoretically these implementations should be feasible and have similar precedents, but these kinds of optimizations may still have some limitations especially when deep integration with TN design was considered.

Currently, PC2 PA is already commercially widely available for TDD band, but some migration is also possible.

Observation 5b: For PC1, it is also potentially feasible to re-use PC2 NR PA, considering the current 30dB output power assumption in 100MHz reference condition, and the power boosting potential in much narrower (200 or even 15KHz) Iot condition. However, extra UE implementation is needed to bypass duplexer and further reduce the insertion loss in case of NTN operation.

There is another potential merit for NR, is that this is the current most up to date technology, and not facing the potential problem of GSM of shrinking deployment and UE implementation in the years ahead.

Observation 5c: Reusing NR RF chain would not have the potential problem of shrinking GSM implementation.

Considering the current situation, the NR based solution is also an implementation way forward. It is up till now not too much study on this NR based solution yet, and based on those observations and situations, it is proposed to do more study on this.

Proposal 7: NR based solution for NB-Iot based Iot-NTN also need to be taken into account in the requirements study. More study is needed for PC1.

Proprietary based (Arch-3)

Existing Systems and components analysis

Currently there already exists a number of satellite communication systems that handheld Device can support such as Tiantong-1 in China mentioned in [4] and Iridium.

For Tiantong, it is using 3 GEO satellites, proprietary baseband and RF to provide low speed voice service. Currently deployed in band n256 with a working bandwidth of 22kHz, these parameters are quite similar to the current NB-IoT based NTN. There are already a lot of UE models to support this, e.g. P70 Pro+ (Huawei), X100 Ultra (vivo), Find X8 Ultra (Xiaomi), Xiaomi 14 Ultra (Xiaomi)…

Iridium has deployed handheld devices for decades, and the following data was found as an example:

According to the current data, both from data sheet and measurements, the proprietary PA dedicated for such satellite systems can reach a conducted power as high as 37dBm in hand held devices, which is considerably higher than PC1. This is attractive for satellite communication which really needs higher power.

Observation 6a: The merits of proprietary satellite PA based solution are:

- high power well beyond what PC1 requires

- already commercialized for multiple handheld devices

- can ensure the best performance

However, although powerful in the output power, one drawback of these proprietary components are the high cost. Currently they are several times more expensive than GSM or NR PA, which makes it less attractive from implementation. Theoretically, if the volume can be as large as NR PA, the price/cost of such proprietary PA can be reduced a similar level. In real scenario, this is not that likely to happen.

Another drawback for such proprietary PAs is that they are not suitable to be used in legacy TN NR/GSM RF chain, since the overall power consumption would be higher compared to legacy PA even the power fallback to legacy level. This makes it difficult to use this kind of PA for TN purpose, which further reduced the possibility to increase the volume of such proprietary satellite PA.

In this understanding, even some of components may still be re-used from TN, at least the proprietary PA can hardly be reused, and extra dedicated switching/selection are also needed.

Observation 6b: The drawbacks of proprietary based solution are:

- Difficult to reuse proprietary PA for TN operation

- Extra switching/selection is needed in UE implementation in case of NTN to use proprietary components

- The cost and complexity is high

Based on those conditions, it seems that reaching PC1 for handheld would not be challenging for this architecture. What is challenging is the cost and complexity, and is likely to make this architecture most “untypical” one among the three architectures discussed. Maybe only the high-end/flagship model would consider this arch in the future, and this makes the components performance not suitable for minimum requirements definition.

Proposal 8a: The actual performance of proprietary based solution may not suitable for minimum requirements definition since:

- high performance but higher cost and complexity

- may not be as typical as GSM/NR based solution.

Measurements

In [5], a set of measurements were done on a proprietary PA for ACLR performance. The conditions are as following:

- One commercial PA which is for TianTong / Beidou …

  - Multi-band PA cover:1980M~2010MHz,1608M~1624MHz and 1668M~1675MHz

  - A PAM designed for Satellite voice, SMS, and data services application with optimized PAE and harmonic suppression.

- Package Size: Approximate 3.6 x 3.6 x 0.7 mm

- Verification using typical PA configurations (VCC etc.)

- NB-Iot based NR signal is verified for n256 Freq = 1995MHz

- PA output power Prated = 29/33/37 dBm

- Configuration 36.521-4, Table 6.2B.2.4.1-1, Configuration ID: 1,3,6,10; + Annex A.2.3.

The results and observations are as following:

Observations 6c: The measurements for proprietary PA shows:

- Output conducted output power can reach 37dBm

- Sufficient margin for ACLR and EVM with no backoff needed

- SEM not verified yet

One of the possible omissions of current verification is SEM still not verified yet. Based on the verification of GSM based (Arch-1) solution, the SEM is likely to be bottleneck of the requirements, and ACLR generally is not that difficult to be reached. However, with more than 5dB back off, the SEM in PC1 level may not be a problem.

Proposal 8b: Further study the SEM performance of proprietary PA.

SAR and Duty cycle

Simplified Background

Basic SAR considerations

SAR measures the rate at which RF energy is absorbed by human tissue, and it is required for any wireless communication device that can transmit RF energy and is operated within 20cm of the human body. SAR usually have a 6-minute averaging period, and this is used in most cases.

There are many test configurations depending on intended use. E.g. for traditional phones, a “head/ear position” is typical, while a “Face/mouth position” is for devices used in front of face (like PTT devices). The difficulty of different positions is different for the device with the same MOP.

The duty cycle has been considered in SAR tests. The Time-averaged SAR = Peak SAR(100% duty cycle) × Duty Cycle. During the evaluation, usually the maximum supported duty cycle in a device or communication system supported would be tested.

Current Satellite system

Some satellite communication terminals have high transmission power, like 35-37 dBm. However, the duty cycle for these systems is typically not high. For example, the Tiantong system has a max duty cycle of around 20%. A low duty cycle can result in lower time averaged SAR values, even with high power.

In addition, different device intended use correspond to different test configurations and test positions, although these details may vary according to regulatory policies of each market.

Considerations for NR-NTN and Iot-NTN

In last meeting, some discussions on duty cycle related solution was discussed as documented in [9]. The reporting of maxUplinkdutycycle, P-MPR was not new, and there are legacy concerns continues. There are also new concepts and schemes reported, such as consider beam service time ratio as an extension of “duty cycle”, reporting UE working status. However, they are not yet complete solutions.

The legacy duty cycle method which simply reduce the power in case higher duty cycle may achieve lower power in more cases since it is actually much more restrict, and can not be directly used. In addition, most companies believe that the SAR management should be based on UE self assessment and can hardly by NW scheduling. This is basically contradicting with the idea that to control the power not to back off by network scheduling.

In order to overcame these difficulties, a more basic approach was tried.

What is two basic ways to control SAR? Answers might be:

- Reduce power (by P-MPR)

- Reduce transmission time (by ?)

Observation 7a: The two basic ways to control SAR is by either reduce the power, or reduce transmission time.

For reducing the power, we have P-MPR which is completely by UE self-assessment, and can achieve maximum flexibility and possible highest power by some algorithm.

For reducing the transmission time, if scheduling restriction from network side is difficult to be coordinate with UE, or confirming a certain maxUplinkdutycycle for a UE. Another possible way is to allow the UE voluntarily dropping some uplink transmissions, to reduce the transmission time. This is somewhat similar to autonomous denial in IDC framework, that in order to control the in-device co-existence between different RATs, sometimes the transmissions are allowed to be dropped by the UE.

Observation 7b: UE voluntarily dropping some uplink transmissions based on self-assessment can reduce SAR.

Of course, such behaviour can be controlled by network, that only be allowed if network chose this “reduce transmission time” scheme over “reduce power scheme”, and UE can indicate to the network whether it will enter such state that a transmission may be dropped at any time, to help the network to make some preparation for this kind of dropping, and understand those cases are UE voluntarily behaviour.

We may also find some use cases for the “UE working status”. However, the status may also be not that important or necessary, since this status can be part of UE self-assessment parameters that needs to be considered. This may somehow simplify the indications.

Based on this understanding, this may be a scheme simple enough, can be controlled by network and still useful.

Proposal 9: Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful.

Conclusion

HPUE Impact for Tx Requirements

Observation 1: For PC2, a typical implementation of Iot-NTN is reusing NR Tx Chain to do transmission, thus there is benefit to align Iot-NTN MOP tolerance with NR HPUE.

Proposal 1a: Using +2/-3dB as MOP tolerance for Iot-NTN for PC2, which is aligned with NR and NR-NTN.

Proposal 1b: Using +2/-3dB as MOP tolerance for Iot-NTN for PC1 as starting point.

Proposal 1c: Treat the topic of adding of upper tolerance for HD-FDD for FD-FDD UE only in one thread.

Observation 2: The removal of GSM_ACLR was also submitted in maintenance agenda to be considered in Rel-18.

Proposal 2: Study more on the effect of SEM before conclude on whether reusing PC3 SEM for PC2/PC1 or not for NB-Iot based Iot-NTN.

Proposal 3: Study the reference architectures more to have a more reasonable and comprehensive baseline for minimum MPR requirements.

Proposal 4: For NB-Iot based Iot-NTN, discuss the proposal, confirm it is applicable for all the power classes, and discuss whether it can be defined from Rel-18.

Proposal 5: Reuse LTE SEM for eMTC-based Iot-NTN PC2 and PC1.

Observation 3: The ACLR and MPR for eMTC based Iot-NTN can be discussed after the set of ACLR guidelines. Requirements for eMTC based with half duplex for PC1 may also need further discussion.

Architecture Analysis for NB-Iot Based Iot-NTN

GSM based

Observation 4a: Reusing GSM is a low cost and low implementation difficulty option in current stage for NB-Iot based Iot-NTN. The continual availability of GSM may have an impact in the future.

Observation 4b: Based on GSM implementation, PC1 is achievable, though power back off (e.g. MPR) is still needed. The power significantly higher than PC2 expected.

Observation 4c: With 3GPP requirements defined, optimization of GSM PA can be expected with moderate cost increase.

Proposal 6: GSM based solution is one possible implementation for NB-Iot based Iot-NTN for up to PC1.

NR Based:

Observation 5a: Reusing NR Tx Chain is one typical implementation for NB-Iot based Iot-NTN, and no major difficulty for PC2 is foreseen.

Observation 5b: For PC1, it is also potentially feasible to re-use PC2 NR PA, considering the current 30dB output power assumption in 100MHz reference condition, and the power boosting potential in much narrower (200 or even 15KHz) Iot condition. However, extra UE implementation is needed to bypass duplexer and further reduce the insertion loss in case of NTN operation.

Observation 5c: Reusing NR RF chain would not have the potential problem of shrinking GSM implementation.

Proposal 7: NR based solution for NB-Iot based Iot-NTN also need to be taken into account in the requirements study. More study is needed for PC1.

Proprietary based

Observation 6a: The merits of proprietary satellite PA based solution are:

- high power well beyond what PC1 requires

- already commercialized for multiple handheld devices

- can ensure the best performance

Observation 6b: The drawbacks of proprietary based solution are:

- Difficult to reuse proprietary PA for TN operation

- Extra switching/selection is needed in UE implementation in case of NTN to use proprietary components

- The cost and complexity is high

Observations 6c: The measurements for proprietary PA shows:

- Output conducted output power can reach 37dBm

- Sufficient margin for ACLR and EVM with no backoff needed

- SEM not verified yet

Proposal 8a: The actual performance of proprietary based solution may not suitable for minimum requirements definition since:

- high performance but higher cost and complexity

- may not be as typical as GSM/NR based solution.

Proposal 8b: Further study the SEM performance of proprietary PA.

SAR and Duty cycle

Observation 7a: The two basic ways to control SAR is by either reduce the power, or reduce transmission time.

Observation 7b: UE voluntarily dropping some uplink transmissions based on self-assessment can reduce SAR.

Proposal 9: Consider to introduce a scheme that UE voluntarily dropping uplink transmissions by self-assessment to control the SAR. This may be simple enough, can be controlled by network and still useful.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412099, Feasibility study and Initial analysis for NTN HPUE RF Tx Requirements, vivo, RAN4#112

[5] R4-2415790, Feasibility study and analysis for Iot NTN HPUE Tx Requirements, vivo, RAN4#112bis

[6] R4-2415229, Topic summary for [112bis][125] NR_IoT_NTN_HPUE_part2,vivo, RAN4#112bis

[7] R4-2417205, WF on HPUE for IoT NTN, vivo, RAN4#112bis

[8] R4-2418202, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, RAN4#113

[9] R4-2418149, Topic summary for [113][127] NR_IoT_NTN_HPUE_part2, Moderator (vivo), RAN4#113

[10] R4-2420404, WF on IoT NTN HPUE, vivo, RAN4#113

[11] R4- 2500766, CR on Remove GSM_ACLR for NB-Iot based Iot-NTN, vivo, Huawei, MediaTek, RAN4#114

- Annex A. TP for adding P-MPR in 36.102 for NB-Iot based Iot NTN

6.2B.4	Configured transmitted Power for category NB1 and NB2

For category NB1 and NB2 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 [7] shall apply except for the definition of PCMAX_L,c which would be substituted by:

	PCMAX_L,c = MIN { PEMAX,c ,  PPowerClass – MPRc – A-MPRc,  P-MPRc}

, wherein

-	The Maximum output power requirements are specified in subclause 6.2B.1

-	The MPR requirements are specified in subclause 6.2B.2

-	The A-MPR requirements are specified in subclause 6.2B.3.

-	P-MPRc is specified in subclause 6.2.5 of TS 36.101 [7]