3GPP TSG-RAN WG4 Meeting # 114	R4-2500765

Athens, GR, 17th – 21st February, 2025

Source: 	vivo

Title: 	Iot NTN HPUE Rx Requirements

Agenda Item:	7.10.2.3.2

Document for:	Approval

Introduction

In RAN4#112, the HPUE for NTN was started. A WF [1] has been agreed. In it, the key agreements related to Rx is :

Issue 4-1: General considerations

- Agreement

  - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class.

…

Issue 4-3: RSD for IoT-NTN

- Agreement:

  - For NB-Iot based Iot-NTN, no RSD needed for HPUE

  - For eMTC based Iot-NTN with half duplex, no RSD needed for HPUE

  - For eMTC based Iot-NTN with full duplex, further discuss RSD impact

From this, the main discussion scope is RSD for eMTC based Iot-NTN with full duplex. For NB-Iot based Iot-NTN and eMTC based Iot-NTN with half duplex, no Rx requirements impact foreseen.

In RAN4#112bis, not too much progress made in Iot NTN Rx part. In RAN4#113, some major progress are made, and the WF in [3] were agreed:

Issue 3-1-1: PC2 1Tx RSD (full duplex eMTC based)

- Proposals

  - N256:

    - 0 dB (vivo, oppo, Ericsson)

    - [1] dB (Qualcomm)

  - N255:

    - 0 dB (oppo)

    - Using n3 RSD as reference (Samsung, last meeting)

  - N254:

    - [0] dB (vivo, Samsung, Ericsson)

- WF

  - N254: [0] dB

  - Other bands FFS;

Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based)

- Proposals

  - Option 1: 0dB for n254/n255/n256 (vivo, oppo)

  - Option 2: others

- WF

  - [0] dB for n254/n255/n256

Issue 3-1-x: General for RSD

Way forward:

- Encourage companies to investigate and discuss the impact of Rx non-linearity in RSD evaluations.

In this paper, a discussion of the impact of HPUE to RSD requirements for Iot NTN are provided.

HPUE Impact for Iot-NTN RSD

General

As already agreed, for all the half duplex mode cases, no RSD is needed. Only for the case of eMTC based Iot NTN with full duplex, RSD is needed to be discussed.

In fact, the general guideline for RSD is somewhat similar to NR which as following:

Observation 1: RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

For Iot NTN, according to the previous agreement in [3], the scenarios to be considered is now as following:

Table 1: Status for requirements consideration and feasibility (eMTC based Iot NTN with full duplex)

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld |
| --- | --- | --- | --- |
| eMTC based with full duplex | PC1 | 1Tx | Non-handheld |
|  | PC2 | 1Tx | Both |

For the particular case of eMTC based Iot NTN with full duplex, actual the guidelines should be the same to NR-NTN, since the conditions are basically the same from the Channel BW.

Proposal 1: For the RSD of eMTC based Iot NTN with full duplex, only PC1 1Tx (Non-handheld) and PC2 1Tx (handheld and non-handheld) need to be discussed.

PC2

For PC2, an analysis has been provided in [7] for NR-NTN RSD analysis, and two proposals are provided:

Proposal 2a: For NR-NTN power class 2,

- For n256, RSD = 0dB, for both 2Tx and 1Tx cases.

- For n255, RSD is FFS

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

It is believed that the basic principles to derive requirements are the same, and these two proposals can be re-used except for minor difference. E.g. if in the future non-zero RSD is considered, different channel bandwidth should be considered.

Based on the previous proposals, and the already agreed 0dB RSD for band 254, the following proposal is as following:

Proposal 2a: For PC2 full duplex eMTC based Iot-NTN,

- For band 256, RSD = 0dB, (1Tx)

- For band 255, RSD is FFS

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

PC1

It is noted that for full duplex eMTC based Iot-NTN, the PC1 analysis is also similar to NR-NTN.

There were already agreements that 0dB RSD would assumed considering only 1Tx and non-handheld was considered.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

General

Observation 1: RSD definition would depend on Tx scenarios including power class, antenna numbers and UE type (handheld and non-handheld).

Proposal 1: For the RSD of eMTC based Iot NTN with full duplex, only PC1 1Tx (Non-handheld) and PC2 1Tx (handheld and non-handheld) need to be discussed.

PC2

Proposal 2a: For PC2 full duplex eMTC based Iot-NTN,

- For band 256, RSD = 0dB, (1Tx)

- For band 255, RSD is FFS

Proposal 2b: For power class 2, there is no differentiation of handheld and non-handheld for RSD.

PC1

Agreements already made.

- References

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[2] R4-2418149, Topic summary for [113][127] NR_IoT_NTN_HPUE_part2, Moderator (vivo), RAN4#113

[3] R4-2420404, WF on IoT NTN HPUE, vivo, RAN4#113

[4] R4-2418203, Iot NTN HPUE Rx Requirements, vivo, RAN4#113

[5] R4-2415789, Analysis for NR NTN HPUE Rx Requirements, vivo, RAN4#112bis

[6] R4-2412100, Initial analysis for NTN HPUE RF Rx Requirements, vivo, RAN4#112

[7] R4-2500763, NR NTN HPUE Rx Requirements, vivo, RAN4#114