3GPP TSG-RAN WG4 Meeting#114		R4-2500778

Athens, GR, Feb 17th -21st, 2025

Agenda item:	7.24.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on system parameters for ambient IoT

Document for:	Discussion

Introduction

In RAN4#113 meeting, in SI stage, conclusion in TR 38.769 about RF requirements is agreed. The WID [1] was approved. This contribution discussed system parameters for ambient IoT in the WI stage as follows.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

Discussion

System parameters

- Operating band

According to the SID, ambient IoT uses FR1 licensed spectrum in FDD. In our understanding, FR1 licensed spectrum FDD could be operating band in R19 WI.

Proposal 1: FR1 licensed spectrum could be operating band in R19 WI.

- Channel bandwidth

R2D

In SI stage, RAN1 has defined bandwidths for R2D: transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D. For an OFDM-based waveform with subcarrier spacing of 15kHz, Btx,R2D ≤ 12RBs, and would be down-selected among:

-	Alt 1: Including 180 kHz, 360 kHz

-	Alt 2: Integer multiple(s) of 180 kHz

-	Alt 3: Integer multiple(s) of the subcarrier spacing

Table 1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value.

Table1: Starting point for M values and the associated minimum Btx,R2D value

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

In RAN4, channel bandwidth includes transmission bandwidth and guard band for co-existence study. In addition, guard band for SFO (i.e., 104 or 105ppm) needs to be considered. Guard band for SFO should be 1/10 transmission bandwidth, guard band for co-existence study results. According to the WID, only D1T1-B needs to be considered, and in RAN4 co-existence study results, there are scenario 2-2 case f and case c found that co-existence problems. In our understanding, in these cases, guard band works hardly unless several MHz’s. We can solve these problems through network and resources scheduling, like that the number of active readers and devices selects the reader with the best signal quality. There is no need to consider guard band for co-existence study for R2D. Based on the above analysis, channel bandwidth for R2D needs to consider transmission bandwidth and guard band for SFO and guard band for co-existence study. Guard band could take a large value between caused by SFO and co-existence study. Channel bandwidth can be calculated using the following equation in NR: GBchannel = (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2. GBchannel=1/10 transmission bandwidth. Or channel bandwidth could be GBchannel plus transmission bandwidth.

Proposal 2: Channel bandwidth for R2D needs to consider transmission bandwidth and guard band for SFO. There is no need to consider guard band for co-existence study for R2D.

Proposal 3: Channel bandwidth for R2D could be two ways.
- Alt1: Reusing the following equation in NR: GBchannel = (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2. GBchannel=1/10 transmission bandwidth.

-Alt2: GBchannel plus transmission bandwidth, GBchannel=1/10 transmission bandwidth.

D2R

In SI stage, RAN1 has defined bandwidths for D2R: transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting D2R. For Btx,D2R of the D2R transmission associated with one/each single-tone of a carrier-wave, the following depict small frequency-shift. RAN1 has discussed 2SB transmission and 1SB transmission. Similarly to R2D channel bandwidth, D2R channel bandwidth includes transmission bandwidth and guard band for co-existence study and guard band for SFO. Generally, the maximum range of small frequency shift depends on the clock frequency. Therefore, transmission bandwidth is strongly related to clock frequency and FDM deployments of devices. FDM devices should be waited for RAN1’s conclusion.

Proposal 4: D2R channel bandwidth could be postponed until RAN1’s conclusion.

Proposal 5: Whether the channel bandwidth needs to be distinguished between 2SB transmission and 1SB transmission needs to be further discussed.

Conclusions

In this paper, we provide our views on system parameters for ambient IoT, we have made the following proposals.

Proposal 1: FR1 licensed spectrum could be operating band in R19 WI.

Proposal 2: Channel bandwidth for R2D needs to consider transmission bandwidth and guard band for SFO. There is no need to consider guard band for co-existence study for R2D.

Proposal 3: Channel bandwidth for R2D could be three ways:
- Alt1:Reusing the following equation in NR: GBchannel = (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2. GBchannel=1/10 transmission bandwidth.

-Alt2: GBchannel plus transmission bandwidth, GBchannel=1/10 transmission bandwidth.

Proposal 4: D2R channel bandwidth could be postpone until RAN1’s conclusion.

Proposal 5: Whether the channel bandwidth needs to be distinguished between 2SB transmission and 1SB transmission needs to be further discussed.

Reference

- RP-243326, New Work Item: Solutions for Ambient IoT(Internet of Things) in NR, Huawei, TSG RAN-P meeting #106