3GPP TSG-RAN4 Meeting #114 	R4-2500782

# Athens, Greece, 17-21 Feburary, 2025

Source: 	Huawei, HiSilicon

Title:  	RF requirements for Ambient IoT BS

Agenda Item:	7.24.2.1

Document for:	Approval

- Introduction

RAN WGs have completed a Rel-19 SI on solutions for Ambient IoT, documented in TR 38.769. Subsequently, a new WI on solutions for Ambient IoT was approved at RAN#106 [1]. The general scope and RAN4 objectives are copied below:

(Copied from RP-243326 [1])

General Scope

The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope:

- The overall objective shall be to standardize the following Ambient IoT device:

Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally.

- Deployment scenario 1 with Topology 1, according to D1T1-B.

- FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum.

- Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor.

- Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command).

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769:

  - Case 1-4 for D1T1-B

(Copied from RP-243326 [1])

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

This contribution we discuss the RF requirements for Type 1-C Ambient-IoT BS based on the work scope and the SI conclusion in TR38.769 [2].

- Ambient IoT BS TX requirements

  - Base station output power

The indoor Ambient IoT BS is type 1-C Micro NR BS characterised by requirements derived from Micro Cell scenarios with a BS to device minimum coupling loss equals to 53 dB.

The rated carrier output power per antenna connector (Prated,c,AC) of the A-IoT BS shall be as specified in table 2.1-1

Table 1: A-IoT BS rated output power limits

| BS class | Prated,c,AC |
| --- | --- |
| A-IoT BS | ≤ 38 dBm |

Proposal 1: Reuse the output power of Medium Range BS for indoor A-IoT BS, as shown in Table 1.

  - Output power dynamic

Output power dynamic requirement usually includes RE power control dynamic range requirement and total power dynamic range requirement.

For NR, the RE power control dynamic range is defined as the difference between the power of an RE and the average RE power for a BS at maximum output power for a specified reference condition.

For A-IoT, RE power control dynamic range is not needed because no RE level operation.

Proposal 2: RE power control dynamic range is not needed for A-IoT BS since there is no RE level operations in an A-IoT R2D carrier.

For NR, total power dynamic range is defined as the ratio of the maximum transmit power (when the number of RBs is full) to the minimum transmit power (when the number of RBs is one).

For A-IoT, based on SI conclusion, based on SI conclusion, Minimum Btx,R2D # of PRBs may be range from 1 to 4.

Copied from TR38.769[2]:

6.1.1.4	R2D bandwidths

…

Table 6.1.1.4-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value.

Note:	Depending on further study, the maximum value of M may be less than 32.

Note:	The performance can be better when transmission bandwidth greater than the minimum Btx,R2D, depending on device processing and transmit power constraint.

Table 6.1.1.4-1: Starting point for M values and the associated minimum Btx,R2D value

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

Unlike NR, in A-IoT, when M>=8, even if the number of PRBs exceeds 1, the A-IoT R2D carrier treats two or more PRBs as a single unit without performing operations at the PRB level. As a result, there's no need to define a total power dynamic range for A-IoT base stations.

Proposal 3: Total power dynamic range is not needed for A-IoT BS since there are no PRB level operations in an A-IoT R2D carrier.

  - Transmit ON/OFF power

2.3.1 Transmitter OFF power

Transmitter OFF power is defined as the mean power during the transmitter OFF period. Generally, it apply only to TDD system, as FDD systems are always in transmitter ON mode. Ambient IoT, which uses FDD spectrum, however, also employs a similar on-and-off switching mechanism.

A simplified D1T1-B A-IoT operation process diagram is shown as Figure 1.

Step 1: The energy signal charges the device.

Step 2: The A-IoT BS sends downlink data information to the device.

Step 3: The CW sends a carrier wave to the device

Step 4: Device backscattering the CW, then A-IoT BS receives and demodulates the backscattering signal.

Figure 1. General D1T1-B A-IoT work process diagram

During the D1T1-B A-IoT operation, A-IoT BS downlink signal alternates between ON and OFF states.When CW transmits carrier wave, the A-IoT BS R2D signal should be off. So it needs to define transmitter OFF power.The NR BS type 1-C NR requirements for transmitter OFF power can be reused.

Proposal 4: For A-IoT BS, the requirements for transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector.

2.3.2 Transmitter transient period

The transmitter transient period is the time period during which the transmitter is changing from the OFF period to the ON period or vice versa. As described in 2.3.1, A-IoT BS employs a TDD similar on-and-off switching mechanism. The NR BS type 1-C NR requirements for Transmitter transient period can be reused.

Proposal 5: A-IoT BS reuse the NR BS type 1-C NR requirements for Transmitter transient period, as shown in Table 2.

Table 2: Minimum requirement for the transmitter transient period for A-IoT BS

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

  - Transmit signal quality

Frequency error and modulation quality are potentially needed for A-IoT BS.

2.3.1 Frequency error

Frequency error performance mainly depends on PLL performance within transceiver chain and the timing and synchronization performance. Since A-IoT BS is micro BS, the frequency error of NR Medium Range BS can be reused.

Table 3: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Medium Range BS | ±0.1 ppm |

Proposal 6: Use the same frequency error requirement for A-IoT BS as specified for NR medium-range BS, as shown in Table 3.

2.3.2 Modulation quality

Consider effect to device modulation performance, it is need to define modulation depth, RF envelope ripple, RF envelop rise/fall time and RF Pulsewidth as in the standard of EPC RFID [3].

Copied from RFID Standard [3]:

Proposal 7: Define RF envelop requirements, including modulation Depth, RF Envelope Ripple, RF Envelope Rise Time, RF Envelope Fall Time and RF Pulse width for A-IoT BS.

However, A-IoT is with some differences with RFID in modulation and encoding. Modulation for A-IoT is OOK-4, for RFID is ASK. For encoding, According to WID [1], A-IoT R2D transmission uses only the Manchester line code as specified in TR 38.769. the encoding method will affect the RF pulse width.

Table 4 R2D difference between A-IoT and RFID

| R2D | AIoT | RFID |
| --- | --- | --- |
| Waveform | OFDM-based | Non-OFDM |
| modulation | OOK-4 | ASK |
| encoding | Manchester | PIE |

Figure 2 shows a simulation baseband waveform of R2D.The red is CP and the blue is R2D data.

Figure 2 Simulation time domain R2D waveform

Based on our baseband simulation waveform, it can be calculated the following values in Table 4 of RF envelop.

Table 4: A-IoT BS RF envelope parameters of baseband output

| Tari | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| 6.25 μs to 25 μs | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Mh = Ml | 0 |  | 0.1(A–B) | V/m orA/m |
|  | RF Pulsewidth | PW |  | 0.5Tari |  | µs |

Note that no other factors are included in the above values. To gain RF envelop requirements, besides baseband waveform, other effects resulted from CP, different chip length and digital filter and RF impairments may need to be considered.

Note that the above values only is depend on baseband waveform, and do not account for other factors. To determine RF envelope requirements, additional considerations such as CP effects, varying chip lengths, digital filtering, and RF impairments may need to be included.

Proposal 8:  The RF envelope requirements primarily depend on the baseband waveform but also need to consider additional factors like CP effects, varying chip lengths, digital filtering, and RF impairments.

  - Unwanted emissions

In SI phase, it is suggested that unwanted emission requirement should be defined because it is the requirement to limit the emission to other channel or system to make this equipment can co-exist with other equipments. OBW requirement, ACLR requirement, OBUE requirement, transmitter spurious emission requirement is potentially needed.

    - Occupied bandwidth

The occupied bandwidth is the width of a frequency band such that, below the lower and above the upper frequency limits, the mean powers emitted are each equal to a specified percentage /2 of the total mean transmitted power. See also Recommendation ITU-R SM.328 [3].

The value of /2 shall be taken as 0.5%.

A-IoT R2D supports only OOK-4 modulation and is OFDM based waveform.

Larger occupied bandwidth may be needed for OOK-4 modulation due to its wider side lobes compared to a legacy NR carrier.

Proposal 9:  Larger occupied bandwidth may be needed for OOK-4 modulation due to its wider side lobes compared to a legacy NR carrier.

2.5.2	Adjacent Channel Leakage Power Ratio

In SI it is assumed that the ACLR of A-IoT BS reuses the same values as legacy NB -IOT gNB (i.e. ACLR1:40dB，ACLR2:50dB).

For co-existence Scenario 2-1c (NR UE only outdoor) in TR38.769, simulation results show that even with A-IoT BS ACLR of 20dB, there is negligible degradation in NR DL throughput, both for average and cell-edge throughput.

Table 5: Scenario 2-1 co-existence conclusions

| Case No. | Aggressor | Victim |
| --- | --- | --- |
| c | Reader | NR DL |

| A-IoT BS ACLR(dB) | Co-existence conclusion |
| --- | --- |
|  | NR UE throughput delta @5% CDF and 50% CDF |
| 40 | NA |
| 20 | 0.08% |

Observation 1: For co-existence Scenario 2-1c (NR UE only outdoor) in TR38.769, simulation results show that even with A-IoT BS ACLR of 20dB, there is negligible degradation in NR DL throughput, both for average and cell-edge throughput.

Observation 2：RF envelop, ACLR and OBUE these requirements are interdependent and can be considered together.

Considering that there is no indoor NR UE, ACLR can be relaxed to 30/40dB for 1RB R2D transmission BW without impacting on outdoor NR UE.

Table 5: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| [ 180kHz] | BWChannel | A-IoT of same BW | Square (BWConfig) | 30 dB |
|  | 2 x BWChannel | A-IoT of same BW | Square (BWConfig) | 40 dB |
| NOTE 1: Other CBW is FFS. |  |  |  |  |

Proposal 10: Specify 1RB channel BW A-IoT carrier ACLR 1/2 as 30/40dB for A-IoT BS as Table 5.

    - Operating band unwanted emissions

Consider the absence of indoor NR UEs and the large side lobes produced by OOK-4 modulation, OBUE can be further investigated.

Proposal 11: Consider the absence of indoor NR UEs and the large side lobes produced by OOK-4 modulation, OBUE can be further investigated.

2.5.4	Transmitter spurious emissions

The transmitter spurious emission limits shall apply from 9 kHz to 12.75 GHz, excluding the frequency range from ΔfOBUE below the lowest frequency of each supported downlink operating band, up to ΔfOBUE above the highest frequency of each supported downlink operating band, where the ΔfOBUE is defined in table 6.6.1-1. For some operating bands, the upper limit is higher than 12.75 GHz in order to comply with the 5th harmonic limit of the downlink operating band, as specified in ITU-R recommendation SM.329.

For A-IoT BS, the transmitter spurious emissions should meet the NR BS requirements specified in Table 6.6.5.2.1-1 and Table 6.6.5.2.1-2 of TS 38.104[6].

Copied from TS38.104:

Table 6.6.5.2.1-1: General BS transmitter spurious emission limits in FR1, Category A

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1, Note 4 |
| 150 kHz – 30 MHz |  | 10 kHz | Note 1, Note 4 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz 12.75 GHz | -13 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz |  | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | -13 dBm | 1 MHz | Note 1, Note 2, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the DL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz.. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. NOTE 5: Applies for Band for which the upper frequency edge of the DL operating band is greater than 5.2 GHz. |  |  |  |

Table 6.6.5.2.1-2: General BS transmitter spurious emission limits in FR1, Category B

| Spurious frequency range | Basic limit | Measurement bandwidth | Notes |
| --- | --- | --- | --- |
| 9 kHz – 150 kHz |  | 1 kHz | Note 1, Note 4 |
| 150 kHz – 30 MHz | -36 dBm | 10 kHz | Note 1, Note 4 |
| 30 MHz – 1 GHz |  | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz |  | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the DL operating band in GHz | -30 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | - 30 dBm | 1 MHz | Note 1, Note 2, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the DL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz. NOTE 4: This spurious frequency range applies only to BS type 1-C and BS type 1-H. NOTE 5: Applies for Band for which the upper frequency edge of the DL operating band is greater than 5.2 GHz. |  |  |  |

Proposal 12: The transmitter spurious emissions for A-IoT BS should meet the NR BS requirements specified in Table 6.6.5.2.1-1 and Table 6.6.5.2.1-2 of TS 38.104.

- Ambient IoT BS Rx requirements

  - Reference sensitivity level

The conclusion of reference sensitivity level for A-IoT BS in SI is as follows: [2]

Copied from TR38.769:

Regarding the reference sensitivity level for A-IoT BS, RAN4 reached the following consensus:

- For BS type 1-C,

PREFSENSdBm=-174dBm+10×log10(BW)+NF+IM+SNR+[ desens target]

-	BW is the transmission bandwidth for D2R.

-	NF is the noise figure of A-IoT BS, equal to 10 dB for Medium Range BS.

-	IM is the implementation margin, equal to 2dB.

-	SNR is the SNR value for D2R decoding, the detailed metrics would be discussed in the WI phase.

-	Desens target is considered for CW signal interference cancellation in D1T1 scenario. The desens level for CW signal cancellation might be different for D1T1-A1/D1T1-A2 and D1T1-B. For D1T1-A2, CW signal interference is coming from self-interference and also potential CW interference from other nodes transmitting CW signal. For D1T1-A1, the CW signal interference is coming from other nodes transmitting CW inside the topology. For D1T1-B, the CW signal interference is coming from CW nodes transmitting outside topology.

Since the D2R channel bandwidth remains undefined, 15 kHz is considered the baseline as the minimum bandwidth option. Therefore, it is proposed to define the reference sensitivity level specifically for 15 kHz D2R channel BW.

Proposal 13: Define the reference sensitivity level specifically for 15 kHz D2R channel BW at least.

To determine the reference sensitivity level, LLS is needed. The D2R modulation SNR should be simulated. Each company provided simulation results, and average will be done. For this simulation, the following parameters are proposed:

Proposal 14: Use the simulation assumptions of D2R listed in Table 6 for LLS to simulate the D2R SNR.

Table 6: D2R LLS simulation parameters

| Parameter | Value |
| --- | --- |
| Channel Model | AWGN |
| SFO | [104 ppm~105 ppm] |
| Transmission bandwidth (double sideband) | 15kHz |
| TBS | 96 bits |
| FEC | 1/3 Convolutional Code |
| Line code | Manchester |
| Repetition | None |
| Carrier-wave waveform | Unmodulated single tone |

  - Dynamic range

Dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal inside the received BS channel bandwidth. For Ambient IoT indoor deployment, the co-channel interference comes from co-channel device transmission and CW interference.

Like in TR38.817-2, the interference level and desired signal can be calculated as follows:

PIntf = -174dBm/Hz+10*log10(BWconfig) + NF + 20 (1)

Pwanted = -174dBm/Hz+10*log10(BWconfig) + NF + 20+SNR+IM (2)

Where

-	BWconfig is the configuration BS channel bandwidth;

-	NF is noise figure of A-IoT BS receiver in dB, where 10dB is assumed for MR;

-	IM is implementation margin, IM is assumed to be 2.5dB;

-	SNR is the value from LLS;

After obtaining the D2R SNR through LLS simulation, the dynamic range can be calculated accordingly.

  - In-band selectivity and blocking

    - Adjacent Channel Selectivity

There is no need to define ACS requirement because there is no multi-device scenario in Rel-19.

Proposal 15: No ACS requirement for A-IoT BS.

    - In-band blocking

RAN4 reached the following consensus in SI: In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, the wanted signal configuration and its corresponding configuration could be postponed to WI phase.

Proposal 16: Reuse the in-band blocking requirement from NB-IoT standalone Medium Range BS for A-IoT BS.

Copied from TS36.104

Table 7.6.1.1-3c: Blocking performance requirement for Medium Range BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7, 11, 13-14,18,19, 21, 24, 42, 43, 48, 54, 65, 66, 70, 103, 106 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |

…

Table 7.6.1.1-4: Interfering signals for blocking performance requirement for NB-IoT standalone operation

| NB-IoT channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 0.2 | ±7.5 | 5MHz E-UTRA signal |

  - Out-of-band blocking

Regarding Out-of-band blocking requirement, the existing out-of-band blocking requirement may be still applicable for A-IoT BS.

Proposal 17: Reuse the out-of-band blocking requirement from NB-IoT standalone Medium Range BS for A-IoT BS.

Copied from 36.104

Table 7.6.1.1-3c: Blocking performance requirement for Medium Range BS for NB-IoT standalone operation

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-5, 7, 11, 13-14,18,19, 21, 24, 42, 43, 48, 54, 65, 66, 70, 103, 106 | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-4 | See table 7.6.1.1-4 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 (Note 2) | PREFSENS +6dB (Note 1) |  | CW carrier |

…

Table 7.6.1.1-4: Interfering signals for blocking performance requirement for NB-IoT standalone operation

| NB-IoT channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 0.2 | ±7.5 | 5MHz E-UTRA signal |

  - Receiver spurious emissions

Existing RX spurious emission requirement comes from the regulation, so it can be reused for A-IoT.

Copied from TS38.104:

Table 7.6.2-1: General BS receiver spurious emissions limits

| Spurious frequency range | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -57 dBm | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz | -47 dBm | 1 MHz | Note 1, Note 2 |
| 12.75 GHz – 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 dBm | 1 MHz | Note 1, Note 2, Note 3 |
| 12.75 GHz - 26 GHz | -47 dBm | 1 MHz | Note 1, Note 2, Note 5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: Applies for Band for which the upper frequency edge of the UL operating band is greater than 2.55 GHz and less than or equal to 5.2 GHz. NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the BS transmitter operating band to ΔfOBUE above the highest frequency of the BS transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.6.1. For multi-band connectors, the exclusion applies for all supported operating bands. NOTE 5: Applies for Band for which the upper frequency edge of the UL operating band is greater than 5.2 GHz. |  |  |  |

Proposal 18: The receiver spurious emissions for A-IoT BS can reuse the NR BS requirements specified in Table 7.6.2-1 of TS 38.104.

  - Receiver intermodulation

    - general intermodulation

For general intermodulation, it is assumed that there is no NR interference indoors, so there's no need to define it.

Proposal 19: No need to define general intermodulation for A-IoT BS since it is assumed that there is no NR interference indoors.

    - narrowband intermodulation

Regarding the narrowband intermodulation requirement, it can reuse the requirement of NB Micro BS.

Copied from TS36.104

Table 7.8.1-6c: Narrowband intermodulation performance requirement for Medium Range BS for NB-IoT standalone

| Channel bandwidth of the lowest/highest carrier received [MHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 0.2 | PREFSENS + 6dB* | -47 | ±340 | CW |
|  |  | -47 | ±880 | 5MHz E-UTRA signal, 1 RB** |
| Note*: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.1-5c. Note**: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. Note***: If a BS RF receiver fails the test of the requirement, the test shall be performed with the CW interfering signal frequency shifted away from the wanted signal by 180 kHz and the E-UTRA interfering signal frequency shifted away from the wanted signal by 360 kHz. If the BS RF receiver still fails the test after the frequency shift, then the BS RF receiver shall be deemed to fail the requirement. |  |  |  |  |

Proposal 20: The Narrowband intermodulation performance requirement for A-IoT BS can reuse the Medium Range BS for NB-IoT standalone requirements specified in Table 7.8.1-6c of TS 36.104.

  - In-channel selectivity

In-channel selectivity (ICS) is a measure of the receiver ability to receive a wanted signal at its assigned resource block locations in the presence of another in-channel wanted signal received at a much larger power spectral density.

NB-IoT only defines ICS for in-band operation and does not define it for standalone operation.

No need to define ICS for A-IoT BS.

Proposal 21: There's no need to define ICS for A-IoT BS.

- Summary

In this contribution, we give our consideration of A-IoT BS requirements, and has the following proposals:

For TX requirements:

Proposal 1: Reuse the output power of Medium Range BS for indoor A-IoT BS, as shown in Table 1.

Table 1: A-IoT BS rated output power limits

| BS class | Prated,c,AC |
| --- | --- |
| A-IoT BS | ≤ 38 dBm |

Proposal 2: RE power control dynamic range is not needed for A-IoT BS since there is no RE level operations in an A-IoT R2D carrier.

Proposal 3: Total power dynamic range is not needed for A-IoT BS since there are no PRB level operations in an A-IoT R2D carrier.

Proposal 4: For A-IoT BS, the requirements for transmitter OFF power spectral density shall be less than -85 dBm/MHz per antenna connector.

Proposal 5: A-IoT BS reuse the NR BS type 1-C NR requirements for Transmitter transient period, as shown in Table 2.

Table 2: Minimum requirement for the transmitter transient period for A-IoT BS

| Transition | Transient period length (µs) |
| --- | --- |
| OFF to ON | 10 |
| ON to OFF | 10 |

Proposal 6: Use the same frequency error requirement for A-IoT BS as specified for NR medium-range BS, as shown in Table 3.

Table 3: Frequency error minimum requirement

| BS class | Accuracy |
| --- | --- |
| Medium Range BS | ±0.1 ppm |

Proposal 7: Define RF envelop requirements, including modulation Depth, RF Envelope Ripple, RF Envelope Rise Time, RF Envelope Fall Time and RF Pulse width for A-IoT BS.

Proposal 8:  The RF envelope requirements primarily depend on the baseband waveform but also need to consider additional factors like CP effects, varying chip lengths, digital filtering, and RF impairments.

Proposal 9:  Larger occupied bandwidth may be needed for OOK-4 modulation due to its wider side lobes compared to a legacy NR carrier.

Observation 1: For co-existence Scenario 2-1c (NR UE only outdoor) in TR38.769, simulation results show that even with A-IoT BS ACLR of 20dB, there is negligible degradation in NR DL throughput, both for average and cell-edge throughput.

Observation 2：RF envelop, ACLR and OBUE these requirements are interdependent and can be considered together.

Proposal 10: Specify 1RB channel BW A-IoT carrier ACLR 1/2 as 30/40dB for A-IoT BS as Table 5.

Table 5: Base station ACLR limit

| BS channel bandwidth of lowest/highest carrier transmitted BWChannel (kHz) | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACLR limit |
| --- | --- | --- | --- | --- |
| [ 180kHz] | BWChannel | A-IoT of same BW | Square (BWConfig) | 30 dB |
|  | 2 x BWChannel | A-IoT of same BW | Square (BWConfig) | 40 dB |
| NOTE 1: Other CBW is FFS. |  |  |  |  |

Proposal 11: Consider the absence of indoor NR UEs and the large side lobes produced by OOK-4 modulation, OBUE can be further investigated.

Proposal 12: The transmitter spurious emissions for A-IoT BS should meet the NR BS requirements specified in Table 6.6.5.2.1-1 and Table 6.6.5.2.1-2 of TS 38.104.

For RX requirements:

Proposal 13: Define the reference sensitivity level specifically for 15 kHz D2R channel BW at least.

Proposal 14: Use the simulation assumptions of D2R listed in Table 6 for LLS to simulate the D2R SNR.

Table 6: D2R LLS simulation parameters

| Parameter | Value |
| --- | --- |
| Channel Model | AWGN |
| SFO | [104 ppm~105 ppm] |
| Transmission bandwidth (double sideband) | 15kHz |
| TBS | 96 bits |
| FEC | 1/3 Convolutional Code |
| Line code | Manchester |
| Repetition | None |
| Carrier-wave waveform | Unmodulated single tone |

Proposal 15: No ACS requirement for A-IoT BS.

Proposal 16: Reuse the in-band blocking requirement from NB-IoT standalone Medium Range BS for A-IoT BS.

Proposal 17: Reuse the out-of-band blocking requirement from NB-IoT standalone Medium Range BS for A-IoT BS.

Proposal 18: The receiver spurious emissions for A-IoT BS can reuse the NR BS requirements specified in Table 7.6.2-1 of TS 38.104.

Proposal 19: No need to define general intermodulation for A-IoT BS since it is assumed that there is no NR interference indoors.

Proposal 20: The Narrowband intermodulation performance requirement for A-IoT BS can reuse the Medium Range BS for NB-IoT standalone requirements specified in Table 7.8.1-6c of TS 36.104.

Proposal 21: There's no need to define ICS for A-IoT BS.

- References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, RAN1 Vice-chair (Huawei), RAN#106 Dec.,2024

[2]3GPP TR 38.769 V2.0.0(2024-12), Study on solutions for ambient IoT (Internet of Things)

[3] EPC® Radio-Frequency Identity Generation-2 UHF RFID Standard - Specification for RFID Air Interface Protocol for Communications at 860 MHz  - 930 MHz, Release 3.0,Jan 2024