3GPP TSG-RAN WG4 Meeting # 114						R4-2500785

# Athens, Greece, 17-21 Feburary, 2025

Source:	Huawei, Hisilicon

Title:	draft TP to TR 38.719-02-01 include CA_n7-n40

Agenda item:			6.3.3(NR_CADC_R19_2BDL_xBUL)

Document for:	Approval

1 Introduction

The new basket WID was approved in RAN #106 meeting [1]. This contribution provides a text proposal for TR 38.719-02-01 to include CA_n7-n40 BCS 4 and 5. Since CA_n7-n40 BCS 0 already exists in TS38.101-1, in this TP only cross band analysis is performed.

2 References

[1]RP-242754, Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei, HiSilicon, Dec.,2024, RAN#106

Text Proposal

<Start of Text Proposal>

5.x CA_n7-n40

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n40 | 2300 MHz –2400 MHz | 2300 MHz –2400 MHz | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n40A | CA_n7A-n40A | n7 | n7 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n40 | n40 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |

5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 summarizes frequency ranges where cross band isolation may occur for CA_ n7-n40.

Table 5.x.1.3-1: Cross-band isolation analysis

| Bands | n7 |  | n40 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 2500 | 2570 | 2300 | 2400 |
| fDL (MHz) | 2620 | 2690 | 2300 | 2400 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 5 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 2450 | 2620 | 2200 | 2500 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 2400 | 2670 | 2100 | 2600 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 2350 | 2720 | 2000 | 2700 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 2300 | 2770 | 1900 | 2800 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 2250 | 2820 | 1800 | 2900 |
| Analysis | There is overlap of the band n7 ACLR3~ACLR5 range with the band n40 DL. Therefore, the cross-band isolation MSD should be studied. |  | There is overlap of the band n40 ACLR3~ACLR5 range with the band n7 DL. Therefore, the cross-band isolation MSD should be studied. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

For the cross-band overlapping from uplink n7 to n40 downlink, and the cross-band overlapping from uplink n40 to n7 downlink, Table 7.3A.6-1 in 38.101 has specified MSD value as follows:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n7 | n40 | 2525 | 50 | 15 | 45 (RBstart=0) | 2397.5 | 5 | 3.7 | >ACLR2 |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2622.5 | 5 | 22.3 | >ACLR2 |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2645 | 50 | 15.6 | >ACLR2 |

No additional MSD is needed.

5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n7-n40, the TIB,c and RIB,c values have been specified in 38.101-1.

5.x.1.5	REFSENS requirements

No additional MSD is needed.

5.x.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

<End of Text Proposal>