3GPP TSG-RAN WG4 Meeting # 114	R4- 2500797

Athens, Greece, 17th – 21st February 2025

Source: 	China Telecom

Title: 	On SAR Solution of R19 HPUE for CA

Agenda Item:	7.1.1.1.1

Document for:	Discussion

Introduction

In meeting RAN4#113，we made following agreements[1] and also had a starting point from paper [2]&[3]. We generally support the framework in paper [2]&[3] for introducing PC1.5 duty cycle SAR solutions but we find several problems needed to be clarified.

Agreement: - Introduction of new duty-cycle IE is not considered - For ENDC (FDD+FDD), only reply on P-MPR, and there is no further spec changes needed - Following as starting point - For PC1.5 of intra-band contiguous and non-contiguous CA: …… - For PC1.5 of inter-band CA: …… - For EN-DC (FDD+TDD): …… - For EN-DC (TDD+TDD): ……

So in this paper we provide our view and feasible scheme.

Discussion

- PC1.5 of intra-band contiguous and non-contiguous CA

For the new added part about PC1.5 of intra-band contiguous and non-contiguous CA in paper [2], if both the UE capabilities on PC1.5 and PC2 should be adopted on determining max duty cycle threshold, we believe it lacks the condition about capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 at the first “else if ” judgment statement. And the description should like below:

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.1-1/2 and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1/2 is 23dBm or lower; - shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2; - else if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less t24han one radio frame); or - if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1/2 is 26dBm or lower; - shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2; - else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2.

Proposal1: If both the UE capabilities on PC1.5 and PC2 should be adopted on determining max duty cycle threshold, the condition to determine if fall back to PC2 should contain the maxUplinkDutyCycle-PC1dot5-MPE-FR1 capability.

- PC1.5 of TDD+TDD EN-DC

The PC1.5 duty cycle solutions on TDD+TDD EN-DC was captured as follows in paper [3], but it may cause some misunderstanding.

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class: – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4; – Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame): – shall apply all requirements for the power class 2supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4. Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame): – shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

From the perspective of conditional statement, when we make a solution with multiple cases, if the above “else if” condition is satisfied, it will not execute those below statements.

And in the description in paper [3], the first “else if” conditional statement can be summarized as duty cycle is less than or equal to 30% or maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16, and the second “else if” conditional statement can be summarized as duty cycle is less than or equal to 15% or maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16. Obviously the first one contains the second one and there will be such a situation that PC1.5 UE fall back to PC2(fall into the first “else if” statement) when we don’t expect it to. And this misunderstanding can be corrected by switching the positions of the two conditional statements as below:

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class: – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or – if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or – if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4; – else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame): – shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4. – Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or – if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame): – shall apply all requirements for the power class 2supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

Proposal2: For PC1.5 TDD+TDD EN-DC duty cycle solution, the first “else if” conditional statement contains the second one and there will be such a situation that PC1.5 UE fall back to PC2(fall into the first “else if” statement) when we don’t expect it to, and this misunderstanding can be corrected by switching the positions of the two conditional statements.

Although the reflection in spec is not strict programming code, we should follow the logical relation. And on the basis of paper [2]&[3], we will also submit two draftCRs for PC1.5 duty cycle solution on CA and EN-DC for 38.101-1&101-3.

Conclusion

Proposal1: If both the UE capabilities on PC1.5 and PC2 should be adopted on determining max duty cycle threshold, the condition to determine if fall back to PC2 should contain the maxUplinkDutyCycle-PC1dot5-MPE-FR1 capability.

Proposal2: For PC1.5 TDD+TDD EN-DC duty cycle solution, the first “else if” conditional statement contains the second one and there will be such a situation that PC1.5 UE fall back to PC2(fall into the first “else if” statement) when we don’t expect it to, and this misunderstanding can be corrected by switching the positions of the two conditional statements.

References

- R4-2420329, WF on HPUE for CA in TN, Samsung, RAN4#113

- R4-2419448, Discussion on SAR solution for Rel-19 HPUE, Huawei, Hisilicon, RAN4#113

- R4-2418744, Further discussion on SAR solutions & general MSD rules, CHTTL, RAN4#113