3GPP TSG-RAN WG4 Meeting # 114	R4-2500804

# Athens, Greece, 17th - 21th, Feb 2025

Source: 	Samsung

Title: 	On intra-band ULCA

Agenda Item:	7.1.1.1.2

Document for:	Discussion

- Introduction

In RAN4#112bis, for intra-band contiguous ULCA, it was agreed that MPR/A-MPR requirements are to be defined based on equal PSD assumption, but it does not preclude companies to present the analysis for unequal PSD case to give the group a picture on how far MPR values could go with unequal PSD. The agreements are excerpted as following [1].

In RAN4#113, further discussion on MPR evaluation assumption was organized for intra-band non-contiguous ULCA. The agreements are excerpted as following [2], equal PSD was adopted as starting point and companies are also encouraged to check the MPR difference with 6dB, which is basically aligned with contiguous case.

Conformance test issue was also discussed a bit in last meeting, and companies agreed the PSD condition for MPR derivation should be informed to RAN5. Another issue to be further discussed is the MPR applicability.

We further present our views on conformance test and MPR applicability in this paper.

- 2. Discussion

2.1 Conformance test for intra-band CA MPR

As elaborated in our paper [3] of last meeting, the following two observations were reached. In summary, our finding is that current RAN5 test configurations/condition does not require “equal PSD” or “equal power split”.

So we think it could be helpful to inform RAN5 the PSD assumption adopted by RAN4 for MPR values derivation, and it is up to RAN5 to decide whether it should be reflected in the conformance test. In addition, we think, it might be good to mention the assumption for intra-band contiguous CA as well. For both C and NC CA, we are still in support of deriving MPR based on equal PSD assumption.

Proposal 1: Inform RAN5 that for both intra-band C and NC CA, the PSD assumption adopted by RAN4 for MPR values derivation for all power classes is “equal PSD”.

2.2 MPR applicability for intra-band NC CA

For contiguous CA, since it is more like a larger single CC, so typically equal PSD is also used in the field. While for NC CA, it is impossible to guarantee PSD is always equal in any cases especially when UE is equipped with dual 26dBm PA for PC1.5.

We are supportive to Ericsson’s proposal, which means the MPR values captured in spec are applicable for all PSD conditions regardless the MPR is derived from which condition of PSD, it should be the understanding for all power classes. But it is not necessarily mean RAN5 should test the different unbalanced PSD cases.

Proposal 2: Inform RAN5 the following MPR applicability.

- For intra-band NC CA with all power classes, MPR values captured in spec are applicable for all PSD conditions regardless of the MPR is derived from which condition of PSD, but it is not necessarily mean RAN5 should test the different unbalanced PSD cases.

Considering the two observations we made on the current RAN5 test conditions, and above two proposals, we donnot think RAN5 change is really needed, but it should be left for RAN5 to discuss and decide.

- 3. Conclusion

Proposal 1: Inform RAN5 that for both intra-band C and NC CA, the PSD assumption adopted by RAN4 for MPR values derivation for all power classes is “equal PSD”.

Proposal 2: Inform RAN5 the following MPR applicability.

- For intra-band NC CA with all power classes, MPR values captured in spec are applicable for all PSD conditions regardless of the MPR is derived from which condition of PSD, but it is not necessarily mean RAN5 should test the different unbalanced PSD cases.

- 4. Reference

[1] R4-2414277, WF on HPUE for CA in TN, Samsung, RAN4#112

[2] R4-2420329, WF on HPUE RF requirements, Samsung, RAN4#113

[3] R4-2418340, Conformance test consideration for intra-band UL NCCA with unequal PSD, Samsung, RAN4#112bis