3GPP TSG-RAN WG4 Meeting # 114	R4-2500807

# Athens, Greece, 17th - 21th, Feb, 2025

Source: 	Samsung

Title: 	Discussion on UE and NW behavior for non-collocated deployment

Agenda Item:	7.5.2.2

Document for:	Discussion

- Introduction

We continue to discuss the remaining open issues about new BS signalling and associated UE behavior in this paper.

The agreements of last meeting are captured in WF [1]. We try to keep this paper short, more details can be found in our paper [2] of last meeting.

- Discussion/ Conclusion

In previous meeting, we discussed UE behavior from two aspects (i.e., “switching mode” and “initial mode”):

- UE initial configuration/behavior on collocated or non-collocated deployment, at the initial connection mode

- UE switching behavior between collocated and non-collocated deployment

One agreement was made on “switching mode” in last meeting as following,

- We support/propose this because our preference is that for the "switch from Type-4 to Type-1 with 6/8 L", UE first switch from Type-4 to Type-1 with 4L, then do MIMO layer re-config to 6/8 L → This is like a "2-step" approach.

- While companies may prefer "1-step" approach, by using maxMIMO-layers=6 or 8 to directly conduct UE to switch, if understand correctly.

By reviewing again Rel-18 RAN2 and RAN4 spec and recalling what had been discussed in Rel-18, we found that there is actually no description on the UE behavior when maxMIMO-layers＝4 in both RAN2 and RAN4. RAN2 spec is excerpted as below for easy reference. For RAN4 spec, we recalled we did have some description in the initial Rel-18 CRs for maxMIMO-layers＝4, but failed to reach agreement thus relevant description were all removed in the end, basically left a hole in RAN4 spec.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

To avoid same debate in Rel-19, we think same handling as in Rel-18 can be adopted in Rel-19, meaning that at least in both Rel-19 RAN2 and RAN4 spec, no description on the UE behavior when maxMIMO-layer＞4 （=6/8）. With that, both "1-step approach" and "2-step approach" are allowed. And adopting one unified solution for both “switching mode” and “connected more” is also acceptable for us.

With above, we propose:

Proposal 1: Confirm the following understanding for the usage of maxMIMO-layers and the new Rel-19 NW signaling:

1）maxMIMO-layers ＞4, it is collocated → This is not necessary to be reflected in RAN2/RAN4 spec

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, follow Rel-18 specification →This was already agreed in last meeting

Note: More specific, “If Rel-19 BS signaling IE is absent” intends for legacy NW or Rel-19 NW without Type-4 feature, “If Rel-19 BS signaling IE is present” intends for Rel-19 NW with Type-4 feature.

We understand there are several feasible solutions for BS signaling, and Type-4 is targeted for FWA. We proposed to discuss the “switching mode” in past meetings just for future proof, it is ok for us to drop it if there is no interest from companies.

Proposal 2: Invite companies to discuss whether to have/specify “switching mode” for Type-4.

- 3. Reference

[1] R4-2420399, WF on non-collocated intra-band EN-DC and NR-CA, KDDI, RAN4#113

[2] R4-2418303, Discussion on UE and NW behavior for non-collocated deployment, Samsung, RAN4#113