3GPP TSG-RAN WG4 Meeting #114	 R4-2500820

Athens, Greece, Feb. 17th – 21st, 2025

Source: 	Murata Manufacturing Co. Ltd

Title: 	MSD test point for DC_(n)3AA BCS1

Agenda item:	6.2.2	UE RF requirements for EN-DC and NE-DC of 2 DL with 2 UL (DC_R19_1BLTE_1BNR_2DL2UL)	[DC_R19_xBLTE_yBNR-Core]

Document for:	Discussion

- Introduction

MSD test points for DC_(n)3AA BCS1 are discussed in this contribution.

- Discussion

  - Background Information

In RAN#106, DC_(n)3AA BCS1 has been requested [1]. Table 1 shows bandwidth combinations for the DC combination.

Table 1: Bandwidth combinations for DC_(n)3AA BCS1

| DC configuration | Uplink DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)3AA | DC_(n)3AA | 5, 10, 15, 20 | 10, 20, 25, 30, 40 |  | 50 | 1 |
|  |  |  | 10, 20, 25, 30, 40 | 5, 10, 15, 20 |  |  |

Compared to DC_(n)3AA BCS0, DC_(n)3AA BCS1 allows operators to use wider channel bandwidths for the NR carrier, i.e., up to 40MHz. This contribution will examine candidate test points for DC_(n)3AA BCS1.

  - DC_(n)3AA BCS1 1UL

When only single switched UL is allowed for DC_(n)3AA BCS1, a 40MHz NR carrier will be available and new test points can be introduced for this new BCS1 configuration. When an NR 40MHz carrier is assigned, the possible LTE DL carriers are 5MHz and 10MHz. As discussed in [2], the LTE DL CC is configured closest to the NR UL CC. Regarding Lcrb allocation, two cases can be considered; Case 1 is that the UL Lcrb configuration is referred from TS 38.101-1 Table 7.3.2-3, which is the method used for DC_(n)5AA and DC_(n)12AA. Case 2 is using smaller aggressor Lcrb allocations to ensure a maximum impact of the Tx emission and highest MSD, as discussed for the DC_(n)3AA BCS0 test point [3]. For example, Figure 1 presents landscape for DC_(n)3AA BCS1 1UL for LTE DL carrier 5MHz.

Figure 1: Landscape of interferences by DC_(n)3AA BCS1 1UL.

Observation 1: For case 1, when the NR UL Lcrb configuration is referred from TS38.101-1 table 7.3.2-3, parts of IM products fall within the entire LTE DL.

Observation 2: For case 2, when adjusted Lcrb are allocated to ensure a maximum interference of the Tx emission, the LTE DL contains IM products, including their peak. In our consideration, when Lcrb configuration is adjusted to 10(RBstart=184) for the LTE DL 5MHz and 10(RBstart=181) for the LTE DL 10MHz case, the interferences will represent the worst-case scenarios.

Proposal 1: Consider the following test points for DC_(n)3AA BCS1 with 1UL configuration in Table 2.

Table 2: Candidate test points for DC_(n)3AA BCS1 with 1UL configuration

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) | (dB) |  |
| DC_(n)3AA | 3 | N/A | 5 | N/A | 1837.5 | [TBD] | FDD |
|  | n3 | 1765 | 40 | 50 (RBstart = 166) | 1860 | N/A |  |
| DC_(n)3AA | 3 | N/A | 10 | N/A | 1835 | [TBD] |  |
|  | n3 | 1765 | 40 | 50 (RBstart = 166) | 1860 | N/A |  |
| DC_(n)3AA | 3 | N/A | 5 | N/A | 1837.5 | [TBD] |  |
|  | n3 | 1765 | 40 | 10 (RBstart = 184) | 1860 | N/A |  |
| DC_(n)3AA | 3 | N/A | 10 | N/A | 1835 | [TBD] |  |
|  | n3 | 1765 | 40 | 10 (RBstart = 181) | 1860 | N/A |  |

  - DC_(n)3AA BCS1 2UL

For the 2UL configuration, RAN4 should confirm with the requester whether 2UL test points for DC_(n)3AA BCS1 should also be specified. The need for 2UL operation will depend on the actual demands of operators.

Proposal 2: RAN4 should confirm with the requester whether to specify 2UL test points for DC_(n)3AA BCS1.

- Conclusion

Observation 1: For case 1, when the NR UL Lcrb configuration is referred from TS38.101-1 table 7.3.2-3, parts of IM products fall within the entire LTE DL.

Observation 2: For case 2, when adjusted Lcrb are allocated to ensure a maximum interference of the Tx emission, the LTE DL contains IM products, including their peak. In our consideration, when Lcrb configuration is adjusted to 10(RBstart=184) for the LTE DL 5MHz and 10(RBstart=181) for the LTE DL 10MHz case, the interferences will represent the worst-case scenarios.

Proposal 1: Consider the following test points for DC_(n)3AA BCS1 with 1UL configuration in Table 2.

Table 2: Candidate test points for DC_(n)3AA BCS1 with 1UL configuration

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) | Channel bandwidth | UL | FC (DL) | MSD | Duplex mode |
|  |  | (MHz) | (MHz) | allocation (LCRB) | (MHz) | (dB) |  |
| DC_(n)3AA | 3 | N/A | 5 | N/A | 1837.5 | [TBD] | FDD |
|  | n3 | 1765 | 40 | 50 (RBstart = 166) | 1860 | N/A |  |
| DC_(n)3AA | 3 | N/A | 10 | N/A | 1835 | [TBD] |  |
|  | n3 | 1765 | 40 | 50 (RBstart = 166) | 1860 | N/A |  |
| DC_(n)3AA | 3 | N/A | 5 | N/A | 1837.5 | [TBD] |  |
|  | n3 | 1765 | 40 | 10 (RBstart = 184) | 1860 | N/A |  |
| DC_(n)3AA | 3 | N/A | 10 | N/A | 1835 | [TBD] |  |
|  | n3 | 1765 | 40 | 10 (RBstart = 181) | 1860 | N/A |  |

Proposal 2: RAN4 should confirm with the requester whether to specify 2UL test points for DC_(n)3AA BCS1.

# References:

[1] RP-243257, “Revised WID for DC_R19_xBLTE_yBNR”, Nokia, RP#106, Madrid, Spain

[2] R4-2417156, “WF on DC_(n)8AA test point and MSD test point clean-up“, Skyworks Solutions Inc., RAN4#112-bis, Hefei, China

[3] R4-2210233, “Single Uplink MSD for DC_(n)3AA”, Skyworks Solutions, Inc., RAN4#103-e, Electronic Meeting