3GPP TSG RAN WG4 Meeting#114                                    R4-2500822

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.19.2

Source:	vivo

Title:	Discussion on CSI reporting requirement framework for CSI prediction

Document for:	Discussion

- Introduction

In R18 SI, we have studied on general aspects, specific issues related to use cases, and interoperability and testability aspects. The agreements have been collected in TR [1]. In RAN#105 meeting, CSI prediction has been agreed for normative work and then the requirement of CSI prediction needs to be defined in this release [2].

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In previous meetings, we have the following agreements for CSI prediction.

Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition

In this contribution, we further provide our views on CSI prediction.

- Discussion on CSI prediction

In the previous meetings, RAN4 have agreed to use relative throughput as requirement metric for CSI prediction, and companies are invited to bring contribution on baseline throughput.

Before the discussion of AI CSI prediction, we would like to review the RAN4 agreement of non-AI CSI prediction. In RAN4#110, it has been agreed to use random precoding based on Type I Single Panel codebook as the baseline throughput.

Issue 1-1-4: clarify test metric for PMI reporting requirements with ‘typeII-Doppler-r18’ codebook Agreement: - Test metric defined as γ1=ttypeII-dopplertrnd, where ttypeII-doppler is X % of the maximum throughput obtained at SNRtypeII-doppler using the typeII-Doppler-r18 precoder configured according to the UE reports, and trnd is the throughput measured at SNRtypeII-doppler with random precoding based on Type I Single Panel codebook

In RAN4#110bis, the test setup and simulation assumptions of non-AI prediction have been agreed.

2.1 Sub-topic 1-1 Test setup and simulation assumptions for TypeII Doppler Issue 1-1-2: N4 and K configuration Agreement: - N4=1 and K=4 for both FR1 FDD and FR1 TDD. Issue 1-1-3: X% of the maximum throughput in Test metric Agreement: - 90% of the maximum throughput in Test metric Issue 1-1-4: Test metric of TypeII-Doppler-r18 codebook Decide the gamma value based on the updated simulation results on the May meeting Issue 1-1-5: Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook Agreement: - For FR1 FDD 15kHz SCS case, test setup is as below descriptions: - Non-CSI slots: scheduled in mod(i, 10) = {1,3,5,7,8,9}, i={0,1,2,…,19} - CSI-RS is scheduled in mod(i, 10) = {0,2,4,6}, i={0,1,2,…,19} - SSB slots: i=0 - TRS slots: i=10 and 11. - CSI report is transmitted in slot#10n+10 to estimate precoder in slot#10n+11 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 17, 18, 19, 21, 23} Issue 1-1-6: Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook Agreement: - For FR1 TDD 30kHz SCS case, test setup is as below descriptions for the simulation of following Rel18 Doppler codebook vs random Type I single panel codebook: - Scheduling pattern for TDD SCS=30kHz with the periodicity of 20ms (40 slots): - Non-CSI slots: scheduled in mod(i, 10) = {5,6}, i={0,1,2,…,39} - CSI slots: scheduled in mod(i, 10) = {0,1,2,3}, i={0,1,2,…,39} - SSB slots: i=0 - TRS slots: i=20 and 21. - CSI report is transmitted in slot#10n+9 to estimate precoder in slot#10n+10 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 16}

From the view of test design, AI prediction and non-AI prediction could follow the similar test procedure, to simply the RAN4 test. Then, the agreement of non-AI prediction could be used as reference and the baseline of AI prediction could be the throughput from random precoding.

In the last meeting, we have the following agreement that AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based). Since the requirement of non-AI prediction has been agreed, AI-prediction would have similar or higher gamma value compared to non-AI prediction.

Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Issue 2-2: Reference throughput Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Issue 2-3: Testing conditions Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition

In the study of R18 and R19, AI CSI prediction is supposed to achieve some performance gain compared to non-AI CSI prediction, and then it may be not suitable for AI CSI prediction to directly reuse the the test setup and simulation assumptions of non-AI CSI prediction. RAN4 could work on the test setup and simulation assumptions of AI CSI prediction, by the future evaluations. For this one-sided use case, reference model should be defined to align results from companies, and then the requirements could be derived based on the aligned results. Then in the next meeting, RAN4 could discuss the simulation assumption and reference model, where RAN1 agreements may be used as starting point.

Proposal 1: AI prediction and non-AI prediction could follow the similar test procedure. AI-prediction would have similar or higher gamma value compared to non-AI prediction.

Proposal 2: Similar test conditions of non-AI prediction would be considered for AI-prediction as starting point.

Following table is test conditions for multiple PMI with 16TX Enhanced Type II codebook for predicted PMI (non-AI prediction, Table 6.3.2.1.8-1 in 38.101). AI prediction can use this table as a starting point. Note that some parameters in this table would be not suitable or needed for AI prediction, for example, aperiodic CSI-RS configuration and codebook configuration. Current RAN1 discussions on AI prediction mainly use periodic CSI-RS, so aperiodic CSI-RS configuration in Table 2-1 would be not needed. Codebook configuration in Table 2-1 is used for TypeII-Doppler-r18 codebook, which may be not suitable for AI prediction. On the other hand, some other parameters would be needed for AI prediction, for example, hyper-parameters of reference model.

Proposal 3: Table 2-1 for non-AI prediction (Table 6.3.2.1.8-1 in 38.101) can be used for AI prediction as starting point. Some parameters in Table 2-1 would be not suitable or needed for AI prediction, such as:

- Aperiodic CSI-RS configuration;

- Codebook configuration for TypeII-Doppler-r18 codebook.

Table 2-1. Test conditions for multiple PMI with 16TX Enhanced Type II codebook for predicted PMI (non-AI prediction, Table 6.3.2.1.8-1 in 38.101)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-20 |
| Antenna configuration |  |  | XP Medium 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (9) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | The number of CSI-RS resources (K) |  | 4 |
|  | Number of CSI-RS ports (X) |  | 16 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  | CDM4 (FD2, TD2) for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  | 1 for CSI-RS resource 1,2,3,4 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) for CSI-RS resource 1,2,3,4 |
|  | CSI-RS interval and offset | slot | Not configured for CSI-RS resource 1,2,3,4 |
|  | aperiodicTriggeringOffset |  | 0 |
|  | Separation between two consecutive CSI-RS resources (m) | slot | 2 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 4 |
| csi-ReportingBand |  |  | 1111111111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 10 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-doppler-r18 |
|  | paramCombination-Doppler-r18 |  | 7 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMI-SubbandsPerCQI-Subband-r18) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r18) |  | 0010 |
|  | Doppler/time-domain basis vector length (N4) |  | 1 |
|  | Doppler-/time-domain unit duration (d) | slot | 1 |
|  | Prediction delay (delta) | slot | 1 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 15 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-24.1 FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

In 2024 Q1 to Q3, it is SI of AI prediction and RAN1 focused on the simulation of AI prediction, to obtain acceptable performance gain compared to non-AI prediction. AI prediction has been agreed for WI in 2024 Q3. In 2024 Q4, RAN1 has been limited to the study on consistency of training/inference. Then, in fact, RAN1 has not start the work of spec impact of AI prediction. LCM aspects, such as monitoring, have not been discussed yet. Then, RAN4 could not discuss LCM requirements (e.g., monitoring) and should wait for RAN1 progress. If there is any spec impact, RAN4 to discuss whether to capture it in the RRM spec.

Proposal 4: LCM requirements (e.g., monitoring) should wait for RAN1 progress. If there is any spec impact, RAN4 to discuss whether to capture it in the RRM spec.

- Summary

Based on above analysis, following proposals and observations are present.

Proposal 1: AI prediction and non-AI prediction could follow the similar test procedure. AI-prediction would have similar or higher gamma value compared to non-AI prediction.

Proposal 2: Similar test conditions of non-AI prediction would be considered for AI-prediction as starting point.

Proposal 3: Table 2-1 for non-AI prediction (Table 6.3.2.1.8-1 in 38.101) can be used for AI prediction as starting point. Some parameters in Table 2-1 would be not suitable or needed for AI prediction, such as:

- Aperiodic CSI-RS configuration;

- Codebook configuration for TypeII-Doppler-r18 codebook.

Table 2-1. Test conditions for multiple PMI with 16TX Enhanced Type II codebook for predicted PMI (non-AI prediction, Table 6.3.2.1.8-1 in 38.101)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-20 |
| Antenna configuration |  |  | XP Medium 16 x 2 (N1,N2) = (4,2) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (9) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | The number of CSI-RS resources (K) |  | 4 |
|  | Number of CSI-RS ports (X) |  | 16 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  | CDM4 (FD2, TD2) for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  | 1 for CSI-RS resource 1,2,3,4 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 12, (2, 4, 6, 8) for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | (5) for CSI-RS resource 1,2,3,4 |
|  | CSI-RS interval and offset | slot | Not configured for CSI-RS resource 1,2,3,4 |
|  | aperiodicTriggeringOffset |  | 0 |
|  | Separation between two consecutive CSI-RS resources (m) | slot | 2 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Not configured |
| Sub-band Size |  | RB | 4 |
| csi-ReportingBand |  |  | 1111111111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 10 |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-doppler-r18 |
|  | paramCombination-Doppler-r18 |  | 7 (L =4, pν =1/2, β=1/2 ) |
|  | R(numberOfPMI-SubbandsPerCQI-Subband-r18) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | CodebookSubsetRestriction |  | 0x 7FF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r18) |  | 0010 |
|  | Doppler/time-domain basis vector length (N4) |  | 1 |
|  | Doppler-/time-domain unit duration (d) | slot | 1 |
|  | Prediction delay (delta) | slot | 1 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 15 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | R.PDSCH.1-24.1 FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.1.3-1. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the dual-cluster beam directions shall be used as specified in AnnexB.2.3.2.3A. The value of relative power ratio (p) shall be fixed as 1 during the test. |  |  |  |

Proposal 4: LCM requirements (e.g., monitoring) should wait for RAN1 progress. If there is any spec impact, RAN4 to discuss whether to capture it in the RRM spec.

- References

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 2023-12.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep 9-12, 2024.