3GPP TSG-RAN WG4 Meeting #114	    	R4-2500846

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.18.3

Source: 	CMCC

Title: 		    Discussion on BS demodulation requirements for interference suppressing with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed test scenario and simulation assumptions for BS with MMSE-IRC receiver, the WF has been approved in [1]. In this contribution, we further discuss the remaining open test parameters.

- Discussion

  - Interference modelling

Number of interferers for 2Rx

  - Option 1: Define BS MMSE-IRC requirements for 2Rx

- Option 1A: 1 interferer

- Option 1B: 2 interferers

- Option 1C: Consider 2 interferers for HomNet and 1 interfere for HetNet

  - Option 2: Do not need to specify requirements for 2Rx

  - Companies are encouraged to provide simulation results for 1 or 2 interferers for 2Rx, to check the performance gain.

Based on our simulation results, for 2Rx, both 1 or 2 interferes scenarios have performance gain between IRC and MRC.

| 1T2R | Gain of HomNet (dB) | Gain of HetNet (dB) |
| --- | --- | --- |
| with 1 interfere (CP-OFDM) | 1.42 - 1.8 | 3.22 - 3.62 |
| with 2 interferes (CP-OFDM) | 1.68 - 2.23 | 3.38 - 4.41 |
| with 1 interfere (DFT-s-OFDM) | 0.93 - 1.24 | 1.77 - 3 |
| with 2 interferes (DFT-s-OFDM) | 0.83 - 1.16 | 1.26 - 3 |

From the point of decreasing the simulation and test effort and ensuring sufficient performance gain, for CP-OFDM, we can compromise to consider the cases of 2 interferes for HomNet and 1 interfere for HetNet. For DFT-s-OFDM, we can compromise to only consider HetNet with 1 interfere.

Proposal 1: For 2Rx and CP-OFDM waveform, consider 2 interferes for HomNet and 1 interfere for HetNet.

Proposal 2: For 2Rx and DFT-s-OFDM waveform, only consider HetNet with 1 interfere.

  - Test parameters and simulation assumptions

Channel bandwidth

- Agreement from RAN4#112bis:

  - For CP-OFDM:

- For performance evaluation purpose, Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz.

- Further discuss the CHBW for requirement definition

- Proposals on the CHBW for requirement definition for CP-OFDM:

  - Option 1:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

  - Option 2:

- For 15kHz SCS, cover 5MHz, 10MHz

- For 30kHz SCS, cover 10MHz, 40MHz

  - Option 3: minimum CBW

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

  - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS

- Proposals for DFT-s-OFDM:

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

Based on current applicability rule, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth which BS supported. Thereby, for CP-OFDM, we support to consider 5MHz, 10MHz, 20MHz for 15kHz, and 10MHz, 20MHz, 40MHz, 100MHz for 30kHz, in order to cover the worst case test for each BS. For DFT-s-OFDM, we are fine to keep the legacy test configuration, 5MHz for 15kHz SCS and 10MHz for 30kHz SCS.

Proposal 3:

- For CP-OFDM

  - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

  - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

- For DFT-s-OFDM

  - For 15kHz SCS, 5MHz

  - For 30kHz SCS, 10MHz

MCS

  - Consider the following options for performance evaluation and as candidates for further down-selection:

o    For 2Rx:

  Option 1: MCS 2

  Option 2: MCS 16

o    For 4Rx:

  Option 1: MCS 16

o    For 8Rx:

  Option 1: MCS 20

  Option 2: MCS 16

- Note: Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 for MCS 2, MCS 16 and MCS 20 respectively.

  - Check whether the existing test applicability rule can be reused if different MCS is configured for different Rx configuration.

Based on our simulation results, for 2Rx, MCS 16 has better performance under the test we proposed in Proposal 1 and proposal 2. For 8Rx, similar performance gain can be achieved between MCS 20 and MCS 16. Therefore, we are fine to use MCS 16 for 2Rx, 4Rx and 8Rx.

|  |  | Gain of MSC 2 (dB) | Gain of MSC 16 (dB) |
| --- | --- | --- | --- |
| 1T2R | HetNet with 1 interfere (CP-OFDM) | 3.22 - 3.41 | 3.45 - 3.62 |
|  | HomNet with 2 interferes (CP-OFDM) | 2.16 | 1.68 - 2.23 |
|  | HetNet with 1 interfere (DFT-s-OFDM) | 1.77 - 1.82 | 2.3 - 3 |
|  | HetNet with 2 interferes (DFT-s-OFDM) | 1.26 - 1.48 | 1.61 - 3 |

|  |  | Gain of MSC 20 (dB) | Gain of MSC 16 (dB) |
| --- | --- | --- | --- |
| 1T8R | HomNet (CP-OFDM) | 3.27 - 3.4 | 3.17 - 3.48 |
|  | HetNet (CP-OFDM) | 6.35 | 6.46 - 6.57 |
|  | HomNet (DFT-s-OFDM) | 3.67 | 3.6 - 3.63 |
|  | HetNet (DFT-s-OFDM) | 7.09 | 6.8 - 7 |

Proposal 4: Use MCS 16 for 2Rx, 4Rx and 8Rx.

Propagation condition

–    For serving cell, consider the following options for performance evaluation and as candidates for further down-selection:

o    Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet

o    Option 2: Consider different channel for different MCS

  Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10

–    For the interference cell, use TDLC300-100.

Based on our simulation results, for MCS 16, both Option 1 and Option 2 have enough performance gain between MMSE-IRC and MRC. The performance gain of Option 1 is little bit larger than Option 2. Therefore, we are both ok for Option 1 and Option 2.

Proposal 5: If MCS 16 is used, then for propagation condition setup, both Option 1 and Option 2 are ok for us.

Waveform – whether to cover DFT-s-OFDM

- Candidate options:

  - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform

  - Option 2: Cover CP-OFDM only

In ICI scenario which usually considering cell edge, it is more likely to use the DFT-s-OFDM waveform with lower PAPR. Therefore, we think the DFT-s-OFDM shall be also covered.

Proposal 6: Cover DFT-s-OFDM related requirements.

DMRS configuration

  - Cover DMRS configuration type 1. FFS whether to include DMRS configuration type 2.

  - Use DMRS port #0 for both serving and interfering PUSCH

  - On DMRS sequence generation, use different DMRS sequence for each gNB:

- For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively

- FFS DMRS sequence generation configuration for DFT-s-DFDM

Regarding DMRS type, we are fine to only cover DMRS type 1 to reduce the simulation and test effort.

Proposal 7: Only cover DMRS type 1.

Test requirement definition

- Candidate options:

  - Option 1: FFS whether SINR based definition is needed

  - Option 2: Use the SNR based requirement definition

In RAN4#112bis, RAN4 has agreed to use INR-based interference modelling, consider of using INR+SNR approach to define the requirement is more generic, we support to use SNR based requirement definition.

Proposal 8: Use SNR based requirement definition.

  - Manufacturer declarations and applicability rules

Applicability rule for different test scenarios

- Proposals on applicability for Homnet and Hetnet scenarios:

  - Proposal 1: HomNet requirements are not applicable for Local Area BS

  - Proposal 2: HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS

- Proposals on additional applicability rule:

  - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped

  - Proposal 2: Introduce the applicability rules that only one of HomNet and HetNet scenarios is tested

In LTE, the test requirements of Homogeneous network are not applicable for Local Area BS and Home BS. We think it is rational to follow similar approach in NR.

In 38.141-1 Clause 4.6, BS class manufactory declaration is as follows:

| D.2 | BS class | Declared as Wide Area BS, Medium Range BS, or Local Area BS. |
| --- | --- | --- |

For Wide Area BS, we think the test requirements of Homogeneous network shall be applicable, while for Medium Range BS and Local Area BS, the test of Homogeneous network can be skipped.

Proposal 9: Introduce the applicability rule that the test requirements of Homogeneous network are not applicable for Medium Range BS and Local Area BS.

- Conclusion

In this contribution, we provides some basic simulation assumptions for BS 8Rx interference suppressing requirement. The proposals are:

Proposal 1: For 2Rx and CP-OFDM waveform, consider 2 interferes for HomNet and 1 interfere for HetNet.

Proposal 2: For 2Rx and DFT-s-OFDM waveform, only consider HetNet with 1 interfere.

Proposal 3:

- For CP-OFDM

  - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

  - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

- For DFT-s-OFDM

  - For 15kHz SCS, 5MHz

  - For 30kHz SCS, 10MHz

Proposal 4: Use MCS 16 for 2Rx, 4Rx and 8Rx.

Proposal 5: If MCS 16 is used, then for propagation condition setup, both Option 1 and Option 2 are ok for us.

Proposal 6: Cover DFT-s-OFDM related requirements.

Proposal 7: Only cover DMRS type 1.

Proposal 8: Use SNR based requirement definition.

Proposal 9: Introduce the applicability rule that the test requirements of Homogeneous network are not applicable for Medium Range BS and Local Area BS.

- Reference

[1] R4-2419780, Way Forward for [113][321] NR_demod_Ph5_Part1_General_BS, China Telecom, MediaTek