3GPP TSG-RAN WG4 Meeting # 114		R4-2500848

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.25.2.1

Source: 	CMCC

Title: 			  Discussion on RRM requirement of OD-SSB for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed some measurement procedures and principles for requirement definition about OD-SSB in NES enhancement WI. The WF has been approved in [1], the details of the open issues are captured in the moderator summary [2]. In this paper, we further discuss OD-SSB related issues, and identify RAN4 impacts.

- General On-demand SSB(OD-SSB) requirements

Issue 1-1-1: RAN1 LS

Agreement:

- RAN4 to confirm that T_min equals to m+3Nslotsubframe,μ+1 from network transmission perspective.

  - Note 1: This does not imply UE shall be able to process/prepare the OD-SSB from time instance A from RAN4 requirement perspective.

- Meanwhile, RAN4 is discussing the additional UE processing/preparation time for OD-SSB reception for RAN4 requirement.

After T_min, if UE need to perform measurement on OD-SSB, as mentioned by companies in past meetings,  additional processing time for re-allocating the measurement searcher resource may be needed. In Case 1 that no AO-SSB, due to the number of frequency carriers to be measured may be changed, UE should re-allocate it’s measurement searcher resource. In Case 2 that AO-SSB is transmitted, if OD-SSB and AO-SSB are within different frequencies and UE is expected to perform measurement on both OD-SSB and AO-SSB, then additional processing time may needed. For other cases, we think the additional processing time is not necessary.

Proposal 1: Only for the following cases if valid, additional processing time (Tprocessing) may be needed for UE to measure the OD-SSB:

- No AO-SSB on the SCell (Case 1)

- AO-SSB is transmitted on the SCell (Case 2), while OD-SSB and AO-SSB are within different frequencies and UE is expected to perform measurement on both OD-SSB and AO-SSB

Regarding how long the additional processing time will take, considering the measurement resource re-allocation updating scheme is not complicated, we think 1 slot can be the starting point.

Proposal 2: For the value of Tprocessing, take 1 slot as the starting point.

Issue 1-1-2: L1 measurement before SCell activation

Proposals

- Option 1: L1 measurements are not supported for deactivated SCell.

- Option 2: Wait RAN1’s further clarification on the applicable scenario.

In legacy, there is no L1 measurement on deactivated SCell. In R19 NES WI, no agreement has been achieved about introducing L1 measurement for deactivated SCell in other working groups so far. Therefore, we prefer Option 1, and the issue can be further discussed if further related input from other working groups.

Proposal 3: L1 measurements are not supported for deactivated SCell if no further related input from other working groups.

Issue 1-1-3: L1 measurement after SCell activation

Proposals

- Option 1: CMCC

  - Unified design between L1 and L3 measurement is preferred, regarding whether OD-SSB or AO-SSB or both will be used for measurement, and when/how to perform the measurement.

- Option 2: CATT, Ericsson, Xiaomi

  - RAN4 to define OD-SSB based L1 measurement requirements based on RAN1 progress.

- Option 3: LG, Apple

  - RAN4 to update the L1 measurement requirement based on OD-SSB.

    - TSSB = the periodicity of the activated OD-SSB configured for L1-RSRP measurement.

Fist of all, we want to clarify the definition about time ‘duration of SCell activation’ and ‘after SCell activation’ in our paper.

- For time ‘during SCell activation’, it is the time duration of after SCell activation command and before valid CSI report

- For time ‘after SCell activation’, it is the time duration of after valid CSI report

So far, RAN1 achieved following agreements about L1 measurement, which confirm OD-SSB could be used for L1 measurement and discuss the CSI report configuration format:

Agreement from RAN1#117 - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB. Agreement from RAN1#118 - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement from RAN1#118bis - For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not

For L1 measurement during SCell activation, as RAN4 agreed in RAN4#112bis meeting, OD-SSB will be used as the baseline.

Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

During SCell activation, if UE needs to perform L1-RSRP measurement, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB. As for CSI reporting, the current definition of TCSI_reporting and TL1-RSRP, report can be reused.

Proposal 4: For L1 measurement during SCell activation, from RAN4 requirement point of view, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB.

Issue 1-1-4: OD-SSB accuracy

Proposals

- Option 1: CMCC, Ericsson, Qualcomm, China Telecom, Apple

  - The legacy measurement accuracy is applied for OD-SSB based measurement.

The legacy measurement accuracy should be applied for OD-SSB based measurement.

Proposal 5: Apply the legacy measurement accuracy for OD-SSB based measurement

Issue 1-1-5: Cell identification

- Proposals

  - Option 1: LG

    - RAN4 to specify cell identification requirements based on the time between multiple OD-SSB activation/deactivation

      - If the time is shorter than a threshold, the UE can skip PSS/SSS detection and/or SSB index acquisition

      - In Case 2, at least skip cell identification at least for Scenario 1. (Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.)

  - Option 2: Ericsson

    - The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

      - Case 1: No always-on SSB on the cell

        - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

        - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [Xs] and then the OD-SSB is transmitted again with the same spatial reception parameter

      - Case 2: Always-on SSB on the cell.

  - Option 3: vivo

    - UE may not need to perform OD-SSB based SCell identification if this SCell is known. RAN4 may further discuss whether the known condition for SCell activation can be re-used here for OD-SSB based cell identification.

- Recommended WF

  - Moderator suggests the group to discuss this issue based on the following sub-issues.

    - The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios:

      - Case 1: No always-on SSB on the cell

        - Option 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

        - Option 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [Xs] and then the OD-SSB is transmitted again with the same spatial reception parameter

(Note: this option has already covered in issue 1-2-3)

        - Option 3: follow SCell activation known condition

        - Other option is not precluded

      - Case 2: Always-on SSB on the cell

        - Option 1: Cell identification can be always skipped

          - Option 1a:  Cell identification can be always skipped in scenario 2

        - Option 2: follow SCell activation known condition

        - Other option is not precluded

In legacy, if the target cell is a new detectable intra-frequency cell, then UE shall be able to identify it within Tidentify_intra_without_index/_with_index.

While, in OD-SSB scenario, if the target cell i.e., deactivated SCell has already been a detected cell, then it is reasonable to skip the PSS/SSS detection and SSB index acquisition to fasten the UE measurement.

From our point of view, the deactivated SCell can be treated as a detected cell at least in following case:

- the UE has sent a valid measurement report based on either AO-SSB or OD-SSB for the SCell being activated [5]s before, the subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power, and on the same frequency carrier as the OD-SSB or AO-SSB used for the valid measurement report, provided the timing to this cell has not changed more than  3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected.

Proposal 6: Skip the PSS/SSS detection and SSB index acquisition at least in following case:

- the UE has sent a valid measurement report based on either AO-SSB or OD-SSB for the SCell being activated [5]s before, the subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power, and on the same frequency carrier as the OD-SSB or AO-SSB used for the valid measurement report, provided the timing to this cell has not changed more than  3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected.

Issue 1-1-7: OD-SSB impact on neighbor cell measurement clarification

- Proposals

  - Option 1: Nokia

    - RAN4 to further clarify the OD-SSB based neighbour cell measurement as follow.

      - If there is an OD-SSB neighbor cell on a serving CC with legacy SCell, the existing measurement requirement based on SMTC shall apply to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

      - If there is OD-SSB SCell on the serving CC, the existing measurement requirement for neighbor cells still apply to the legacy neighbor cells on the CC.

  - Option 2: Huawei

    - For neighbor cell measurement when UE performs deactivated SCell L3 measurement based on OD-SSB, RAN4 to considering following two options:

      - UE is not required to perform neighbor cell measurement

      - UE performs neighbor cell measurement based MeasCycle.

- Recommended WF

  - Moderator suggests the group to discuss the issue with the following options.

    - Option 1: The existing measurement requirement based on SMTC shall apply to

      - the OD-SSB neighbor cell only if OD-SSB is activated on the cell

      - other neighbour cell

    - Option 2: UE is not required to perform neighbor cell measurement

    - Option 3: The existing measurement requirement based on MeasCycleSCell shall apply.

    - Other option is not precluded

From our perspective, the motivation of sending OD-SSB is to accelerate SCell activation procedure while achieve the network power saving gain, which results in discontinuous OD-SSB transmission. Such kind of OD-SSB is not suitable for performing L3 neighbour cell measurement.

However, if anyway UE need to perform serving and neighbour cell measurement on the OD-SSB carrier frequency, then the existing measurement requirement can be applied if OD-SSB is activated on the neighbour cell, which includes following cases:

- Case 1: On the carrier frequency of the MO configured by an activated serving cell,  if there is an OD-SSB neighbor cell, then the existing measurement requirement based on SMTC/MG shall be applied to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

- Case 2: On the carrier frequency of the serving cell MO configured by an deactivated SCell, or on the carrier frequency of the AO-SSB transmitted by an deactivated SCell, if there is an OD-SSB neighbor cell, then the existing measurement requirement based on MeasCycleSCell shall be applied to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

While if UE don’t need to perform continuous serving cell measurement on the OD-SSB carrier frequency, then the requirement will be applied if OD-SSB is activated on both serving cell and neighbour cell, which includes following case:

- Case 3: The deactivated SCell only transmit the OD-SSB, then the existing measurement requirement based on MeasCycleSCell shall be applied to the OD-SSB neighbor cell only if OD-SSB is activated on both serving cell and neighbour cell.

Proposal 7: For OD-SSB based neighbour cell measurement:

- On the carrier frequency of the MO configured by an activated serving cell,  if there is an OD-SSB neighbor cell, then the existing measurement requirement based on SMTC/MG shall be applied to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

- On the carrier frequency of the serving cell MO configured by an deactivated SCell, or on the carrier frequency of the AO-SSB transmitted by an deactivated SCell, if there is an OD-SSB neighbor cell, then the existing measurement requirement based on MeasCycleSCell shall be applied to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

- If the deactivated SCell only transmit the OD-SSB, the existing measurement requirement based on MeasCycleSCell shall be applied to the OD-SSB neighbor cell only if OD-SSB is activated on both serving cell and neighbour cell.

- OD-SSB based deactivated SCell L3 measurement (Case 1)

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement

Agreement:

For the case of CSSF = 1:

- OD-SSB periodicity regardless of measCycleSCell and DRX cycle.

  - FFS whether a lower bound will be specified.

FFS for the other cases of CSSF.

For the fast L3 measurement requirement, in order to achieve the maximum network power saving gain, UE perform the measurement following the OD-SSB periodicity regardless of measCycleSCell and DRX cycle can be the baseline, that UE could finish the measurement with the shortest time duration. This measurement behavior has no related with CSSF value.

Proposal 8: Regardless of CSSF value,  measCycleSCell and DRX cycle, UE should perform OD-SSB based deactivated SCell measurement based on OD-SSB periodicity.

To guarantee the L3 filter gain, we are fine to introduce the lower bound of OD-SSB based deactivated SCell measurement requirement, the legacy lower bound can be the baseline, we would like to further investigate whether a smaller value can be used.

Proposal 9: Introduce the lower bound of OD-SSB based deactivated SCell measurement requirement, which should be equal to or smaller than the legacy lower bound of intra/inter-frequency measurement.

Issue 1-2-2: OD-SSB based deactivated SCell measurement – CSSF in fast mode measurement

- Option 1: The same CSSF as legacy

- Option 2:

  - OD-SSB based L3 measurement is prioritized over legacy L3 measurement, if cell identification is needed for OD-SSB-based L3 measurements.

    - RRM requirements on PSCC and other SCCs are not applicable during the time-window.

  - Legacy L3 measurement on other SCC are not impacted, if OD-SSB based cell identification is not needed.

Other option is not precluded.

In FR1, the searcher assumption is that PCC will occupy one searcher, SCC and inter-frequency MO occupy the other searcher with same priority.

In FR2, the searcher assumption is that PCC will occupy one searcher, SCC where neighbour cell measurement is required occupy half of the other one searcher, SCC where neighbour cell measurement is not required and inter-frequency MO occupy half of the other one searcher with same priority.

One issue is whether the priority will be increased for SCC with OD-SSB. Based on our understanding, OD-SSB has higher priority than AO-SSB in one SCell, but the priority among SCells should not be changed by OD-SSB triggering.

The second issue is whether both OD-SSB and AO-SSB are expected to be measured when they are not on the same frequency layer. Considering the measurement results combination between OD-SSB and AO-SSB on different frequency layer is not straight-forward, we think UE will not perform L3 measurement on both AO-SSB and OD-SSB which on different frequency layers.

With our consideration on these two issues, we think the CSSF assumption and algorithm can be kept as existing way.

Proposal 10: The legacy CSSF can be reused.

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure

Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation?

Agreement:

- After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective.

- The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)?

Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined?

Agreement:

The time window is based on the number of OD-SSB samples and UE provided one valid report.

- The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy

- UE should have enough number of OD-SSB samples to achieve the measurement requirement.

- The number of OD-SSB is FFS.

- FFS whether UE measurement reporting is needed

Sub-issue 5: What’s the UE behaviour when OD-SSB further activation/reconfiguration after T2?

FFS.

For Sub-issue 2&5 of Issue 1-2-3, the interval (T2, stage 4) between multiple times OD-SSB activation/deactivation is up to network configuration, but the UE measurement behavior after T2 can be further defined in RAN4.

- If T2 is less than the the maximum interval [5]s to avoid cell identification procedure, the UE is assumed to skip the PSS/SSS detection and SSB index acquisition as we analyzed in Issue 1-1-5, and perform deactivated SCell measurement with the fast mode, provided that the subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power and on the same frequency carrier as the OD-SSB used for the last valid measurement report, and the timing to this cell has not changed more than  3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected.

- If T2 is larger than the the maximum interval [5]s to avoid cell identification procedure, the UE is assumed to perform both cell identification and measurement with the fast mode.

Proposal 11: Interval (T2, stage 4) between multiple times OD-SSB activation/deactivation is up to network configuration, the UE measurement behavior after T2 should be:

- If T2 is less than the the maximum interval s to avoid cell identification procedure, the UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition as we analyzed in Issue 1-1-5, and perform deactivated SCell measurement with the fast mode, provided that the subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power and on the same frequency carrier as the OD-SSB used for the last valid measurement report, and the timing to this cell has not changed more than   3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected.

- If T2 is larger than the the maximum interval to avoid cell identification procedure, the UE is assumed to performs both cell identification and measurement with the fast mode.

For Sub-issue 3&4: Based on our analysis about T2, if UE need to perform both cell identification and measurement with the fast mode measurement, the time window length should be min (Tidentify_intra_with/without_index + T_min + Tprocessing, TOD-SSB_activation, Treport),  if UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition in the fast mode measurement, the time window length should be min (TSSB_measurement_period_intra + T_min + Tprocessing, TOD-SSB_activation, Treport), of which:

- For the requirements of Tidentify_intra_with/without_index and TSSB_measurement_period_intra, measCycleSCell is replaced by OD-SSB periodicity,

- TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication,

- Treport is the time duration between OD-SSB activation indication and valid measurement report.

- Proposal 12: The time window is pre-defined, which length should be:

- If UE need to perform both cell identification and measurement with the fast mode measurement, the time window length should be min (Tidentify_intra_with/without_index + T_min + Tprocessing, TOD-SSB_activation, Treport),

- If UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition in the fast mode measurement, the time window length should be min (TSSB_measurement_period_intra + T_min + Tprocessing, TOD-SSB_activation, Treport),

  - For the requirements of Tidentify_intra_with/without_index and TSSB_measurement_period_intra, measCycleSCell is replaced by OD-SSB periodicity,

  - TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication,

  - Treport is the time duration between OD-SSB activation indication and valid measurement report.

A rough illustration of OD-SSB based deactivated SCell measurement procedure proposed by us is as follows:

- OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

- Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 1-1: The measurement periodicity is OD-SSB periodicity

  - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB

- Option 2: UE always follow legacy requirements based on measCycleSCell

- Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

For Case 2, we prefer to use the same design as Case 1, that introducing a time window for UE. In the time window, UE performs fast mode measurement, after the time window, UE performs slow mode measurement.

For a new detectable cell, the window length is pre-defined as min (Tidentify_intra_with/without_index + T_min + Tprocessing, TOD-SSB_activation, Treport), for a detected known cell, the window length is pre-defined as min (TSSB_measurement_period_intra + T_min + Tprocessing, TOD-SSB_activation, Treport), of which:

- For the requirements of Tidentify_intra_with/without_index and TSSB_measurement_period_intra, measCycleSCell is replaced by OD-SSB periodicity,

- TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication,

- Treport is the time duration between OD-SSB activation indication and valid measurement report.

Proposal 13: After OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE performs slow mode measurement.

Proposal 14: For a new detectable cell, the window length is pre-defined as min (Tidentify_intra_with/without_index + T_min + Tprocessing, TOD-SSB_activation, Treport), for a detected known cell, the window length is pre-defined as min (TSSB_measurement_period_intra + T_min + Tprocessing, TOD-SSB_activation, Treport), of which:

- For the requirements of Tidentify_intra_with/without_index and TSSB_measurement_period_intra, measCycleSCell is replaced by OD-SSB periodicity,

- TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication,

- Treport is the time duration between OD-SSB activation indication and valid measurement report.

Issue 1-3-3: Scenario 1 - Transition period between measurement

- Proposals

During the transition period,

  - Option 1: CMCC, ZTE

    - UE shall be allowed to restart the ongoing evaluation using the second SSB type when the measured SSB type changes.

Subsequent to the transition period, the UE shall use an evaluation period corresponding to the second SSB type.

- Recommended WF

  - Further discussion

If UE perform the legacy L3 measurement follows ‘measCycleSCell’ before on-demand SSB triggering, and perform OD-SSB based fast mode L3 measurement after OD-SSB triggering, there will be a transition period between measurement.

With the time-window design, we think UE shall be allowed to restart the ongoing evaluation using the second SSB type when the measured SSB type changes.

Proposal 15: With the design of time window, UE shall be allowed to restart the ongoing evaluation using the second SSB type when the measured SSB type changes.

Issue 1-3-4: Scenario 3 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered

- Proposals

  - Option 1: Nokia

    - UE follows legacy measurement behavior assuming they are configured as separate measurement objects.

    - RAN4 can discuss how to define the measurement requirement on the MO with OD-SSB when it is activated.

- Recommended WF

  - Postpone the discussion until RAN1 LS response.

About the Q3 from RAN4 LS: What is the relation in terms of frequency location between the always-on SSB and OD-SSB? RAN1 achieved following agreement:

Agreement Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

Based on the current progress, it is not clear whether the scenario 3, i.e., OD-SSB and always-on SSB are within different SSB carrier frequencies is valid or not, the discussion can be postponed until further RAN1’s input.

Issue 1-3-5: Always-on SSB and OD-SSB time/frequency domain relation

- Background

From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible based on RAN1’s current agreements.

  - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different frequencies.

- Proposals

  - Option 1: CMCC

    - RAN4 to define the unified design for Scenario 1, 2 and 3.

  - Option 2: Qualcomm, CATT, Samsung, Ericsson

    - RAN4 to deprioritize the AO-SSB and OD-SSB scenarios discussion until RAN1 further response.

    - RAN4 to first focus on the scenario 1.

  - Option 3: Nokia

    - RAN4 to start the discussion for scenario 1 and scenario 3.

  - Option 4: Qualcomm

    - Requirements should only be defined for the case when both types of SSB fit into the same periodic structure.

  - Option 5: Intel

    - No limitation is applied to the time and frequency allocation between OD-SSB and always-on SSB in RAN4 spec.

  - Option 6: Apple

    - Do not define requirement for scenario 3 in case 2.

- Recommended WF

  - RAN4 to confirm the pervious agreements and wait RAN1’s LS response.

About the questions from RAN4 LS, RAN1 achieved following agreement:

Agreement Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1: - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Further update to be made based on RAN1#119 progress. Agreement Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1: - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB Agreement Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Agreement Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1: - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

According to the current progress from RAN1, since the time/frequency location between always-on SSB and OD-SSB are still under discussion, RAN4 can still first focus on the scenario 1, and wait RAN1’s LS response.

But anyway, if scenario 2 and 3 are valid scenarios, the unified design among three scenarios is preferred by us, let UE have common behavior.

Proposal 16: RAN4 still first focus on the scenario 1, and wait RAN1’s LS response of scenario 2 and 3.

Proposal 17: If scenario 2 and 3 are valid scenarios, the unified design of UE behavior among three scenarios is preferred.

- OD-SSB based SCell activation

Issue 1-4-1: The known cell condition – measurement reporting

Agreement:

- In case 1, known/unknown condition should be defined based on only OD-SSB.

- In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB.

  - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue.

  - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB.

  - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

The SSB measured for known cell condition and the SSB used in SCell activation procedure can be same or different, but the SCell activation delay requirements shall be defined case by case.

If the SSB measured for known cell condition and the SSB used in SCell activation procedure are same, then the current known SCell activation delay methodology can be reused.

If the SSB measured for known cell condition and the SSB used in SCell activation procedure are different, when OD-SSB and AO-SSB are transmitted on the same carrier frequency and with same transmission power, the current known SCell activation delay methodology can be reused. Otherwise, the AGC adjustment time should be included in the known SCell activation delay requirement.

Proposal 18: The SSB measured for known cell condition and the SSB used in SCell activation procedure can be same or different:

- If the SSB measured for known cell condition and the SSB used in SCell activation procedure are same, the current known SCell activation delay methodology can be reused.

- If the SSB measured for known cell condition and the SSB used in SCell activation procedure are different, the current known SCell activation delay methodology can be reused for the case of OD-SSB and AO-SSB are transmitted on the same carrier frequency and with same transmission power. Otherwise, the AGC adjustment time should be included in the known SCell activation delay requirement.

Issue 1-4-2: The known cell condition – period of known condition

- Proposals

  - Option 1: CMCC, Nokia, LG

    - Reuse the legacy known/unknown cell condition, max(5*measCycleSCell,  5*DRX cycles) in FR1.

  - Option 2: Intel

    - In Case 1, update the known condition period with the max(5 * [measCycleODSSB], 5*DRX cycle).

    - In Case 2, update the known condition period with the max(5*measCycleSCell, 5*[measCycleODSSB], 5 * DRX cycle).

- Recommended WF

  - Further discussion

About how known/unknown condition is defined, in legacy FR1, the known case requires UE to sent a valid measurement report during a period before the reception of the SCell activation command, the period equal to max(5*measCycleSCell,  5*DRX cycles), which related to the legacy L3 measurement requirement.

In other words, as long as UE reported the measurement report during max(5*measCycleSCell,  5*DRX cycles) before the SCell activation command, UE could maintain the known states of this deactivated SCell. From this point of view, we think this known/unknown condition can be reused even for OD-SSB case. We illustrate the cases in the figure below.

Proposal 19: Reuse the legacy known/unknown cell condition.

Issue 1-4-6: Separate MAC CE processing time

- Background

In RAN2 #127bis meeting, the following agreement was achieved.

RAN2#127bis Agreements on OD-SSB SCell

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- Proposals

RAN4 to discuss how to handle the separate OD-SSB MAC CE processing time

  - Option 1: CATT

    - The separate MAC CE processing time is needed in the activation delay requirement.

  - Option 2: CATT

    - The separate OD-SSB MAC CE processing time can be absorbed in the waiting time for the first available SSB.

RAN4 to discuss whether any restriction or requirements applicability are needed for scenario #2A regarding the time gap between the MAC CE for OD-SSB indication and SCell activation.

- Recommended WF

  - Further discussion

Due to network may transmit separate MAC CEs for SCell activation and OD-SSB indication, UE may need separate MAC CE processing for these two MAC CEs. However, from requirement definition point of view, we think the OD-SSB indication MAC CE processing time can be absorbed in the waiting time for the first available SSB.

Proposal 20: The OD-SSB indication MAC CE processing time can be absorbed in the waiting time for the first available SSB.

- Others

Issue 1-5-1: The type of OD-SSB based SCell activation

FFS: RAN4 to define OD-SSB based SCell activation delay requirements for the following one or more scenarios.

- Option 1: Direct SCell activation at SCell addition

- Option 2: Multiple SCells activation

- Option 3: Deactivated PUCCH SCell activation

- Option 4: Rel-17 Fast SCell Activation

- Option 5: Rel-18 SCell Activation with the L3 reporting during activation

In RAN1#117 and RAN2#127 meeting, following agreements has been achieved:

Agreement from RAN1: For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement from RAN2: RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

Since RAN1 has agreed to support RRC based OD-SSB transmission indication in the case of direct SCell activation, after RAN4 concludes on the MAC CE based single SCell activation case, the OD-SSB based direct SCell activation delay at SCell addition requirement should be specified at least.

Proposal 21:  Specify the OD-SSB based direct SCell activation delay at SCell addition requirement.

Similar as MAC CE triggered SCell activation, the SSB measured for known cell condition (AO-SSB) and the SSB used in SCell activation procedure (OD-SSB) can be different, then if OD-SSB and AO-SSB are transmitted on the same carrier frequency and with same transmission power, the current known SCell activation delay methodology can be reused. Otherwise, the AGC adjustment time should be included in the known SCell activation delay requirement.

Proposal 22: For OD-SSB based direct SCell activation at SCell addition, the SSB measured for known cell condition (AO-SSB) and the SSB used in SCell activation procedure (OD-SSB) can be different:

- If OD-SSB and AO-SSB are transmitted on the same carrier frequency and with same transmission power, the current known SCell activation delay methodology can be reused. Otherwise, the AGC adjustment time should be included in the known SCell activation delay requirement.

Since OD-SSB is activated during SCell activation, the SCell activation delay requirement should be defined based on OD-SSB periodicity.

Proposal 23: For OD-SSB based direct SCell activation at SCell addition, the requirement should be defined based on OD-SSB periodicity.

- Conclusion

Based on the discussion above, the following proposals and observations are concluded.

Proposal 1: Only for the following cases if valid, additional processing time (Tprocessing) may be needed for UE to measure the OD-SSB:

- No AO-SSB on the SCell (Case 1)

- AO-SSB is transmitted on the SCell (Case 2), while OD-SSB and AO-SSB are within different frequencies and UE is expected to perform measurement on both OD-SSB and AO-SSB

Proposal 2: For the value of Tprocessing, take 1 slot as the starting point.

Proposal 3: L1 measurements are not supported for deactivated SCell if no further related input from other working groups.

Proposal 4: For L1 measurement during SCell activation, from RAN4 requirement point of view, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB.

Proposal 5: Apply the legacy measurement accuracy for OD-SSB based measurement

Proposal 6: Skip the PSS/SSS detection and SSB index acquisition at least in following case:

- the UE has sent a valid measurement report based on either AO-SSB or OD-SSB for the SCell being activated [5]s before, the subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power, and on the same frequency carrier as the OD-SSB or AO-SSB used for the valid measurement report, provided the timing to this cell has not changed more than  3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected.

Proposal 7: For OD-SSB based neighbour cell measurement:

- On the carrier frequency of the MO configured by an activated serving cell,  if there is an OD-SSB neighbor cell, then the existing measurement requirement based on SMTC/MG shall be applied to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

- On the carrier frequency of the serving cell MO configured by an deactivated SCell, or on the carrier frequency of the AO-SSB transmitted by an deactivated SCell, if there is an OD-SSB neighbor cell, then the existing measurement requirement based on MeasCycleSCell shall be applied to the OD-SSB neighbor cell only if OD-SSB is activated on the cell.

- If the deactivated SCell only transmit the OD-SSB, the existing measurement requirement based on MeasCycleSCell shall be applied to the OD-SSB neighbor cell only if OD-SSB is activated on both serving cell and neighbour cell.

Proposal 8: Regardless of CSSF value,  measCycleSCell and DRX cycle, UE should perform OD-SSB based deactivated SCell measurement based on OD-SSB periodicity.

Proposal 9: Introduce the lower bound of OD-SSB based deactivated SCell measurement requirement, which should be equal to or smaller than the legacy lower bound of intra/inter-frequency measurement.

Proposal 10: The legacy CSSF can be reused.

Proposal 11: Interval (T2, stage 4) between multiple times OD-SSB activation/deactivation is up to network configuration, the UE measurement behavior after T2 should be:

- If T2 is less than the the maximum interval s to avoid cell identification procedure, the UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition as we analyzed in Issue 1-1-5, and perform deactivated SCell measurement with the fast mode, provided that the subsequent OD-SSB is transmitted with the same spatial reception parameter, same transmission power and on the same frequency carrier as the OD-SSB used for the last valid measurement report, and the timing to this cell has not changed more than   3200/2µ Tc while L3 filtering has not been used, when L3 filtering is used, an additional delay can be expected.

- If T2 is larger than the the maximum interval to avoid cell identification procedure, the UE is assumed to performs both cell identification and measurement with the fast mode.

- Proposal 12: The time window is pre-defined, which length should be:

- If UE need to perform both cell identification and measurement with the fast mode measurement, the time window length should be min (Tidentify_intra_with/without_index + T_min + Tprocessing, TOD-SSB_activation, Treport),

- If UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition in the fast mode measurement, the time window length should be min (TSSB_measurement_period_intra + T_min + Tprocessing, TOD-SSB_activation, Treport),

  - For the requirements of Tidentify_intra_with/without_index and TSSB_measurement_period_intra, measCycleSCell is replaced by OD-SSB periodicity,

  - TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication,

  - Treport is the time duration between OD-SSB activation indication and valid measurement report.

Proposal 13: After OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE performs slow mode measurement.

Proposal 14: For a new detectable cell, the window length is pre-defined as min (Tidentify_intra_with/without_index + T_min + Tprocessing, TOD-SSB_activation, Treport), for a detected known cell, the window length is pre-defined as min (TSSB_measurement_period_intra + T_min + Tprocessing, TOD-SSB_activation, Treport), of which:

- For the requirements of Tidentify_intra_with/without_index and TSSB_measurement_period_intra, measCycleSCell is replaced by OD-SSB periodicity,

- TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication,

- Treport is the time duration between OD-SSB activation indication and valid measurement report.

Proposal 15: With the design of time window, UE shall be allowed to restart the ongoing evaluation using the second SSB type when the measured SSB type changes.

Proposal 16: RAN4 still first focus on the scenario 1, and wait RAN1’s LS response of scenario 2 and 3.

Proposal 17: If scenario 2 and 3 are valid scenarios, the unified design of UE behavior among three scenarios is preferred.

Proposal 18: The SSB measured for known cell condition and the SSB used in SCell activation procedure can be same or different:

- If the SSB measured for known cell condition and the SSB used in SCell activation procedure are same, the current known SCell activation delay methodology can be reused.

- If the SSB measured for known cell condition and the SSB used in SCell activation procedure are different, the current known SCell activation delay methodology can be reused for the case of OD-SSB and AO-SSB are transmitted on the same carrier frequency and with same transmission power. Otherwise, the AGC adjustment time should be included in the known SCell activation delay requirement.

Proposal 19: Reuse the legacy known/unknown cell condition.

Proposal 20: The OD-SSB indication MAC CE processing time can be absorbed in the waiting time for the first available SSB.

Proposal 21:  Specify the OD-SSB based direct SCell activation delay at SCell addition requirement.

Proposal 22: For OD-SSB based direct SCell activation at SCell addition, the SSB measured for known cell condition (AO-SSB) and the SSB used in SCell activation procedure (OD-SSB) can be different:

- If OD-SSB and AO-SSB are transmitted on the same carrier frequency and with same transmission power, the current known SCell activation delay methodology can be reused. Otherwise, the AGC adjustment time should be included in the known SCell activation delay requirement.

Proposal 23: For OD-SSB based direct SCell activation at SCell addition, the requirement should be defined based on OD-SSB periodicity.

- Reference

- R4-2420098, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

- R4-2418282, Topic summary for [113][224] Netw_Energy_NR_enh_Part1, Moderator (Ericsson)