3GPP TSG-RAN WG4 Meeting # 114                                    R4-2500866

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.2.1

Source: 	CMCC

Title: 	Discussion on new RF requirement for SBFD

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [2]

In this contribution, we continue the RF requirements discussion based on the output of study item phase.

- Discussion

2.1 Location of transient period between SBFD and non-SBFD

Although there will be a transition phase between slots designated for SBFD operations and those not designated for SBFD, it was agreed that there is no necessity to define a transmitter transition period requirement for different configurations of SBFD. The unresolved issue now is to ascertain the placement of the transition period. Our opinion is to designate the SBFD slot to handle the transition period, allowing for the necessary rearrangement of RF resources, thereby preserving the integrity of the TDD slot.

Proposal 1: Transmitter transient period shall be located within the SBFD symbol.

2.2 In-channel adjacent subband leakage ratio requirements

In last meeting, agreements are listed as below:

Agreement: - New requirement for in-channel adjacent subband leakage ratio should be defined - FFS whether the requirement can be defined similar to the ACLR requirement and use existing ACLR requirement as baseline.

When co-exist with legacy NR network, interference may still occur even with infinitely increased SBFD performance. But when co-exist with SBFD of different configuration, interference could be avoided by increased SBFD performance. So before defining sub-band Tx leakage when falling into adjacent carrier, we should identify the assumption of adjacent network performance, i.e. with legacy 3GPP requirement or with enhanced requirements.

Proposal 2: Identify the assumption of adjacent network performance before defining sub-band Tx leakage.

Proposal 3: Legacy ACLR requirements could be used as starting point.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: Transmitter transient period shall be located within the SBFD symbol.

Proposal 2: Identify the assumption of adjacent network performance before defining sub-band Tx leakage.

Proposal 3: Legacy ACLR requirements could be used as starting point.

# 4. Reference

[1] TR 38.858

[1] R4-2419907, Way Forward for [113][306] NR_duplex_evo_General