3GPP TSG-RAN WG4 Meeting #114				                      R4-2500879

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.24.2.3

Source: 	CMCC

Title: 	Discussion on CW requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI[1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the requirements for CW node.

- Discussion

Agreement in SI phase:

- Whether requirements potentially needed or not is summarized in the following table:

| RF Requirement for CW for D1T1 | Requirements potentially needed or not |
| --- | --- |
| Output power | YES |
| ON/OFF power | TBD in WI phase |
| Frequency error | YES |
| Phase noise | No for inside topology TBD in WI phase for outside topology |
| Unwanted emission | YES |
| Spurious emissions | YES |
| Intermodulation | TBD in WI phase |

Since only CW outside topology is within Rel-19 WI scope, according to SID conclusion, only output power, unwanted emission and spurious emission are needed. According to the WID [1], only single tone CW is supported in Rel-19.

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769:

  - Case 1-4 for D1T1-B

For output power, 23dBm is used for co-existence evaluation for the case that CW is transmitted on FDD UL. However, from co-existence evaluation, higher output power is also possible due to better emission requirements compared to UE IBE. Hence, we propose to also consider output power higher than 23dBm, e.g. 33dBm. And there is no need to define power class for CW, only define maximum output power is enough.

The unwanted emission and spurious emission of NB-IOT can be considered as starting point, some adjustment on the offset need to be considered.

Proposal 1: For output power, define maximum output power (e.g. 33dBm) for CW node.

Proposal 2: For emission requirements, use NB-IOT requirements as baseline and adjust the offsets.

# 3. Conclusions

In this contribution, we discuss the RF requirements for CW node. The proposals are:

Proposal 1: For output power, define maximum output power (e.g. 33dBm) for CW node.

Proposal 2: For emission requirements, use NB-IOT requirements as baseline and adjust the offsets.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.

- R4-2420470, WF on RF requirements for A-IOT, Moderator (Huawei).