3GPP TSG-RAN WG4 Meeting #114	R4-2500882

# Athens, Greece, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for FR2-1 L3 measurement delay reduction

Agenda item:	7.17.2.1

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on some open issues on L3 measurement delay reduction with multi-Rx simultaneous reception based on [1].

Discussion

Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

In the last RAN4 meeting, the triggering condition to activate/deactivate multi-Rx for L3 measurement based on serving cell’s signal quality was agreed for scenario 1/2/3/4. In general, Rel-18 UAI ‘multiRx-PreferenceFR2’ can be reused.

- Proposal 1: Rel-18 UAI ‘multiRx-PreferenceFR2’ can be reused to quit multi-Rx operation for L3 measurement.

However, for futher UE power saving, additional conditions to quit multi-Rx operation for L3 measurement (scenario 1/2/3/4) can be considered. Aftere activating multi-Rx based on serving cell’s signal quality, UE might not be maintain multi-Rx operation if UE reports valid measurement result. Therefore, additional condtion for deactivating multi-Rx operation can be considered based on whether to report valid measurement result.

- Proposal 2: Additional condition for deactivating multi-Rx for scenario 1/2/3/4 could be considered as below:

  - After activating multi-Rx for L3 measurement based on serving cell’s signal quality, UE can deactivate multi-Rx operation when UE reports valid measurement result.

Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

The SSB based intra-frequency measurement requirements without gaps for FR2-1 in Table 1 has been defined considering UE Rx beam sweeping.

Table 1 Measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

M values depending on power class are below.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

For UE supporting FR2-1 power class 3, M=24, and it means that 4 measurement samples with 6 Rx beam sweeping or 3 measurement samples with 8 Rx beam sweeping. The reason for using [measurement sampels x Rx beam sweeping] for L3 measurement is to allow different Rx beam sweeping pattern or beam width for the measurement by UE implementation. That is, it is already different Rx beam sweeping factor were considered for the measurement. So, when UE activates multi-Rx operation for L3 measurement, it is important to reduce the total measurement delay that UE can measure with multi-Rx. Therefore, it is better to introduce scaling value < 1 than to introduce Rx BSF value.

- Proposal 3: For L3 measurement, introduce scaling value (Pmulti-Rx) < 1 to reduce measurement delay

Based on the Proposal 3, L3 measurement delay requirement using multi-Rx reception can be below table. Pmulti-Rx is less than 1 and UE capability value.

Table 2 Example of proposed measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Pmulti-Rx x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Pmulti-Rx x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Pmulti-Rx x Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Conclusion

In this contribution, we provide views on activating condition for multi-Rx simultaneous reception and how to define core requirements for L3 measurement delay reduction, and we propose

- Proposal 1: Rel-18 UAI ‘multiRx-PreferenceFR2’ can be reused to quit multi-Rx operation for L3 measurement.

- Proposal 2: Additional condition for deactivating multi-Rx for scenario 1/2/3/4 could be considered as below:

  - After activating multi-Rx for L3 measurement based on serving cell’s signal quality, UE can deactivate multi-Rx operation when UE reports valid measurement result.

- Proposal 3: For L3 measurement, introduce scaling value (Pmulti-Rx) < 1 to reduce measurement delay

Table. Example of Proposed measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Pmulti-Rx x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Pmulti-Rx x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Pmulti-Rx x Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Reference

[1] R4-2420093, “WF on RRM requirements for NR_RRM_Ph5_Part1,” Apple.