3GPP TSG-RAN WG4 Meeting #114	R4-2500894

# Athens, Greece, 17th – 21st February, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Low Band CA operation

Agenda item:	7.8.3

Document for:	Discussion

Introduction

New WID low band CA via switching [1] has been approved, and the objectives are below.

The objectives are: Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In this contribution, we provide our views on RRM issues for low band CA via switching between FDD and SDL CC.

Discussion

For low band CA, it assumed that UE has single antenna to aggregate low band CC. When SCell operation is triggered by RRC based semi-static switching pattern, UE needs to switch to the SCell (Case 1  Case 2), and UE switches back to PCell after SCell operation (Case 2  Case 1)

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

- Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

Before going into detailed discussion on this WI, it is better to align the high level assumption of low band CA SCell operation for defining RRM requirements. Since both carriers are co-loacated and synchronized as mentioned in WID, SCell could borrow the AGC and T/F tracking from PCell, which means that it can be operated in SSB-less cell. On the other hand, one may have different view, e.g. SSB should be baseline.

- Proposal 1: RAN4 to specify assumption of low band CA SCell operation, e.g., SSB-less SCell or not.

SDL SCell should be activated before low band CA with switching is performed, so RAN4 needs to discuss SCell activation/deactivation delay for inter-band CA based on single antenna.

- Proposal 2: RAN4 to define SCell activation/deactivation delay for inter-band CA with switching

Based on WID, RAN1 will specify semi-static switching pattern based on RRC configuration. Based on the pattern, UE switches from Case 1 to Case 2 and vice versa, and after some switching delay, UE can recive the DL signal from the SCell. The switching delay from the RF perspective might be discussed in RF session, and based on the switching delay, RAN4 RRM needs to define overall switching delay to complete the switch carrier to receive DL signal.

- Proposal 3: RAN4 to define the delay requirement to complete the switch carrier to receive DL signal for inter-band CA with switching considering RF switching delay discussion

While receiving the singal from the SCell according to semi-static switching pattern, SMTC or reference signal from PCell could be overlapped. RAN4 needs to discuss whether to switch to PCell or maintain operation in SCell. Based on the conclusion, clear UE behavior should be defined in case of collision between PCell measurement and SCell data reception.

- Proposal 4: RAN4 to define UE behavior or related requirements in case of collision between PCell measurement and SCell data reception.

Conclusion

In this contribution, we provide our initial views on RRM issues for low band CA via switching between FDD and SDL CC, and we propose

- Proposal 1: RAN4 to specify assumption of low band CA SCell operation, e.g., SSB-less SCell or not.

- Proposal 2: RAN4 to define SCell activation/deactivation delay for inter-band CA with switching

- Proposal 3: RAN4 to define the delay requirement to complete the switch carrier to receive DL signal for inter-band CA with switching considering RF switching delay disucssion

- Proposal 4: RAN4 to define UE behavior or related requirements in case of collision between PCell measurement and SCell data reception.

Reference

[1] RP-243317, “New WID on low band carrier aggregation via switching”