3GPP TSG-RAN WG4 Meeting #114	R4-2500903

# Athens, GR, 17th – 21st February, 2025

Source:	ZTE Corporation

Title:	Discussion on the core maintenance of SSB-less SCell operation of Network energy saving for NR

Agenda Item:	5.27.1

Document for:	Approval

- Introduction

In RAN4#113 meeting, some discussion around the SSB-less SCell were held and the progress on core part were achieved in [1]. They are summarized as below:

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell Sub-issue 1: Whether servingCellMO can be present for inter-band SSB-less SCell. Agreement: RAN4 provide analysis on the categorization of intra-frequency/inter-frequency, whether MG is needed and any other aspects (if identified) for following cases: - When servingCellMO is configured - When servingCellMO is not configured RAN4 provide the above analysis to RAN2 (if consensus can be reached) and wait for RAN2 decision on whether servingCellMO is present for carriers of SSB-less SCell. Issue 1-1-2: EPRE conditions updating - Proposals - Option 1: When EPRE difference is beyond 12 dB and below 34 dB, UE is allowed one additional P-TRS for the AGC settling. - Option 2: Specify the threshold of Y as the value not smaller than 30dBm. A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y. Agreement: Conclude the number without [] in the next meeting. Issue 1-1-4: intra-band NCCA Agreement: The issue is closed.

In this document, we provide our views on the remaining issues around SSB-less SCell operation.

- Discussion

Feasibility conditions

In previous meeting, the side condition of power difference has been warmly discussed with the following agreements achieved:

Agreement: - For EPRE difference within X dB, keep the existing requirement. - X equals to or larger than 12 dB - For EPRE difference beyond X dB and no larger than Y dB, add one more P-TRS occurrence. - Further discuss whether the upper bound of Y is needed. If needed, Y is larger than 25dB. - From RAN4 perspective, AGC compensation for carrier frequency difference can be optionally implemented by UE. Remove “[after pre-compensation]” from the spec, i.e., not specify “AGC compensation” for carrier frequency difference in the spec. - Further discuss the X and Y values in this meeting. Try to conclude in the 2nd round on Thursday afternoon. Additional Agreement: X = 12; Y = [30]

Originally the pre-compensation is used by UE to overcome the predicted power difference between SSB-less SCell and reference cell since of the BW difference and carrier frequency difference, unfortunately some UE vendors raised concern on the UE implementation of pre-compensation. So the solution falls back to two set of requirements defined. Follow this logic, one threshold to distinguish the application of two requirements is needed, i.e. the threshold of X. When the EPRE difference is within X dB, the requirement of two P-TRS/A-TRS samples is assumed. Otherwise, the requirement of three P-TRS samples is assumed. For the former case, one TRS sample is used to perform AGC and the other TRS sample is used to perform fine time tracking. For the latter case, two TRS samples are used to perform AGC and the other TRS sample is used to perform fine time tracking. But for the latter case, even two TRS samples are kept to perform AGC, some company still shows concern on the risk if much larger EPRE difference is possible. To prevent such extreme case of much larger EPRE difference, the threshold of Y was introduced. The threshold Y implies the UE is still capable of performing AGC within 2 TRS samples under a EPRE difference as much as Y dB. Regarding the value of Y, we prefer the value not smaller than 30 dBm.

Proposal 1: Specify the threshold of Y as the value not smaller than 30dBm.

It should be noted that for A-TRS case, due to the special A-TRS bursts framework, so only two samples of measurement is allowed. So only one set of requirement is defined for the A-TRS case, which accommodates 2 samples to perform both AGC and fine time tracking.

Proposal 2: A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y.

Neighbor cells on carrier of SSB-less SCell

Due to time limited, this issue was discussed in last meeting but not deeply enough. While companies provide good analysis logically, this issue was divided into three sub-issues. We provide our further view on these sub-issues one by one.

- Sub-issue 1: Whether servingCellMO can be present for inter-band SSB-less SCell

- Actually in our paper submitted in last meeting, we provide overall analysis from the perspective of signalling. We raise this sub-issue not only triggered by RAN2 progress, but also aim to address the confliction existing in current RAN4 spec.

- RAN2 achieved the following agreements in RAN2#127bis:

Agreements 1 From RAN2 perspective, no benefit is identified for configuring servingCellMO for inter-band SSB-less SCell. 2 No LS is sent to R4. 3 Discuss in the next meeting on whether to capture “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO, depending on R4 progress. - 4 Postpone the discussion on intra-band SSB-less SCell.

- Since no RAN4 progress in Orlando meeting, RAN2 has no further discussion regarding the configuration of servingCellMO.

- Actually from RAN4 perspective, the more we should pay attention to, is how to guarantee no conflict between the SSB-less SCell definition and the demand of SSB based neighbour cell measurement. For the SSB-less SCell, if the SSB based neighbour cell measurement is expected at this frequency, under this case, for the sake of SSB based neighbour cell measurement, the SMTC should be configured in the relevant MO, but so as to meet the definition of SSB-less SCell as below, actually no SMTC could be configured. So how to address such conflict, one simple solution is not to declare this MO as ServingCellMo.

- The definition of SSB-less SCell is captured in the 38.133 as below: - For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell...

- Observation 1: There is conflict between the current SSB-less SCell definition and the demand of SSB based neighbour cell measurement.

- Regarding how to understand“the centre frequency of the SSB of the serving cell”, it should be clarified as the frequency configured in MO and declared as ServingCellMo instead of the absoluteFrequencySSB defined through ServingCellConfigCommon -> DownlinkConfigCommon ->FrequencyInfoDL -> absoluteFrequencySSB. Since the configuration of ServingCellConfigCommon -> DownlinkConfigCommon ->FrequencyInfoDL -> absoluteFrequencySSB is only applicable to PSCell/SCell, not any IE in ServingCellConfigCommonSIB/ServingCellConfigCommon/ServingCellConfigCommonSIB is used to configure the frequency of PCell as actually UE determines the SSB frequency of PCell by blind search of sync raster.

Proposal 3: From RAN4 perspective, we should pay more attention on how to guarantee no conflict between the SSB-less SCell definition and the demand of SSB based neighbour cell measurement.

- One straightforward solution to address the conflict between the SSB-less SCell definition and the demand of SSB based neighbour cell measurement is not to declare this MO as ServingCellMo in ServingCellConfig as RAN2 concluded. While if follow this, it is hard to category the SSB based neighbour cell measurement as intra-frequency or inter-frequency since lack of criteria, no “the centre frequency of the SSB of the serving cell” existence. It seems necessary to inform RAN2 about this consequence.

- Another solution to address the conflict between the SSB-less SCell definition and the demand of SSB based neighbour cell measurement is to allow the declaration of ServingCellMo but explicit configuration of SSB-less SCell is needed, then UE could recognize SSB-less SCell via specific signalling instead of the above rule in current 38.133. At the same time, the definition of intra-f or inter-f neighbour cell measurement is clear since the legacy criteria of comparing “the centre frequency of the SSB of the serving cell”with the centre frequency of the SSB of the neighbour cell still work.

Proposal 4: To meet the definition of SSB-less SCell, if SMTC is configured in the MO of SSB-less SCell, two solutions could be considered:

- Solution 1: The MO should not be declared as ServingCellMo in ServingCellConfig. Then the SSB of this MO is only transmitted from the neighbour cell.

- -  For the case of no SSB measurement demand at any neighbour cell of the same frequency, the MO of the SSB-less SCell can be declared as ServingCellMo in ServingCellConfig.

- Solution 2: No restriction on configuration of ServingCellMo. But introduce explicit signaling to configure SSB-less SCell.

- Considering RAN2 is still suspending to wait for RAN4 progress, it is better for RAN4 to conclude in this meeting, or at least to send LS to RAN2 to inform them what does RAN4 concern and the subsequent issues to address. RAN4 could internally discuss sub-issue 2 and sub-issue 3.

Proposal 5: Try to conclude in this meeting, at least to send LS to RAN2 to inform them what does RAN4 concern and the subsequent issues to address if no progress achieved in RAN4.

- Sub-issue 2: Definition on intra-frequency or inter-frequency

- Whether define the SSB based neighbour cell measurement at the SSB-less frequency as intra-frequency or inter-frequency, the main difference lies in the cell identification/measurement period. We compared the period requirements between intra-frequency and inter-frequency, for the measurement without gap, the sample numbers for intra-f/inter-f are consistent, so no difference on the period requirements. For the measurement with gap, inter-f cell identification and measurement require 3 more samples compared with the intra-f case. Therefore to reduce the measurement delay, it is better to define such neighbour cell measurement as intra-frequency type. it seems it could be defined as intra-frequency measurement since actually there is serving cell at this frequency.

Proposal 6: To reduce the measurement delay, it is better to define such neighbour cell measurement as intra-frequency type since actually there is serving cell at the frequency.

- Sub-issue 3: Whether MG is needed

Whether the neighbour cell measurement is performed with gap or without gap, to be simplify, follow the legacy rule specified Clause 9.2.1[2] for intra-frequency.

Proposal 7: Whether the neighbour cell measurement is performed with gap or without gap, follow the legacy rule specified Clause 9.2.1 for intra-frequency.

- Conclusion

In this contribution, we have the following proposals for the Network power saving:

Proposal 1: Specify the threshold of Y as the value not smaller than 30dBm.

Proposal 2: A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y.

- Observation 1: There is conflict between the current SSB-less SCell definition and the demand of SSB based neighbour cell measurement.

Proposal 3: From RAN4 perspective, we should pay more attention on how to guarantee no conflict between the SSB-less SCell definition and the demand of SSB based neighbour cell measurement.

Proposal 4: To meet the definition of SSB-less SCell, if SMTC is configured in the MO of SSB-less SCell, two solutions could be considered:

- Solution 1: The MO should not be declared as ServingCellMo in ServingCellConfig. Then the SSB of this MO is only transmitted from the neighbour cell.

- -  For the case of no SSB measurement demand at any neighbour cell of the same frequency, the MO of the SSB-less SCell can be declared as ServingCellMo in ServingCellConfig.

- Solution 2: No restriction on configuration of ServingCellMo. But introduce explicit signaling to configure SSB-less SCell.

Proposal 5: Try to conclude in this meeting, at least to send LS to RAN2 to inform them what does RAN4 concern and the subsequent issues to address if no progress achieved in RAN4.

Proposal 6: To reduce the measurement delay, it is better to define such neighbour cell measurement as intra-frequency type since actually there is serving cell at the frequency.

Proposal 7: Whether the neighbour cell measurement is performed with gap or without gap, follow the legacy rule specified Clause 9.2.1 for intra-frequency.

- Reference

- R4-2420121, “WF on RRM requirements for NR network energy saving”, Huawei

- TS 38.133 v18.8.0