3GPP TSG-RAN4 Meeting # 114	R4-2500944

Athens, Grace, Feb.17th – 21th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38101-2 - EESS protection(Annex H) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_mmWave_protect-Core |  |  |  |  |  | Date: |  |  | 2025-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce RF requirements to protect EESS in the 23.6-24.0 GHz frequency range (Annex H part.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Define modifiedMPRbehavior indication for newly NS_205 and CA_NS_205 for both n257 and n258. 2. Define modifiedMPRbehavior indication applies for legacy NS_202 for both n257 and n258. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ModifiedMPRbehavior indication wil not be reflected correctly in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex H.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change >

# H.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [13] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination. Moreover, the bits in the field can explicitly indicate NS value(s) supported by a UE.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [13] by an 8-bit bitmap per supported NR band.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band |  | Index of field (bit number) |  | Definition (description of the supported functionality if indicator set to one) |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n257 |  | 0 (leftmost bit) |  | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 |  | - This bit shall be set to 1 by a UE supporting n257 |  |
|  |  | 1 |  | - NS_202 as defined in clause 6.5.3.2.3 or both NS_202 and CA_NS_202 as defined in clause 6.5A.3.2.3 of 38.101-2 [v15.x.0] |  | - This bit shall be set to 1 by a UE supporting n257 or both n257 and CA_n257 |  |
|  |  | 2 |  | - NS_205 as defined in clause 6.5.3.2.6 or both NS_205 and CA_NS_205 as defined in clause 6.5A.3.2.5 of 38.101-2 [v15.x.0] |  | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |  |
| n258 |  | 0 (leftmost bit) |  | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 |  | - This bit shall be set to 1 by a UE supporting n258 |  |
|  |  | 1 |  | Void |  |  |  |
|  |  | 2 |  | - NS_203 as defined in clause 6.5.3.2.4 or both NS_203 and CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.12.0 |  | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |  |
|  |  | 3 |  | - NS_205 as defined in clause 6.5.3.2.6 or both NS_205 and CA_NS_205 as defined in clause 6.5A.3.2.5 of 38.101-2 [v15.x.0] |  | - This bit shall be set to 1 by a UE supporting n258 or both n258 and CA_n258 |  |
| n260 |  | 0 (leftmost bit) |  | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 |  | - This bit shall be set to 1 by a UE supporting n260 |  |
| n261 |  | 0 (leftmost bit) |  | - FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 |  | - This bit shall be set to 1 by a UE supporting n261 |  |

<End of change >