3GPP TSG-RAN4 Meeting # 114	R4-2500945

Athens, Grace, Feb.17th – 21th, 2025

# Source:	ZTE Corporation

# Title:	TP for TR38.749 Study outcome in previous releases(Clause 5)

# Agenda Item:	6.22.2

# Document for:	Approval

- Introduction

In this contribution, a TP to TR38.749[1] is provided for study outcome in previous releases(Clause 5).

Noted that the reference number in this TP align with the reference number in the approved TP to TR 38.749 on regulatory aspects in R4-2418920.

- Reference

- [1] R4-2417199	TR skeleton for Rel-19 EESS,  Huawei, HiSilicon

- Text Proposal

----- Start of TP -----

- 5	Study outcome in previous releases for EESS

In Rel-15, protection of EESS frequency range 23.6-24GHz was discussed with the following study outcomes:

- In RAN4#88 meeting, CR(R4-1811499) was agreed to introduce NS_201and CA_NS_201 for NR band n258, compliance with the additional spurious emission requirement of -8dBm/200MHz based on ECC decision [6].

-  In RAN4#92 meeting, Draft CR(R4-1910261) was endorsed to introduce NS_202 and CA_NS_202 for NR bands n257 and n258 with updates NS_201 and CA_NS_201 A-MPR requirements, compliance with -10dBm/100MHz additional spurious emission requirement to keep the consistence with ERC Rec. 74-01[3].

- In RAN4#95e meeting, CR(R4-2006785) was agreed to update the NS_202 requirements to stay consistent with EU emissions requirement changes (+1 dBm/200MHz).

In Rel-16, protection of EESS frequency range 23.6-24GHz was also discussed based on the outcome of WRC-19, the study outcome can be summarized in the following:

- In RAN4#97e meeting, CR (R4-2016785) was agreed to introduce NS_203 and CA_NS_203 for NR band n258 from Rel-15 TS38.101-2 specification, compliance with +1dBm/200MHz additional spurious emission requirement. ModifiedMPRbehaviour indication for NS_203 and CA_NS_203 is also captured in the specification.

- NS_201 and CA_NS_201 are obsolete.

- - In addition, RAN4 agreed to apply -5dBm/200MHz after Sep, 2027. But it is not reflected in RAN4 specification that revision is needed before the requirement taking effect, given that companies didn’t come to consensus. More details can be referred to the clause 7.5.4 in TR38.817-01.

----- End of TP -----