# 3GPP TSG-RAN WG4 Meeting #114                               R4-2500952

# Athens, GR, 17th – 21st February, 2025

# Title: 	Discussion on UE Capability/UE behavior and network signaling for 	Type 4a Type 4b EN-DC/NR-CA

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.2.2

# Document for:	Approval

# 1 Introduction

  - This contribution shares our views about UE Capability/UE behavior when When maxMIMO-layers ＞=4.

- Discussion

Some agreements are reached in RAN4 #113bis meeting [1] and some UE behaviour/NW signaling related issues need further discussion, one issue is the BS signaling to switch UE type with different type 1. The UE type 2 related UE capability and NW signaling are appended in the Annex for reference.

- <Issue 2-2-3: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b> Way forward: - When maxMIMO-layers = 2 - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present, UE is Type 1. - If Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent, UE is Type 2. NOTE: This is specified Rel-18 behavior. No need to further discuss. - When maxMIMO-layers ＞4 - FFS in the next meeting. - When maxMIMO-layers = 4 - FFS in the next meeting. NOTE: maxMIMO-layers means the existing BS signaling.

The above agreement from WF for maxMIMO-layers = 2 states that when maxMIMO-layers is configured 2 the UE is type 1 or type 2 regardless of whether the UE reports the UE type or whether the network is R18 or R19. The controversial point is the UE type determination when the UE is configured as maxMIMO layers=4 or maxMIMO layers>4.

When maxMIMO-layers ＞4, only Rel-19 type 1 can support the number of layers. So it is preferred to determine the UE type as Rel-19 type 1 regardless of how NW signaling is configured or combined to simplify the discussion.

Proposal 1. If maxMIMO-layer > 4 , Rel-19 type 1 is assumed.

When maxMIMO-layers =4, Rel-18 type 1 and Rel-19 type 4 can support the number of layers. So explicit indication for UE type is needed.

Figure 2-1 shows the situations for UE type determination when maxMIMO-layers =4.

Figure 2-1 UE type determination for the case when UE type 4 is reported

For figure 2-1(a), when the network is Rel-18 and maxMIMO-layers =4, if the signaling scheme of Rel-18 is followed, there is a contradiction between NW signaling and maxMIMO-layers =4. For this situation, we tend to determine UE type as UE type1.

For figure 2-1(b), when the network is Rel-19 and maxMIMO-layers =4, NW signaling needs to indicate Rel-18 UE type 1 or type 4. The situation where the Rel-19 signaling is not provided is equivalent to figure 2-1(a).

Proposal 2. If maxMIMO-layer = 4 and Rel-19 NW signaling is provided, the NW signaling needs to specify type 4 or type 1. If maxMIMO-layer = 4 and NW signaling is not provided, type 1 is assumed.

# 3 Conclusion

In this contribution, we give our viewpoint about requirements for type 4a/4b for NonCol_intraB_ENDC_NR_CA.

The conclusions are:

Proposal 1. If maxMIMO-layer > 4 , Rel-19 type 1 is assumed.

Proposal 2. If maxMIMO-layer = 4 and Rel-19 NW signaling is provided, the NW signaling needs to specify type 4 or type 1. If maxMIMO-layer = 4 and NW signaling is not provided, type 1 is assumed.

# 4	Reference

- R4-2420399,WF on non-collocated intra-band EN-DC and NR-CA, KDDI

# Annex

nonCollocatedTypeMRDC-r18 (NW signaling) This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16.

nonCollocatedTypeNR-CA-r18 (NW signaling) This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NRCA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

interBandMRDC-WithOverlapDL-Bands-r16(UE capability) -- R4 2-19 FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum Indicates whether the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [5] and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 [5] and inter-band RF requirements. If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [5] and intra-band RF requirements.

requirementTypeIndication-r18(UE capability) -- R4 33-2: Support network control of requirement network applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 Indicates whether the UE supports network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeMRDC-r18 for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands as specified in TS 38.331 [9]. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16.

intraBandNR-CA-non-collocated-r18 (UE capability) -- R4 33-1: Support of intra-band non-collocated NR CA operation Indicates whether the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [2], and TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. If the capability is not reported, the UE only supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. A UE supporting this feature shall also support network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeNR-CA-r18 for intra-band non-collocated NR-CA, as defined in TS 38.331 [9].