3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500989

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Further discussion on SAN RF core requirements for Ku-band for NR NTN

Agenda item:	7.11.5

Document for:	Approval

- Introduction

In RAN4#113 meeting, the FR1 and FR2 down selection for Ku-band NTN in WF[1] was agreed to define two sets of bands, one for FR1 numerology and other for FR2 numerology. So the SAN RF core requirements for FR1-NTN and FR2-NTN require further discussion.

In this contribution, we provide our views on SAN RF core requirements for Ku-band for NR NTN.

- Discussion

SAN type and RF core requirements

As per the WF [1], the agreement concerning FR1 and FR2 down selection for Ku-band NTN is shown as below:

WF on numerology: Agree to define two sets of bands, one for FR1 numerology and the other for FR2 numerology. RAN4 will further discuss SCS options.

As per the WF [2], the remaining issues concerning SAN RF core requirements are shown as below:

- Sub-topic 2-1: General issues for Ku band SAN RF requirements #### Issue 2-1-1: Supported SAN type for NR NTN Ku band - At least the OTA type of requirements will be specified, FFS on whether conducted type of requirements is considered

As per the updated WID for Ku-band NTN [3], the objectives concerning SAN RF core requirements are shown as below:

- Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band.

As above, the agreement on FR1 and FR2 down selection in WF [1] shows that the SAN RF core requirements for FR1-NTN and FR2-NTN need to be defined. From our understanding, for such 11GHz and 14GHz high frequency, in order to minimize high frequency cable loss and increase antenna gain, the SAN type 1-H with TAB connectors is not necessary, the SAN type 1-O and SAN type 2-O are sufficient. So, it is reasonable not to define RF requirement for SAN type 1-H.

Proposal 1: Not to define RF requirement for SAN type 1-H.

For OTA type of requirements, the initial applicability considerations for radiated requirements for SAN type 1-O and SAN type 2-O for NTN Ku-band separately are shown in Table 2-1.

Table 2-1 initial applicability considerations for radiated requirements for Ku-band NTN

| SAN RF requirements |  |  | SAN type 1-O for NTN Ku band | SAN type 2-O for NTN Ku band |
| --- | --- | --- | --- | --- |
| Radiated Tx Requirements |  |  |  |  |
| 9.2 Radiated transmit power |  |  | Need to check whether ±2.2 dB EIRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.3 OTA SAN output power |  |  | Need to check whether ±2 dB TRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.4 OTA output power dynamics | 9.4.2 OTA RE power control dynamic range |  | Reused. | Reused (N/A). |
|  | 9.4.3 OTA total power dynamic range |  | Reused if no new CBW will be introduced. | Reused if no new CBW will be introduced. |
| 9.5 OTA transmit ON/OFF power |  |  | Reused (N/A). | Reused (N/A). |
| 9.6 OTA transmitted signal quality | 9.6.1 OTA frequency error |  | Reused. | Reused. |
|  | 9.6.2 OTA modulation quality |  | Reused. | Reused. |
|  | 9.6.3 OTA time alignment error |  | Reused (N/A). | Reused (N/A). |
| 9.7 OTA unwanted emissions | 9.7.2 OTA occupied bandwidth |  | Reused. | Reused. |
|  | 9.7.3 OTA Adjacent Channel Leakage Power Ratio |  | FFS pending on co-existence study conclusion. | FFS pending on co-existence study conclusion. |
|  | 9.7.4 OTA out-of-band emissions |  | Reused. | Reused. |
|  | 9.7.5 OTA transmitter spurious emissions | General OTA transmitter spurious emissions requirements | Reused | Reused. |
|  |  | Protection of the SAN receiver | Reused (N/A). | Reused (N/A). |
|  |  | Additional spurious emissions requirements | Reused (N/A). | Reused (N/A). |
| 9.8 OTA transmitter intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| Radiated Rx Requirements |  |  |  |  |
| 10.2 OTA sensitivity |  |  | Reused. | Reused (no OTA sensitivity for SAN type 2-O, the OTA sensitivity is the same as OTA reference sensitivity level). |
| 10.3 OTA reference sensitivity level |  |  | Need to check whether noise figure for below 6GHz for SAN type 1-O is applicable for 14GHz for Ku-band. | Need to check whether EISREFSENS_50M declared range for Ka-band is applicable for Ku-band. |
| 10.4 OTA dynamic range |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |
| 10.5.1 OTA adjacent channel selectivity |  |  | FFS pending on co-existence study conclusion | FFS pending on co-existence study conclusion |
| 10.5.2 OTA in-band blocking |  |  | Reused (N/A). | Reused (N/A). |
| 10.6 OTA out-of-band blocking |  |  | Need to check whether 20MHz ΔfOOB for FUL,high – FUL,low < 100 MHz for SAN type 1-O is applicable for operating band characteristics in Ku-band | Reused. |
| 10.7 OTA receiver spurious emissions |  |  | Reused (N/A). | Reused (N/A). |
| 10.8 OTA receiver intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| 10.9 OTA in-channel selectivity |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |

The above initial applicability considerations for radiated requirements for Ku-band NTN can be as starting point.

Proposal 1: The following initial applicability considerations for radiated requirements for Ku-band NTN can be as starting point.

| SAN RF requirements |  |  | SAN type 1-O for NTN Ku band | SAN type 2-O for NTN Ku band |
| --- | --- | --- | --- | --- |
| Radiated Tx Requirements |  |  |  |  |
| 9.2 Radiated transmit power |  |  | Need to check whether ±2.2 dB EIRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.3 OTA SAN output power |  |  | Need to check whether ±2 dB TRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.4 OTA output power dynamics | 9.4.2 OTA RE power control dynamic range |  | Reused. | Reused (N/A). |
|  | 9.4.3 OTA total power dynamic range |  | Reused if no new CBW will be introduced. | Reused if no new CBW will be introduced. |
| 9.5 OTA transmit ON/OFF power |  |  | Reused (N/A). | Reused (N/A). |
| 9.6 OTA transmitted signal quality | 9.6.1 OTA frequency error |  | Reused. | Reused. |
|  | 9.6.2 OTA modulation quality |  | Reused. | Reused. |
|  | 9.6.3 OTA time alignment error |  | Reused (N/A). | Reused (N/A). |
| 9.7 OTA unwanted emissions | 9.7.2 OTA occupied bandwidth |  | Reused. | Reused. |
|  | 9.7.3 OTA Adjacent Channel Leakage Power Ratio |  | FFS pending on co-existence study conclusion. | FFS pending on co-existence study conclusion. |
|  | 9.7.4 OTA out-of-band emissions |  | Reused. | Reused. |
|  | 9.7.5 OTA transmitter spurious emissions | General OTA transmitter spurious emissions requirements | Reused | Reused. |
|  |  | Protection of the SAN receiver | Reused (N/A). | Reused (N/A). |
|  |  | Additional spurious emissions requirements | Reused (N/A). | Reused (N/A). |
| 9.8 OTA transmitter intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| Radiated Rx Requirements |  |  |  |  |
| 10.2 OTA sensitivity |  |  | Reused. | Reused (no OTA sensitivity for SAN type 2-O, the OTA sensitivity is the same as OTA reference sensitivity level). |
| 10.3 OTA reference sensitivity level |  |  | Need to check whether noise figure for below 6GHz for SAN type 1-O is applicable for 14GHz for Ku-band. | Need to check whether EISREFSENS_50M declared range for Ka-band is applicable for Ku-band. |
| 10.4 OTA dynamic range |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |
| 10.5.1 OTA adjacent channel selectivity |  |  | FFS pending on co-existence study conclusion | FFS pending on co-existence study conclusion |
| 10.5.2 OTA in-band blocking |  |  | Reused (N/A). | Reused (N/A). |
| 10.6 OTA out-of-band blocking |  |  | Need to check whether 20MHz ΔfOOB for FUL,high – FUL,low < 100 MHz for SAN type 1-O is applicable for operating band characteristics in Ku-band | Reused. |
| 10.7 OTA receiver spurious emissions |  |  | Reused (N/A). | Reused (N/A). |
| 10.8 OTA receiver intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| 10.9 OTA in-channel selectivity |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |

- Conclusion

This contribution provides analysis on SAN RF core requirements for Ku-band for NR NTN, and the following proposal is concluded as follows:

Proposal 1: Not to define RF requirement for SAN type 1-H.

Proposal 2: The following initial applicability considerations for radiated requirements for Ku-band NTN can be as starting point.

| SAN RF requirements |  |  | SAN type 1-O for NTN Ku band | SAN type 2-O for NTN Ku band |
| --- | --- | --- | --- | --- |
| Radiated Tx Requirements |  |  |  |  |
| 9.2 Radiated transmit power |  |  | Need to check whether ±2.2 dB EIRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.3 OTA SAN output power |  |  | Need to check whether ±2 dB TRP accuracy for below 6GHz is applicable for 11GHz for Ku-band. | Reused. |
| 9.4 OTA output power dynamics | 9.4.2 OTA RE power control dynamic range |  | Reused. | Reused (N/A). |
|  | 9.4.3 OTA total power dynamic range |  | Reused if no new CBW will be introduced. | Reused if no new CBW will be introduced. |
| 9.5 OTA transmit ON/OFF power |  |  | Reused (N/A). | Reused (N/A). |
| 9.6 OTA transmitted signal quality | 9.6.1 OTA frequency error |  | Reused. | Reused. |
|  | 9.6.2 OTA modulation quality |  | Reused. | Reused. |
|  | 9.6.3 OTA time alignment error |  | Reused (N/A). | Reused (N/A). |
| 9.7 OTA unwanted emissions | 9.7.2 OTA occupied bandwidth |  | Reused. | Reused. |
|  | 9.7.3 OTA Adjacent Channel Leakage Power Ratio |  | FFS pending on co-existence study conclusion. | FFS pending on co-existence study conclusion. |
|  | 9.7.4 OTA out-of-band emissions |  | Reused. | Reused. |
|  | 9.7.5 OTA transmitter spurious emissions | General OTA transmitter spurious emissions requirements | Reused | Reused. |
|  |  | Protection of the SAN receiver | Reused (N/A). | Reused (N/A). |
|  |  | Additional spurious emissions requirements | Reused (N/A). | Reused (N/A). |
| 9.8 OTA transmitter intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| Radiated Rx Requirements |  |  |  |  |
| 10.2 OTA sensitivity |  |  | Reused. | Reused (no OTA sensitivity for SAN type 2-O, the OTA sensitivity is the same as OTA reference sensitivity level). |
| 10.3 OTA reference sensitivity level |  |  | Need to check whether noise figure for below 6GHz for SAN type 1-O is applicable for 14GHz for Ku-band. | Need to check whether EISREFSENS_50M declared range for Ka-band is applicable for Ku-band. |
| 10.4 OTA dynamic range |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |
| 10.5.1 OTA adjacent channel selectivity |  |  | FFS pending on co-existence study conclusion | FFS pending on co-existence study conclusion |
| 10.5.2 OTA in-band blocking |  |  | Reused (N/A). | Reused (N/A). |
| 10.6 OTA out-of-band blocking |  |  | Need to check whether 20MHz ΔfOOB for FUL,high – FUL,low < 100 MHz for SAN type 1-O is applicable for operating band characteristics in Ku-band | Reused. |
| 10.7 OTA receiver spurious emissions |  |  | Reused (N/A). | Reused (N/A). |
| 10.8 OTA receiver intermodulation |  |  | Reused (N/A). | Reused (N/A). |
| 10.9 OTA in-channel selectivity |  |  | FFS pending on further IoT of own system evaluation | FFS pending on further IoT of own system evaluation |

- References

- R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#113

- R4-2416563, Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112bis

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#106

- R4-2413525, Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104