3GPP TSG-RAN WG4 Meeting #114                                                                                                      R4-2500991

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Discussion on framework simplification for co-location/co-existence requirements

Agenda item:	9.1

Document for:	Approval

- Introduction

In RAN#106 meeting, the WF [1] on Task RAN4 to work on framework simplification for co-location/co-existence requirements was agreed.

In this contribution, we provide our views on framework simplification for co-location/co-existence requirements.

- Discussion

2.1 Co-location/co-existence requirements and specifications

As contribution [2] in RAN#106, there are 15 BS specifications (listed below) are related to framework simplification for co-location/co-existence requirements:

38.106

38.115-1

38.174

38.176-1

38.176-2

36.104

36.141

37.104

37.141

38.104

38.141-1

38.141-2

37.105

37.145-1

37.145-2

We think that the Transmitter spurious emissions (Additional spurious emissions requirements, Co-location with other base stations), and Out-of-band blocking (Co-location with other base stations) in above 15 BS specifications need to be discussed for framework simplification for co-location/co-existence requirements, since we need more work to update them for new new co-location/co-existence operating bands at same time.

Observation 1: Transmitter spurious emissions (Additional spurious emissions requirements, Co-location with other base stations), and Out-of-band blocking (Co-location with other base stations) in 15 BS specifications need to be discussed for framework simplification.

As mentioned above, there are so many specifications related to framework simplification for co-location/co-existence requirements, it is very difficult to find the differences between them to solve the problem. For the sake of discussion, let’s first discuss how to simplify the core specifications for co-location/co-existence requirements, since the conformance specifications can take the same approach as core specifications.

Observation 2: First discuss how to simplify the core specifications for co-location/co-existence requirements, since the conformance specifications can take the same approach as core specifications.

The 6 BS core specifications related to framework simplification for co-location/co-existence requirements are listed below:

38.106

38.174

36.104

37.104

38.104

37.105

2.2 Framework simplification for Transmitter spurious emissions additional spurious emissions requirements

To compare difference of additional spurious emissions requirement in the 6 BS core specifications, we use additional spurious emissions requirement for the UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 co-existence system as example in the following Table 2.2-1 to analyze.

Table 2.2-1: Example for comparing the difference of additional spurious emissions requirement in the 6 BS core specifications

| Specification | System type to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note | Apply for |
| --- | --- | --- | --- | --- | --- | --- |
| 36.104 | UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 - 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band 1 or 65, since it is already covered by the requirement in clause 6.6.4.2. | E-UTRA BS |
| 37.104 | UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 - 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band 1 or 65, since it is already covered by the requirement in sub-clause 6.6.1.2. | Multi-Standard Radio (MSR) BS |
| 37.105 | The MSR spurious emission basic limits are the same as those specified in 3GPP TS 37.104. The single RAT UTRA FDD spurious emission basic limits are the same as those specified in 3GPP TS 25.104. The single RAT UTRA TDD spurious emission basic limits are the same as those specified in 3GPP TS 25.105. The single RAT E-UTRA operation spurious emission basic limits are the same as those specified in 3GPP TS 36.104. |  |  |  |  | Active Antenna System (AAS) BS |
| 38.104 | UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 – 1980 MHz | -49 dBm | 1 MHz | This requirement does not apply to BS operating in band n1 or n65, since it is already covered by the requirement in clause 6.6.5.2.2. | NR BS |
| 38.106 | UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 – 1980 MHz | -49 dBm | 1 MHz | This basic limit does not apply to repeater operating in band n1 or n65, since it is already covered by the basic limit in clause 6.6.5.2.2. | repeater |
| 38.174 | UTRA FDD Band I or E-UTRA Band 1 or NR Band n1 | 1920 – 1980 MHz | -49 dBm | 1 MHz |  | IAB-DU and IAB-MT |

As above, for TS 37.105, it uses reference approach to define transmitter spurious emissions additional spurious emission requirement, so the transmitter spurious emissions additional spurious emission requirement in TS 37.105 is not needed to be simplified.

From No. 2, 3, 4 and 5 column of above table, the System type to co-exist with, Frequency range for co-existence requirement, Basic limits, and Measurement bandwidth in 36.104, 37.104, 38.104, 38.106 and 381.74 are same. But from No. 6 column of above table, the Note in 36.104, 37.104, 38.104, 38.106 and 381.74 are different, the Note in 36.104, 37.104 indicate that the E-UTRA Band does not apply, but the Note in 38.104, 38.106, and 38.174 indicate that NR band does not apply. Since E-UTRA Band number is different from NR band number, so the Note in 38.104, 38.106, and 38.174 can’t refer to Note in 36.104, 37.104. It is needed to consider how to deal with the difference of Note for transmitter spurious emissions additional spurious emissions requirements in 36.104, 37.104, 38.104, 38.106, and 38.174 for framework simplification.

Observation 3: It is needed to consider how to deal with the difference of Note for transmitter spurious emissions additional spurious emissions requirements in 36.104, 37.104, 38.104, 38.106, and 38.174 for framework simplification.

2.3 Framework simplification for Transmitter spurious emissions Co-location with other base stations

To compare difference of transmitter spurious emissions requirement for co-location with other base stations in the 6 BS core specifications, we use transmitter spurious emissions requirement for co-location with other base stations for the UTRA FDD Band XI or E-UTRA Band 11 co-location system as example in the following Tables 2.3-1, 2.3-2, and 2.3-3 to analyze.

Table 2.3-1: Example for comparing the difference of transmitter spurious emissions requirement for co-location with other base stations in 36.104, 37.104, 37.105 and 38.104

| Specification | Type of co-located BS | Frequency range for co-location requirement | Basic limits |  |  | Measurement bandwidth | Note | Apply for |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA BS | MR BS | LA BS |  |  |  |
| 36.104 | UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 –1447.9 MHz | -96 dBm | N/A | N/A | 100 kHz | This is not applicable to E-UTRA BS operating in Band 50 or 75 | E-UTRA BS |
| 37.104 | UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band 50, 51, 75, 76 | Multi-Standard Radio (MSR) BS |
| 37.105 | The MSR spurious emission basic limits are the same as those specified in 3GPP TS 37.104. The single RAT UTRA FDD spurious emission basic limits are the same as those specified in 3GPP TS 25.104. The single RAT UTRA TDD spurious emission basic limits are the same as those specified in 3GPP TS 25.105. The single RAT E-UTRA operation spurious emission basic limits are the same as those specified in 3GPP TS 36.104. |  |  |  |  |  |  | Active Antenna System (AAS) BS |
| 38.104 | UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 –1447.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to BS operating in Band n50, n75, n91, n92, n93 or n94 | NR BS |

Table 2.3-2: Example for comparing the difference of transmitter spurious emissions requirement for co-location with other base stations in 38.106

| Specification | Type of co-located BS | Frequency range for co-location requirement | Basic limits |  |  | Measurement bandwidth | Note | Apply for |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA repeater | MR repeater | LA repeater |  |  |  |
| 38.106 | UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 –1447.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz | This is not applicable to repeater operating in Band n50, n75, n91, n92, n93 or n94 | repeater |

Table 2.3-2: Example for comparing the difference of transmitter spurious emissions requirement for co-location with other base stations in 38.174

| Specification | Type of co-located BS | Frequency range for co-location requirement | Basic limits |  |  | Measurement bandwidth | Note | Apply for |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA IAB-DU and WA IAB-MT | MR IAB-DU | LA IAB-DU and LA IAB-MT |  |  |  |
| 38.174 | UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 –1447.9 MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  | IAB-DU and IAB-MT |

As above, for TS 37.105, it uses reference approach to define transmitter spurious emissions requirement for co-location with other base stations, so the transmitter spurious emissions requirement for co-location with other base stations in TS 37.105 is not needed to be simplified.

For TS 36.104, there are no the Basic limits for MR BS and LA BS, and only Basic limits for WA BS are defined. From above tables, the System type to co-exist with, Frequency range for co-existence requirement, Basic limits, and Measurement bandwidth for transmitter spurious emissions requirement for co-location with other base stations in 36.104, 37.104, 38.104, 38.106 and 381.74 are same. But the Note for transmitter spurious emissions requirement for co-location with other base stations in 36.104, 37.104, 38.104, 38.106 and 381.74 are different. The Note in 36.104 is that Band 50 or 75 does not apply, the Note in 37.104 is that Band 50, 51, 75, 76 does not apply, the Note in 38.104 is that Band n50, n75, n91, n92, n93 or n94 does not apply, the Note in 38.106 is that Band n50, n75, n91, n92, n93 or n94 does not apply, but there is not Note in TS 38.174. It is needed to consider how to deal with the difference of Note for transmitter spurious emissions for co-location with other base stations in 36.104, 37.104, 38.104, 38.106, and 38.174 for framework simplification.

Observation 4: It is needed to consider how to deal with the difference of Note for transmitter spurious emissions for co-location with other base stations in 36.104, 37.104, 38.104, 38.106, and 38.174 for framework simplification.

2.4 Framework simplification for Out-of-band blocking Co-location with other base stations

To compare difference of out-of-band blocking requirement for co-location with other base stations in the 6 BS core specifications, we use out-of-band blocking requirement for co-location with other base stations for the UTRA FDD Band XI or E-UTRA Band 11 co-location system in 36.104, 37.104 and 37.105 as example summarized in following Table 2.4-1 to analyze.

Table 2.4-1: Example for comparing the difference of out-of-band blocking requirements for co-location with other base stations in 36.104, 37.104, 37.105

| Specification | Type of co-located BS | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power for WA BS (dBm) | Interfering Signal mean power for MR BS (dBm) | Interfering Signal mean power for LA BS (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal | Apply for |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36.104 | UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 –1495.9 | +16** | N/A | N/A | PREFSENS + 6dB* | CW carrier | E-UTRA BS |
| 37.104 | UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 | +16** | +8** | -6** | PREFSENS + x dB* | CW carrier | MSR BS |
| 37.105, For MSR | UTRA FDD Band XI or E-UTRA Band 11 | 1 475.9 - 1 495.9 | +16 | +8 | -6 | PREFSENS + x dB (NOTE 1) | CW carrier | AAS BS |
| 37.105, For single RAT UTRA | The single RAT UTRA FDD AAS BS of Wide Area BS class may optionally fulfil minimum requirements for co-location blocking specified in 3GPP TS 25.104 [6], subclause 7.5.2. … |  |  |  |  |  |  |  |
| 37.105, For single RAT E-UTRA | The single RAT E-UTRA AAS BS of Wide Area BS class may optionally fulfil minimum requirements for co-location blocking specified in 3GPP TS 36.104 [8], subclause 7.6.2. … |  |  |  |  |  |  |  |

As above, for UTRA FDD Band XI or E-UTRA Band 11 co-location system, the out-of-band blocking requirements for co-location with other base stations in 36.104, 37.104, 37.105 (for MSR) are same. so, out-of-band blocking requirements for co-location with other base stations in 36.104, 37.104, 37.105 (for MSR) can be simplified.

For TS 38.104, the out-of-band blocking Co-location minimum requirements for BS type 1-C and BS type 1-H in Table 7.5.3-1 of TS 38.104 are copied as below:

Table 7.5.3-1: Blocking performance requirement for NR BS when co-located with BS in other frequency bands.

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS +6dB (Note 1) | +16 | +8 | x (Note 2) | CW carrier |
| NOTE 1: PREFSENS depends on the BS channel bandwidth as specified in Table 7.2.2-1, 7.2.2-2, and 7.2.2-3. NOTE 2: x = -7 dBm for NR BS co-located with Pico GSM850 or Pico CDMA850 x = -4 dBm for NR BS co-located with Pico DCS1800 or Pico PCS1900 x = -6 dBm for NR BS co-located with UTRA bands or E-UTRA bands or NR bands NOTE 3: The requirement does not apply when the interfering signal falls within any of the supported uplink operating band(s) or in ΔfOOB immediately outside any of the supported uplink operating band(s). NOTE 4: For unsynchronized base stations (except in band n46, n96 and n102), special co-location requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |  |

The OTA out-of-band blocking Co-location minimum requirements for BS type 1-O in 10.6.2.2-1 of TS 38.104 are copied as below:

Table 10.6.2.2-1: OTA blocking requirement for co-location with BS in other frequency bands

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | EISminSENS + 6 dB (Note 1) | +46 | +38 | +24 | CW carrier |
| NOTE 1: EISminSENS depends on the BS class and on the BS channel bandwidth, see clause 10.2. NOTE 2: The requirement does not apply when the interfering signal falls within any of the supported uplink operating band(s) or in ΔfOOB immediately outside any of the supported uplink operating band(s). |  |  |  |  |  |

For TS 38.106, there are no out-of-band blocking co-location minimum requirements in TS 38.106.

For TS 38.174, the Out-of-band blocking Co-location minimum requirements for IAB-DU type 1-H in TS 38.174 use reference approach as below:

Minimum requirement is the same as specified for BS type 1-H in TS 38.104 [2], subclause 7.5.3.

The Out-of-band blocking Co-location minimum requirements for IAB-MT type 1-H in Table 7.5.6-1 of TS 38.174 are copied as below:

Table 7.5.6-1: Blocking performance requirement for the IAB Node

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA IAB Node (dBm) | Interfering signal mean power for LA IAB Node (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS +6dB (Note 1) | +16 | x (Note 2) | CW carrier |
| NOTE 1: PREFSENS depends on the IAB-MT channel bandwidth as specified in subclause 7.2.1 and subclause 7.2.2. NOTE 2: x = -7 dBm for IAB-MT co-located with Pico GSM850 or Pico CDMA850 x = -4 dBm for IAB-MT co-located with Pico DCS1800 or Pico PCS1900 x = -6 dBm for IAB-MT co-located with UTRA bands or E-UTRA bands or NR bands NOTE 3: The requirement does not apply when the interfering signal falls within any of the supported downlink operating band(s) or in ΔfOOB immediately outside any of the supported downlink operating band(s). |  |  |  |  |

And, the OTA out-of-band blocking Co-location minimum requirement for IAB-MT type 1-O and IAB-DU type 1-O in Table 10.6.4-1 of TS 38.174 are copied as below:

Table 10.6.4-1: OTA blocking requirement for co-location with BS or IAB-Node in other frequency bands

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA IAB-DU and WA IAB-MT (dBm) | Interfering signal mean power for MR IAB-DU (dBm) | Interfering signal mean power for LA IAB-DU and LA IAB-MT(dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | EISminSENS + 6 dB (Note 1) | +46 | +38 | +24 | CW carrier |
| NOTE 1: EISminSENS depends on the IAB class and on the IAB channel bandwidth, see clause 10.3. NOTE 2: The requirement does not apply when the interfering signal falls within any of the supported downlink operating band(s) or in ΔfOOB immediately outside any of the supported downlik operating band(s). |  |  |  |  |  |

As above the out-of-band blocking Co-location minimum requirements in TS 38.104, 38.106, and 38.174, when introducing new frequency bands in RAN4 specifications, the out-of-band blocking Co-location minimum requirements in TS 38.104, 38.106, and 38.174 are not required to be updated. From our understanding, the method for out-of-band blocking Co-location minimum requirements in TS 38.104, 38.106, and 38.174 in a interfering band agnostic way can be used to simplify out-of-band blocking Co-location minimum requirements for 36.104, 37.104, 37.105 (for MSR).

Observation 5: The method for out-of-band blocking Co-location minimum requirements in TS 38.104, 38.106, and 38.174 in a interfering band agnostic way can be used to simplify out-of-band blocking Co-location minimum requirements for 36.104, 37.104, 37.105 (for MSR).

From above analysis, NR specifications (38.104, 38.174, and 38.106) have similar Note for spurious emission co-location/co-existence requirements, and LTE specifications (36.104, 37.104 and 37.105) have similar Note for spurious emission co-location/co-existence requirements, but the Note for NR specifications is different from that for LTE specifications. So for core specifications, it is hard to use any one of 6 BS core specifications as baseline for reference approach due to the difference of Note in 6 BS core specifications.

Based on above discussion, the following two methods can be considered for 6 BS core specifications framework simplification:

Method1: Create one new core specification for co-existence /co-location requirement, all 6 BS core specifications can be referred to new core specification.

Method 2: Update 38.174 and 38.106 with reference to 38.104, and Update 36.104 and 37.105 with reference to 37.104.

Observation 6: The following two methods can be considered for 6 BS core specifications framework simplification:

Method1: Create one new core specification for co-existence /co-location requirement, all 6 BS core specifications can be referred to new core specification.

Method 2: Update 38.174 and 38.106 with reference to 38.104, and Update 36.104 and 37.105 with reference to 37.104.

At this stage, only R19 version will be defined for 5G, so method 2 is more suitable for 5G, while method 1 is more appropriate 6G. So we can take different method for 5G and 6G.

Proposal 1: For 6G core specifications, create one new core specification for co-existence /co-location requirement, all 6 BS core specifications can be referred to new core specification.

Proposal 2：For 5G NR and LTE core specifications, Update 38.174 and 38.106 with reference to 38.104, and Update 36.104 and 37.105 with reference to 37.104.

- Conclusion

This contribution provides analysis on framework simplification for co-location/co-existence requirements, and the following proposal is concluded as follows:

Observation 1: Transmitter spurious emissions (Additional spurious emissions requirements, Co-location with other base stations), and Out-of-band blocking (Co-location with other base stations) in 15 BS specifications need to be discussed for framework simplification.

Observation 2: First discuss how to simplify the core specifications for co-location/co-existence requirements, since the conformance specifications can take the same approach as core specifications.

Observation 3: It is needed to consider how to deal with the difference of Note for transmitter spurious emissions additional spurious emissions requirements in 36.104, 37.104, 38.104, 38.106, and 38.174 for framework simplification.

Observation 4: It is needed to consider how to deal with the difference of Note for transmitter spurious emissions for co-location with other base stations in 36.104, 37.104, 38.104, 38.106, and 38.174 for framework simplification.

Observation 5: The method for out-of-band blocking Co-location minimum requirements in TS 38.104, 38.106, and 38.174 can be used to simplify out-of-band blocking Co-location minimum requirements for 36.104, 37.104, 37.105 (for MSR).

Observation 6: The following two methods can be considered for 6 BS core specifications framework simplification:

Method1: Create one new core specification for co-existence /co-location requirement, all 6 BS core specifications can be referred to new core specification.

Method 2: Update 38.174 and 38.106 with reference to 38.104, and Update 36.104 and 37.105 with reference to 37.104.

Proposal 1: For 6G core specifications, create one new core specification for co-existence /co-location requirement, all 6 BS core specifications can be referred to new core specification.

Proposal 2：For 5G NR and LTE core specifications, Update 38.174 and 38.106 with reference to 38.104, and Update 36.104 and 37.105 with reference to 37.104.

- References

- RP-243288, Moderator's summary of discussion about R19-RAN4-BSRF, RAN4 Chair (Huawei), RAN#106

- RP-242767, On new objective under NR_BS_RF_req_evo, Nokia, Ericsson, ZTE, Huawei, HiSilicon, RAN#106