3GPP TSG-RAN WG4 Meeting #114	R4-2501020

# Athens , GR, 17th – 21st Feb, 2025

Title: 	Discussion on Event triggered L1 measurement reporting for Rel-19 LTM enhancements

Source: 	CATT

Agenda item:	7.27.2.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the RRM requirements for event triggered L1 measurement reporting were discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the RRM impact for event triggered L1 measurement for LTM and give our views.

- Discussion

- 2.1 L1 Filtering assumption for event triggered L1 report

The candidate solutions for filtering assumption event triggered L1 report in the last meeting are copied as follows [1]:

<Candidate options> - Option 1: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT, ZTE, CTC, E///) - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. - Otherwise, there is no impact on RAN4. - Option 2: In event triggered L1 measurement in mobility, do not consider L1 filter for requirements. (Samsung, Xiaomi, CMCC, Apple, HW) - Option 2a: RAN4 to define RRM requirements for the event triggered L1 report with the assumption of no specific L1 filtering. This decision can be revisited if other working groups overturn their decision on L1 filtering. (QC)

As we know, the filtering discussed in RAN1 is mainly to resolve the ping-pong effect when switching, and it is L1 specified filtering for time and spatial domain for event triggered L1 report not for event evaluation. However, there is still no conclusion on whether to introduce such a L1 filtering in RAN1.

Since the current measurement requirements defined in R18 LTM didn’t consider the any filtering, if L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report. Otherwise, there is no impact on RAN4.

Observation 1: The L1 filtering discussed in RAN1 is mainly to resolve the ping-pong effect when switching, and it is L1 specified filtering for time and spatial domain for event triggered L1 report.

Observation 2: The current measurement requirements defined in R18 LTM didn’t consider the any filtering.

Proposal 1: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input.

- If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

- Otherwise, there is no impact on RAN4.

- 2.2 Event triggered reporting delay for L1 reporting

In the last meeting, RAN4 agreed the high level event-triggered L1 reporting requirement for SSB based measurements to follow legacy framework of L3 measurement. The agreement for L1 event triggered reporting delay is copied as below [3]:

<Agreement> - Agree on the following high level proposal: - The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period [on the cells corresponding to the event or on the candidate cells]. - FFS reuse L1-RSRP measurement period from the existing spec with Treport = 0. - Additional time TSSB_time_index for SSB index acquisition is needed, unless deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell.

We are generally fine with the above agreement, but there is still  controversy about the first bullet and further discussion is needed.

The controversy on the attributive of L1 RSRP measurement period in the first bullet lies in whether L1 measurement includes service cells in addition to candidate cells. Some companies pointed that MIMO considers both current cell and candidate cell needs to be measured, but we think that RAN4 need to decouple MIMO and LTM. We support to follow legacy to only compare the measured candidate cells [3].

Proposal 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period on the candidate cells.

- 2.3 Impact of UL resource waiting time

In the last meeting, the Candidate options for Impact of UL resource waiting time is copied as below [1]:

<Candidate options> - Option 1: For L1 event triggered reporting, it needs to specify that L1 result which triggers L1 reporting will be included in L1 reporting. (Xiaomi) - It means that if L1 reporting procedure starts, UE can continue to measure for L1, however, the following L1 result will not be evaluated for event anymore. - Option 2: for event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting. (CMCC, Apple, vivo) - Option 3: RAN4 to discuss components and criteria for defining TTime to first UL channel (E///) - Once the event is met, UE needs to send the indication on the SR or special SR to the NW to indicate to the NW that event is met. It can be denoted as TTime to first UL channel.

Even though there is a new special SR for L1 reporting agreed in RAN2, it is only one use case and it can be part of the test configuration in perf stage according to the last meeting. Therefore, we prefer to follow the same approach for event triggered L3 measurement reporting to support option 2.

Proposal 3: For event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

- Conclusion

This document discussed the issues for event triggered L1 measurement for LTM and the following proposals and observations are presented:

Observation 1: The L1 filtering discussed in RAN1 is mainly to resolve the ping-pong effect when switching, and it is L1 specified filtering for time and spatial domain for event triggered L1 report.

Observation 2: The current measurement requirements defined in R18 LTM didn’t consider the any filtering.

Proposal 1: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input.

- If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

- Otherwise, there is no impact on RAN4.

Proposal 2: The event triggered measurement reporting delay shall be no larger than the L1-RSRP measurement period on the candidate cells.

Proposal 3: For event triggered L1 measurement reporting, it is proposed that measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on, which is to follow the approach for event triggered L3 measurement reporting.

- Reference

[1]      R4-2420101 WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

[2]      R4-2418285 Topic summary for [113][227] NR_Mob_Ph4_Part1, Apple

[3]      3GPP TS 38.133, NR Requirements for support of radio resource management