3GPP TSG-RAN WG4 Meeting # 114                                                              R4-2501032

Athens, GR, 17th – 21st February, 2025

Title: 	                 Discussion on fragmented DL carriers

Source: 	CATT

Agenda item:	7.6.3

Document for:	Discussion

- Introduction

This contribution provides our analysis about the maximum input level for fragmented carriers according to the WF [1].

- Discussion

In WF [1], the maximum power level of in-gap interference needs to be further studied.

Issue 3-2: Discussion on maximum power level of in-gap interference

WF:

For one Rx RF chain, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm, specific value is FFS

For intra-band NC CA, the existing ACS interference signal power level limited value is up to -25 dBm, the carriers in CA don’t fully share Rx chain, and each Rx chain has BPF, which can provide suppression to the frequency range outside the passband. In general, filter suppression increases as the frequency range to filter passband offset increases, and it can be seen in figure 1. ACS interference signals frequency range are adjacent to wanted signal frequency range, so the in-gap and out-of-gap interference signal power level can be greater than or equal to -25 dBm.

Figure 1 Carrier 1 and carrier 2 have separate BPF

Figure 2 Two carriers fully share the Rx chain

For fragmented carriers, the two carriers fully share the Rx chain, and the in-gap interference will fall into the passband of the filter, rejection of filter is very limited, and it can be seen in figure 2, so the maximum power of the in-gap interference should be less than or equal to -25dBm.

Proposal 1: For one Rx RF chain, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm.

- Summary

This contribution provides our analysis about the maximum power level of in-gap interference according to the WF [1], and we have the following proposal:

Proposal 1: For one Rx RF chain, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm.

Reference

[1] R4-2420519, “WF on fragmented DL study”, Mediatek, RAN4#113