3GPP RAN WG4 Meeting #114	R4-2501034

Athens, GR, February 17th – February 21st, 2025

Title: 	Discussion on general aspects for AIML for NR air

Source: 	CATT

Agenda item:	7.19.1

Document for:	Discussion

Introduction

In RAN4#113 meeting, RAN4 continued discussing the general aspects of Rel-19 AI/ML for NR air interface and the agreements were captured in [1]. Some open issues summarized in [2] need further discussions and we would like to present our views on some of the open issues in this contribution.

Discussion

Requirement goals

For AI/ML based beam management, the test target depends on the way of how the performance requirements will be defined. When defining performance requirements, different levels of overhead reduction need to be taken into account for a given SINR. For example, predicting 64 beams based on measuring 8 beams or 16 beams respectively, leading to 87.5% and 75% overhead reduction, and different performance requirements are defined for both cases. Now assuming that the reduction ratio is 0, which means SetB is the same as SetA, and then prediction is performed. In this case, the performance requirements of AI/ML based BM shall not be worse than the legacy requirements given the same SINR, i.e., no performance degradation is expected.

Next assuming that reduction ratio increases, whether lower RSRP results are allowed is the key point in this issue. If the legacy accuracy values are reused for all overhead reduction ratios, it means only one set of performance requirements will be defined and it is exactly the same as the legacy requirements. Option 2 means that higher RSRP results will be defined, but it is not the main purpose of this feature which is to reduce overhead rather than improving performance. Option 3 is that lower RSRP results are allowed. Considering different overhead reduction ratios, larger margin values can be defined for higher overhead reduction ratios. For instance, ±1 dB margin can be defined for overhead reduction ratio of 50% and ±2 dB margin for 75%.

Observation 1: Test target of AI/ML based BM depends on the way of how performance requirements will be defined.

Proposal 1: For AI/ML based beam management, two options can be considered for defining performance requirements:

- Option 1: The legacy performance requirements are reused, regardless of overhead reduction ratio.

- Option 2: The legacy performance requirements are used as baseline, and larger margin values are defined for higher overhead reduction ratios.

Post deployment testing

In RAN4#112bis meeting, a concrete summary capturing the issues that are considered for post deployment testing and potential options were captured in the WF [1]. In brief, the following two issues are the targets for post deployment testing:

•	Issue 1: Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance.

•	Issue 2: Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance.

To resolve issue 1, two options are proposed:

•	Option 1: Conduct the validation of a change in AI functionality before its deployment/activation in already deployed UEs (in the field or in lab conditions).

•	Option 2: Using performance monitoring and LCM procedures.

Issue 1 can be divided into different scenarios according to the types of changes. For the changes referring to update or fine tuning, conducting the validation in lab conditions cannot ensure a consistent performance between field and post deployment testing, since the parameters are updated or fine-tuned are highly scenario-dependent. Even the updated functionality passes the post deployment testing, its performance in the field cannot be guaranteed. If the validation is conducted in the field, from our perspective, it is the same as the LCM procedures, i.e., performance monitoring.

Observation 2: For changes due to update or fine tuning of models,

- Validation in lab conditions cannot ensure a consistent performance between field and post deployment testing, because the updates or fine tuning are scenario-dependent.

- Validation in the field is the same as the performance monitoring procedure.

For the changes referring to addition/removal of models, the discussion needs to be based on RAN2 agreements on the definitions of functionality and procedures. The related RAN2 agreements are reproduced below [3]:

RAN2#127: Agreements on definitions 1 Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) 2 Applicable functionalities refers to functionalities that the UE is ready to apply for inference 3 Activated functionalities refers to functionalities already enabled for performing inference Agreements on procedures - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

As highlighted contents show, after addition/removal of models is completed and before corresponding functionalities can start to perform inference, UE needs to decide the applicability of functionalities. Therefore whether a functionality impacted by addition/removal of models can be activated or not can rely on the applicability checking procedure. At least at current stage, we do not see necessary validations of functionalities impacted by changes after UE is deployed.

Observation 3: For changes due to addition/removal of models, applicability checking procedure of functionality can handle the potential impacts on functionality performance.

For issue2, the impacts mainly come from data drift / mismatch, resulting from inconsistent scenarios between field and the training phase. In our understanding, the purpose of the post deployment testing for this issue is trying to identify the reasons why AI/ML functionality performs badly in the field. Firstly, it is already beyond the purpose of RAN4 performance testing which tests AI/ML capability and its basic performance. Secondly, even such post deployment testing is carried out and the reason is found out, what UE can do is, for example, to switch functionality or fallback to non-AI functions, which is also a part of LCM procedures to us.

Hence, RAN4 should prioritize Option 2, and consider Option 1 if it is proved necessary in some scenarios which should be clarified by the proponents of Option 1.

Observation 4: For impacts due to data drift/mismatch, the post deployment testing is actually trying to identify the reasons why AI/ML functionality performs badly in the field, which is beyond the scope of RAN4 performance testing and belongs to management of AI/ML functionality.

Proposal 2: Impacts due to changes of models and data drift / mismatch be handled by at least the following procedures:

- Performance monitoring and LCM procedures

- applicability checking procedure of functionality

Proposal 3: RAN4 prioritizes Option 2 and proponents of Option 1 further clarify the motivation and applicable scenarios.

Number of tests

This issue was brought up a few meeting ago without any agreements achieved and actually depends on the definition of functionality. In RAN1#119 meeting, the following RAN2 terminologies on beam management were clarified by RAN1 [4]:

RAN1#119 Agreement: In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “ functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

According to the above agreement, the granularity of “functionality” for applicable and activated functionality may be the same and may refer to CSI-ReportConfig, while the granularity of “functionality” for supported functionality may be a feature group which has a wider scope. In our opinion, RAN4 needs to define at least one test case for a single supported functionality. If plural test cases will be defined for a single supported functionality, RAN4 can change the parameters by referring to the contents of IE CSI-ReportConfig.

Proposal 4: RAN4 to define at least one test case for a supported functionality, i.e., a feature group. Defining plural test cases can base on the contents of IE CSI-ReportConfig.

Conclusions

This paper discussed the general issues related to AI/ML for NR air interface, and following observations and proposals are provided:

Observation 1: Test target of AI/ML based BM depends on the way of how performance requirements will be defined.

Proposal 1: For AI/ML based beam management, two options can be considered for defining performance requirements:

- Option 1: The legacy performance requirements are reused, regardless of overhead reduction ratio.

- Option 2: The legacy performance requirements are used as baseline, and larger margin values are defined for higher overhead reduction ratios.

Observation 2: For changes due to update or fine tuning of models,

- Validation in lab conditions cannot ensure a consistent performance between field and post deployment testing, because the updates or fine tuning are scenario-dependent.

- Validation in the field is the same as the performance monitoring procedure.

Observation 3: For changes due to addition/removal of models, applicability checking procedure of functionality can handle the potential impacts on functionality performance.

Observation 4: For impacts due to data drift/mismatch, the post deployment testing is actually trying to identify the reasons why AI/ML functionality performs badly in the field, which is beyond the scope of RAN4 performance testing and belongs to management of AI/ML functionality.

Proposal 2: Impacts due to changes of models and data drift / mismatch be handled by at least the following procedures:

- Performance monitoring and LCM procedures

- applicability checking procedure of functionality

Proposal 3: RAN4 prioritizes Option 2 and proponents of Option 1 further clarify the motivation and applicable scenarios.

Proposal 4: RAN4 to define at least one test case for a supported functionality, i.e., a feature group. Defining plural test cases can base on the contents of IE CSI-ReportConfig.

References

[1] R4-2420336, WF on AI/ML air interface part2, Ericsson, RAN4#113.

[2] R4-2418152, Topic summary for [113][130] NR_AI/ML_air_part2, Ericsson , RAN4#113.

[3] R2_127_ChairNotes_24-08-23_17-00_eom, RAN2#127.

[4] Chair notes RAN1#119 eom1, RAN1#119.