3GPP RAN WG4 Meeting #114	R4-2501039

Athens, GR, Feburary 17th – Feburary 21st, 2025

Title: 	Discussion on testing issues for CSI compression two-sided models

Source: 	CATT

Agenda item:	7.20.3

Document for:	Discussion

Introduction

In RAN4#113 meeting, RAN4 discussed the testability and interoperability issues in Rel-19 AI/ML for NR air interface and agreements were captured in [1]. But some testability and interoperability issues for two-sided models need to be further discussed. In this contribution, we will continue to discuss those issues.

Discussion

Option 3 analysis

In RAN4#113 meeting, RAN4 chose three decoders with low, medium, high SGCS among the decoders submitted by companies and two tricks were provided for further simulations:

Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset) - The 3 decoders are selected based on low, medium, high mean SGCS out of available decoders - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different. Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s). - At least an encoder with the agreed model structure should be considered when making own encoder. Optionally companies can try own encoder with their preferred structure. Companies should report the structure if different.

We mainly analyzed track 1 and observed that when using the test decoder and corresponding dataset (the one used to train that test decoder) to train a new encoder, the SGCS performance can reach an expected level. If the corresponding dataset is not used, it requires that the training dataset used to train the encoder has the same characteristic as that of the corresponding dataset. When the characteristics are not aligned, the performance of newly trained encoder and test decoder pair is lower, much lower in some cases, than the performance of joint training.

Observation 1: When both test decoder and the dataset used to train the test decoder are used to train an initialized encoder, the SGCS performance can reach an expected level.

Proposal 1: RAN4 to define test decoder and capture the corresponding training dataset.

Furthermore, we tried to change the structure of encoder and evaluated the performance. We found that if the structure of encoder has an obvious difference (e.g., more ResNet blocks are added), the training cannot converge, i.e., the SGCS performance could not reach the expected level. Therefore it would be beneficial to capture the structure of encoder for reference.

Proposal 2: RAN4 to document the structure of encoder for reference, and further study where and how to capture the structure of reference encoder.

Conclusions

This contribution discussed some testability and interoperability issues for AI/ML enabled CSI compression and some observations and proposals are provided:

Observation 1: When both test decoder and the dataset used to train the test decoder are used to train an initialized encoder, the SGCS performance can reach an expected level.

Proposal 1: RAN4 to define test decoder and capture the corresponding training dataset.

Proposal 2: RAN4 to document the structure of encoder for reference, and further study where and how to capture the structure of reference encoder.

References

[1] R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated, RAN4#113.