3GPP RAN WG4 Meeting #114	R4-2501040

Athens, GR, February 17th – February 21st, 2025

Title: 	Discussion on impacts of AIML mobility on RRM requirements

Source: 	CATT

Agenda item:	7.21.2

Document for:	Discussion

Introduction

In RAN4#113 meeting, RAN4 discussed the RRM impacts of AI/ML based mobility and the agreements were captured in the WF [1]. In this contribution, we will further discuss the open issues listed in [2] and present our views.

Discussion

L3 RSRP prediction accuracy definition

In last meeting, RAN4 discussed the definition of L3 RSRP prediction accuracy with the following agreement achieved:

RAN4#113: Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2

The ground truth in the definition needs to be further discussed for FR1 and FR2 where the interested scenarios and test methods are different, which affects the definition of ground truth. According to RAN4 agreements in RAN4#112bis, the following scenarios and use cases are prioritized in RAN4:

RAN4#112bis: Agreements: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. Scenario number; Priority; Evaluation scenario; Target study goal; Methodology. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Agreement: - Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

Considering that AI/ML based beam management is still discussing the details of OTA test methodology, including the definition of ground truth which is not agreed yet. Therefore discussion on the definition of ground truth in FR2 in this study item can be postponed and RAN4 should focus on the aspects in FR1.

Proposal 1: RAN4 to postpone the discussion on the definition of ground truth in FR2.

In FR1, RAN4 already agreed that conducted testing will be adopted, in which case test equipment has the knowledge of ideal measurement results at UE side for each corresponding beam. Consequentially, TE can calculate the L3 cell-level measurement results based on the known beam-level measurement results, beam consolidation/selection method and L3 filtering coefficients. Hence, ground truth for L3 filtered cell-level RSRP in FR1 can be the ideal L3 cell-level RSRP results for both temporal and frequency domain predictions.

Proposal 2: The definition of ground truth in FR1 can be the ideal L3 filtered cell-level measurement results for both temporal and frequency domain predictions.

Impacts on beam level measurements

In RAN4#112bis, the following agreement was achieved for further discussion:

RAN4#112bis: Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

For 2nd and 3rd bullets, considering that RAN2 still deprioritizes L3 filtered beam-level prediction sub-use cases and they focuses on FR2 intra-frequency temporal domain case A, i.e., scenario 4, at the current stage, RAN4 can study the impacts of L1 beam-level on RRM requirements only.

TR 38.744: 3 sub-use cases are considered for beam-level RRM measurement prediction: Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s) Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: L3 filtered beam level prediction sub-use cases are lower priority and should focus on FR2 intra-frequency temporal domain case A.

Proposal 3: RAN4 not to consider studying impact on RAN4 requirement due to L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

LCM related requirements

Regarding the LCM related requirements, some proposals were brought up and recommended WFs were provided in last meeting:

RAN4#113: - Recommended WF Candidate options / tentative agreements: - Option 1: RAN4 should focus first on performance metrics and relevant requirements and post-pone the discussion on LCM related issues. - Option 2: RAN4 should start discussion LCM related issues in parallel of discussion on performance metrics.

From our perspective it is pre-mature to discuss LCM related requirements at current stage considering this is a study item and actually more RAN2 progress is needed. In addition, the outcomes in Rel-19 AI/ML Air Interface WI, which is not closed yet, can also be reused. Hence, there is no need to discuss LCM requirements in two AI/ML items in parallel. We suggest agreeing the recommended WF Option 1.

Proposal 4: RAN4 should focus first on performance metrics and relevant requirements and post-pone the discussion on LCM related issues.

Conclusions

This contribution discussed the impacts of AI/ML mobility on RRM requirements and some proposals are provided:

Proposal 1: RAN4 to postpone the discussion on the definition of ground truth in FR2.

Proposal 2: The definition of ground truth in FR1 can be the ideal L3 filtered cell-level measurement results for both temporal and frequency domain predictions.

Proposal 3: RAN4 not to consider studying impact on RAN4 requirement due to L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

Proposal 4: RAN4 should focus first on performance metrics and relevant requirements and post-pone the discussion on LCM related issues.

References

[1] R4-2420095, WF on AI/ML Mobility, Nokia, RAN4#113.

[2] R4-2418278, Topic summary for [213][220] FS_NR_AIML_Mob, Nokia, RAN4#113.