3GPP TSG RAN WG4 Meeting #114		R4-2501068

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Further discussion on system parameters for LP-WUS

Agenda Item:	7.26.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

Regarding whether or not to introduce channel raster for LP-WUR, RAN4#113 has made the following agreements, which concludes no channel raster introduction, however the need of the limited set of frequencies supported by LP-WUR could be further discussed [1]:

Issue 1-2-1: Channel raster for LP-WUR Agreements: - No new channel raster is introduced dedicated for LP-WUS/WUR. - FFS on whether there is need of the limited set of frequencies supported by LP-WUS to reduce the receiver complexity and cost

RAN4 also agreed on PRB grids alignment between LP-WUS and NR, and also flexible allocation of LP-WUS within the wider NR carrier, which leads to the fact that the centre frequency of LP-WUS is always a multiple of 10kHz for NR bands with 100kHz channel raster. In this contribution, we propose to make clarification on this in both in TP and CR. Moreover, since there is no explicit channel raster defined for LP-WUS, it is not clear how a UE in RRC_IDLE mode obtains the information on the centre frequency of the LP-WUS assigned to it, hence we propose to inquire RAN2 by sending an LS.

# 2 Discussion

In previous meetings, RAN4 has made the following agreements on the number of RBs for LP-WUS with 15kHz SCS and LP-WUS allocation [2]:

Issue 1-2-3: number of RBs for LP-WUS with 15kHz SCS Agreements: - Follow RAN1 agreement for number of RBs for LP-WUS with 15KHz SCS, i.e., 11RBs.

Issue 1-2-2: considerations on LP-WUS allocation Agreements: - RAN4 to assume PRB grid alignment between LP-WUS and NR. - RAN4 to assume the LP-WUS RBs can be flexibly allocated within the wider NR carrier.

As illustrated in Fig. 1, for an NR band with 100kHz channel raster, the distance between the centre frequency of the NR carrier and LP-WUS depends on the oddity of the number of PRBs of the NR carrier,

Fig. 1, WUS and NR PRB grids

The centre frequency of the LP-WUS carrier for both cases can be expressed uniformly as:

c (t) = n*100kHz + q*SCS*12 + m*6*SCS         (1)

where n, q is an integer, and m = 0 or 1.

For 15kHz SCS, the Eq. (1) is further expressed as:

c(t) = n*100kHz + q*180kHz + m*90kHz           (2)

Actually, Eq. (2) can be simplified as:

c(t) = n*100kHz +p * 90kHz                         (3)

where p is an integer between -272 and 272.

The final set indicated by Eq. (3) is {10kHz, 20kHz, 30kHz, …, m*10kHz, …}, which means c(t) = t * 10k. Therefore, a channel raster with 10kHz can ensure the flexible allocation of an LP-WUS carrier within a wider NR carrier with 100kHz channel raster.

Hence, this alignment ensures that the centre frequency of LP-WUS within an NR carrier with 100kHz channel raster is always a multiple of 10 kHz allowing for reduced cost and complexity in the frequency synthesizer of an LP-WUR by generating frequencies in the form of n*10 kHz.

Observation: The alignment of PRB grids between LP-WUS and NR, along with the flexible allocation of LP-WUS within the wider NR carrier, ensures that the centre frequency of LP-WUS is always a multiple of 10 kHz for NR bands with a 100 kHz channel raster.

It is also beneficial to make clarification on this both in TR and TS by adding a sentence “This alignment ensures that the centre frequency of LP-WUS is always a multiple of 10 kHz for NR bands with 100kHz channel raster”. Our companion papers capture the proposed updates in TP and draft CR respectively [3][4][5].

Proposal 1: RAN4 to make clarification that the centre frequency of LP-WUS is always a multiple of 10 kHz for NR bands with a 100 kHz channel raster to allow for reduced cost and complexity in the frequency synthesizer of an LP-WUR.

If a UE supports LP-WUS, then it means LP-WUS should be supported in all three RRC states, including RRC_IDLE. For RRC_CONNECTED and RRC_INACTIVE, the UE can be notified by signalling from network with the information on the centre frequency of LP-WUS assigned to this UE, however, for a UE in RRC_IDLE mode, it does not know the information since there is no LP-WUS channel raster defined, thus the UE cannot do a frequency scanning to find a valid LP-WUS, unless the information is broadcast in some SIB. With such consideration, we propose to send an LS to ask RAN2 the question to reach a common and clarified understanding.

Proposal 2: Send an LS to RAN2 inquiring how a UE in RRC_IDLE mode obtains the information on the centre frequency of the LP-WUS assigned to this UE.

# 3 Conclusion

In this contribution, we have the following observation and proposals on system parameters for LP-WUS:

Observation: The alignment of PRB grids between LP-WUS and NR, along with the flexible allocation of LP-WUS within the wider NR carrier, ensures that the centre frequency of LP-WUS is always a multiple of 10 kHz for NR bands with a 100 kHz channel raster.

Proposal 1: RAN4 to make clarification that the centre frequency of LP-WUS is always a multiple of 10 kHz for NR bands with a 100 kHz channel raster to allow for reduced cost and complexity in the frequency synthesizer of an LP-WUR.

Proposal 2: Send an LS to RAN2 inquiring how a UE in RRC_IDLE mode obtains the information on the centre frequency of the LP-WUS assigned to this UE.

# 4 Annex – draft LS

3GPP TSG RAN WG4 Meeting #114 R4-250xxxx Athens, Greece, Feb 17th – 21st , 2025 Title: LS on acquisition on the LP-WUS location for a UE supporting LP-WUR in RRC_IDLE state Response to: Release: Rel-19 Work Item: NR_LPWUS-Core Source: RAN4 To: RAN2 Cc: Contact Person: Name: Aijun CAO E-mail Address: ajcao@catt.cn Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: - 1. Overall Description: RAN4 has discussed the channel arrangements for LP-WUS/LP-WUR and agreed that PRB grid should be aligned between LP-WUS and NR, and the LP-WUS RBs can be flexibly allocated within the wider NR carrier. With such agreements, the center frequency of LP-WUS is always a multiple of 10 kHz for NR bands with a 100 kHz channel raster. However, RAN4 also agreed that no channel raster is introduced dedicated for LP-WUS/LP-WUR, hence, it is not clear to RAN4 how a UE in RRC_IDLE state supporting the feature can acquire the information on the center frequency of the LP-WUS assigned to this UE, unless the information is broadcast in some SIB. 2. Actions: To RAN WG2: RAN4 respectively asks RAN2 to take into account the above information and to answer the question above. 3. Date of Next TSG-RAN Meetings: TSG RAN WG4 Meeting #114-bis Apr. 17 – Apr. 21, 2025 China TSG RAN WG4 Meeting #115 May 19 – May 23, 2025 Malta

# Reference

[1] R4-2420528, “WF on UE RF requirements for LP-WUS”, vivo

[2] R4-2417112, “WF on UE RF requirements for LP-WUS”, vivo

[3] R4-2501069, “TP to TR 38.774 on Clause 5 (System parameters)”, CATT

[4] R4-2501070, “draftCR to TS 38.101-1 on system parameters for LP-WUS”, CATT

[5] R4-2501071, “draftCR to TS 38.101-2 on system parameters for LP-WUS”, CATT