3GPP TSG RAN WG4 #114	R4-2501323

Athens, Greece, Feb 17 – 21, 2025

Agenda item:		7.13.3

Source:	Nokia

Title:	On OTA test enhancements

Document for:		Approval

# Introduction

A Way Forward [1] was agreed in RAN4#113 regarding OTA test enhancements. This contribution further elaborates Nokia’s views on the items related to OTA test enhancements.

# Discussion

## CLTA definition

The following was agreed in the WF [1] of RAN4#113:

Way forward: - It is confirmed that other (than CLTA) test antennas are already allowed by existing network-node test specifications to be used for the OTA co-location requirements testing, but technical description for using another antenna is not defined. - Further investigate horn antennas as alternative (to CLTA) test antenna for high frequency bands. Provide inputs on the following aspects (not limited to): - Horn antenna performance evaluation and comparison with CLTA. - Test setup configuration for horn antennas, e.g. test antenna placement, etc. - Frequency range applicable for horn antennas. - Investigate whether there are feasible solutions to reassure that alternative (to CLTA) test methods provide the same EUT testing results (e.g. how to avoid situation, where one test method would pass, and the other would fail). Note: Analysis on other antennas than horn antennas are not precluded.

We have extended our horn antenna performance evaluation and comparison with CLTA with a new measurement campaign. Below is the analysis of the measurements with two different wideband horn antennas. We have selected the antennas to cover the needed frequency range for FR1, but the frequency range of wideband horn antennas is not limited to this. They are readily available for frequencies at least up to 110 GHz.

We have proposed in our earlier contributions that using wideband horn antennas as CLTA’s could be very beneficial concerning cost and test efforts. Single column passive antennas are not readily available for all of the required frequency bands, and they typically meet the CLTA requirements only at relatively narrow frequency ranges. Currently a lot of custom-made antennas are needed in many cases. This makes testing especially in higher and lower frequencies become highly complex and time-consuming. However, there is a need to ensure that wideband horn antennas deliver correct results. To see how wideband horn antennas behave when used as CLTAs with the same setup was tested. The spurious emissions measurements for 3GPP frequency bands were done for two AAS and it was measured with side-by-side distances of 10cm and 20cm for CLTA and the horn antenna on both sides as depicted in setup A and B, respectively. In the first measurement campaign, three measurements were repeated in order to secure some average result for the spurious emissions. This time the averaging was done over 200 samples. The horn antenna vertical alignment was in the center of the DUT and both +45° and -45° polarizations were measured for each setup. The setups are shown in Figure 1 and Figure 2. One DUT was measured with each setup.

Figure 1. Measurement setup for the wideband horn antenna measurements.

Similar setup was used for the CLTAs as depicted in Figure 2.

Figure 2. Measurement setup for the CLTA

The results show also in this measurement campaign that when optimizing the setup to minimize noise, there is no difference in the noise level between a wideband horn antenna and the CLTA. If the DUT has no spurs above the noise level, the results are very similar. When there are spurs visible above the noise level, they are both seen by the wideband horn antenna and the CLTA. In our previous contribution we saw that as the spurs are fluctuating in time, more averaging is needed to get more accurate results. But even with 200 samples averaging, there are still differences in the results. The results show clearly that wideband horn antennas perform very well in detecting low power signals. Figures below show the measurement results as the difference of spurious emission level measured with CLTA and two different wideband horn antennas. Measurements were done with standard 10cm distance and also doubling the distance to 20cm to see what effect the distance plays on these measurements.

Note: In figures below, positive difference indicates horn antenna gives higher value vs. CLTA

Figure 3. DUT2 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup A, 10cm distance

Figure 4. DUT2 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup B, 10cm distance.

Figure 5. DUT2 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup A, 10 cm distance.

Figure 6. DUT2 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup B, 10cm distance.

Figure 7. DUT2 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup A, 20cm distance.

Figure 8. DUT2 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup B, 20cm distance.

Figure 9. DUT2 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup A, 20cm distance.

Figure 10. DUT2 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup B, 20cm distance.

As one can see from the results, wideband horn antennas do not give exactly same results as CLTAs. But stillit is impossible to say which one gives more correct results. As the measurements are done in the near-field, even the slightest change in the placement of the measurement antenna has an effect. One thing to note here is that the horn antennas receive different frequency bands in different parts of the antenna. The higher frequencies are received in the back part of the antenna, and it might be needed to move the horn antenna forward according to measured frequency band. During the measurements, the front part of the horn antenna was aligned with the radome of the DUT, which might not give correct results.. One option would be to use a predefined spurs that could be altered in power level and frequency. If the power level and the frequency of the spurs were known, then the comparison measurement would have a great meaning. It can be also seen from the measurement results that doubling the distance between the DUT and the measurement antenna doesn’t necessarily correspond to lower measurement result. More studies are needed if there is any reason to change to currently defined measurement distance of 10cm.

Increasing the averaging to 200 samples does not bring significant benefit.

It would be beneficial to have predefined and tunable spurs to verify test antennas performance.

Doubling the measurement distance to 20cm doesn’t guarantee lower results.

## BS-to-BS isolation analysis

Regarding the BS-to-BS isolation analysis, the following was agreed in RAN4#113:

Way forward: - Provide inputs on the proposed BS-to-BS coupling derivation methodology, considering valid co-location deployment scenarios: - Impact of beam steering for coupling analyses, - Investigation on the set of impacted specifications (core, test, etc.), - Investigation on impact for specific requirements (spurious emission, receiver blocking, [TDD OFF power] and TX IMD) - Investigation on the set of impacted OTA co-location test requirements, - Considered deployments to be modelled (horizontal antennas stacking, vertical stacking, etc.), - Other inputs not precluded. - Aim at band-agnostic solution for BS-to-BS isolation analysis, prioritizing FR1 bands (higher bands beyond FR1 as second priority).

The need to enable co-location requirements that reflect the actual deployments of BS’s especially for the high FR1 bands has been emerging in RAN4. Currently the requirements have the same limit for the lower frequencies and the higher frequencies. RAN4 needs to investigate the BS-to-BS coupling loss that would enable to have reasonable values for higher FR1 frequencies that would then correlate to more accurate co-location requirements. As the WF indicates, there are multiple things to consider, and we list our view on each of the items below.

### Impact of beam steering

Beam steering in AAS is a crucial technology that enables improved coverage, increased capacity, enhanced signal quality, dynamic resource allocation, and reduced power consumption. These benefits contribute to a more efficient and reliable wireless network. Considering a side-by-side deployment with horizontal beam steering, where the majority of beam steering occurs and the side lobes are the worst, then the coupling loss might be completely different when the main beam is steered in the direction of the victim BS. We have demonstrated the effect of beam steering in our contributions R4-2415674 [2] and R4-2418018 [3] and showed it has a significant effect. The steering range and other parameters for the simulations need to be agreed for the simulation setup.

Beam steering is a technology of paramount importance for AAS.

Beam steering need to be considered as an important parameter in the simulation setup.

### Impacted specifications

We expect that the possibly impacted specifications will be the BS radio transmission and reception specifications TS36.104, TS37.104, TS37.105 and TS38.104. Also, there is an impact on the conducted and radiated conformance testing specifications TS36.141, TS37.141, TS 37.145-1, TS37.145-2, TS38.141-1 and TS38.141-2. There is also expected impact on the corresponding specifications for IAB and repeaters.

Radio transmission and reception specifications together with conducted and radiated conformance testing specifications for BS, IAB and repeaters could be impacted.

### Impacted requirements

The impacted requirements will be the co-location requirements TX spurious emissions, RX blocking,TX IMD and TDD OFF power requirements.

TX spurious emissions, RX blocking, TX IMD and TDD OFF power requirements could be impacted.

### Impacted OTA co-location test requirements

Only the conducted testing requirement limits for the co-location requirements might change depending on the coupling level results achieved in the simulations. Those would differ from the current limits based on the frequency and on the agreed simulation assumptions. The OTA limits, however, would not be affected due to the calculation of the limits from the conducted ones. Currently the distance of the CLTA and the DUT has been set to 10cm to reflect the 30dB coupling loss. If the coupling loss would be larger for higher frequencies, there could be a need to adjust the distance between CLTA and DUT accordingly. This, however, should be studied further as there might not be much effect if the distance is increased.

Conducted requirement limits might change depending on the coupling level.

OTA requirement limits would not change.

If results indicate a larger coupling loss for higher frequencies, then the distance between CLTA and DUT could be increased to reflect higher coupling loss. Needs to be studied further.

### Deployments to be modelled

As the situation where two AAS are located side-by-side would be the worst-case scenario due to beamforming happening mostly on the horizontal plane, it should be prioritized in the simulations. It is still a viable method how co-location of AAS is done in the sites and would produce the worst coupling loss results. Other methods can be studied as well but all companies should prioritize this method firstly.

Beam-steering is applied mostly on the horizontal plane and side-by-side is the worst case in regarding beam steering.

Prioritize side-by-side deployment in the evaluation of the BS-to-BS isolation.

# Conclusion

This document shares Nokia’s views on OTA test enhancements. Following observations and proposals were introduced:

- Increasing the averaging to 200 samples does not bring significant benefit.

- It would be beneficial to have predefined and tunable spurs to verify test antennas performance.

- Doubling the measurement distance to 20cm doesn’t guarantee lower results.

- Beam steering is a technology of paramount importance for AAS.

- Radio transmission and reception specifications together with conducted and radiated conformance testing specifications for BS, IAB and repeaters could be impacted.

- TX spurious emissions, RX blocking, TX IMD and TDD OFF power requirements could be impacted.

- Conducted requirement limits might change depending on the coupling level.

- OTA requirement limits would not change.

- If results indicate a larger coupling loss for higher frequencies, then the distance between CLTA and DUT could be increased to reflect higher coupling loss. Needs to be studied further.

- Beam-steering is applied mostly on the horizontal plane and side-by-side is the worst case in regarding beam steering.

- Beam steering need to be considered as an important parameter in the simulation setup.

- Prioritize side-by-side deployment in the evaluation of the BS-to-BS isolation.

# References

- R4-2419797 “Way Forward for [113][303] NR_BS_RF_Part2_CLTA”, RAN4#113, Orlando, USA, 18th – 22nd November, 2024.

- R4-2415674 ”On OTA test enhancements - CLTA”, RAN4#112bis, Hefei, China, 14th – 18th October, 2024.

- R4-2418018 ”On OTA test enhancements”, RAN4#113, Orlando, USA, 18th – 22nd November, 2024.