3GPP TSG RAN WG4 #114	R4-2501327

Athens, Greece, Feb 17 – 21, 2025

Agenda item:		7.23.1

Source:	Nokia

Title:	On SBFD general aspects and system parameters

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112bis [1] regarding evolution of NR duplex operation. There were not so many agreements made regarding SBFD general aspects and system parameters. Also, the open items are listed in the WF. The most important topic is related to subband configurations and guardbands. This contribution displays Nokia’s view on SBFD general aspects and system parameters.

# Discussion

## CLI handling impact on RF requirement

The following was agreed in RAN4#112 and remained open for RAN4#112bis:

- Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

Regarding the FFS, our preference is to define the BS RF requirements without considering the potential impact of gNB-to-gNB CLI handling mechanisms, i.e., the FFS should not be considered. First, for the sake of progress we should not risk slowing down the work in RAN4 with revisiting some requirements pending on RAN1 progress. Secondly, the CLI handling schemes discussed in RAN1 are mainly applicable to co-channel CLI and their applicability to adjacent channel CLI is questionable. In fact, RAN1 discussions on gNB-to-gNB CLI handling currently focuses on enabling time/frequency coordination and spatial coordination (beam nulling and beam pairing). All the discussed mechanisms require, at least,

i) exchange of CLI-RS configurations across gNBs,

ii) identification of aggressor gNBs via measurements,

and iii) exchange across gNBs of measurement results and additional information to enable the aggressor-victim coordination.

All the above rely on signaling between gNBs over the Xn interface (intra-operator signaling). However, there is no standardized inter-operator interface and therefore exchange of information between operators is not possible. Moreover, the discussed CLI handling schemes are only optional features that are not expected to be supported by all the base stations in the field. Lastly, the impact of each CLI handling on the RF requirements is quite challenging to measure due to the fact that it is quite dependent on the specific deployment, load conditions, frequency of the information exchange across nodes, etc. The arguments above support the following proposal:

RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

## DL/UL subband configurations support for SBFD

### Clarification on declaration and flexibility

The following was agreed in RAN4#113:

Agreement: - The SBFD capable BS product shall not operate any SBFD configuration (DL/UL subband) other than those that have been declared - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - FFS on how to capture this in the specification. - This restriction shall at least be applied to FR1 WA BS - FFS other BS classes - FFS the possibility of testing the worst-case scenario in each RF front end operational configuration from the supported SBFD configurations. - FFS how to capture this in the specifications

The agreement mentioned above is valid for FR1 WA BS and in our view, this is also applicable to FR1 MR BS. As FR1 MR BS is very similar to FR1 WA BS and has high output power in the top of the range, the same restriction should also be applied for FR1 MR BS. For the possibility of testing the worst-case scenario in each RF front end, operational configuration would be very beneficial to reduce required testing, but as we do not have any fixed configurations and each vendor can have a different one, it will be almost impossible to define the worst-case scenario.

The restrictions agreed for FR1 WA BS are also applicable for FR1 MR BS.

### DL/UL subband configurations support for SBFD

The following was agreed in RAN4#113:

Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

In the last RAN4 meeting, there was no consensus on whether the number of UL subband sizes to be declared by the gNB is 2 or 3. In the same meeting, it has also been agreed that the gNB shall not operate any declared configuration that has not been tested. Our view is that we should limit the complexity and testing at the gNB, therefore, we support that maximum 2 specific UL subband sizes are allowed to be declared.

Regarding the first FFS in this agreement, we should discuss if we support different sizes for specific UL subband for DUD and DU/UD. In our view, one of the 2 specific UL subband sizes can be applicable to the DUD configuration, and the other to the DU/UD configuration.

Regarding both the DL and the UL subband sizes, we support that the restriction is also applicable to the FR1 MR BS.

For a specific channel BW supported for SBFD, maximum of 2 specific UL subbands sizes are allowed to be declared.

The restriction on maximum number of specific UL subband sizes to be declared is also applicable to the FR1 MR BS.

The restriction on maximum number of specific DL subband sizes to be declared is also applicable to the FR1 MR BS.

### Other restriction on DL subband size

The following was agreed in RAN4#113:

Way Forward: - FFS the following restriction on DL subband size: - For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size). - FR2 minimum CBW to support SBFD is 100MHz.

The restriction on DL subband size mentioned above can be agreed on.

For FR2, considering SSB bandwidth and sync raster step size of 28.8 MHz and 17.28 MHz. respectively, 100 MHz seem sufficient for DU/UD case, but not for DUD case. Note that RAN1 is still discussing whether SBFD slots containing SSB transmissions should be still considered as ‘SBFD slots’ or should be converted to ‘DL only slots’ instead. Under the assumption that SSBs are transmitted on SBFD slots, 200 MHz would be needed for DUD case.

For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size) and FR2 minimum CBW to support SBFD is 100MHz for DU/UD case and 200MHz for DUD case. The minimum bandwidth requirement for FR2 DUD case can be revisited depending on future RAN1 agreements on SSB transmission on SBFD symbols.

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. The following proposals were listed:

- RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

The restrictions agreed for FR1 WA BS are also applicable for FR1 MR BS.

For a specific channel BW supported for SBFD, maximum of 2 specific UL subbands sizes are allowed to be declared.

The restriction on maximum number of specific UL subband sizes to be declared is also applicable to the FR1 MR BS.

The restriction on maximum number of specific DL subband sizes to be declared is also applicable to the FR1 MR BS.

For SBFD, at least one DL SB BW should be no less than (SSB BW + sync raster step size) and FR2 minimum CBW to support SBFD is 100MHz for DU/UD case and 200MHz for DUD case. The minimum bandwidth requirement for FR2 DUD case can be revisited depending on future RAN1 agremeents on SSB transmission on SBFD symbols.

# References

- R4-2419907 “Way Forward for [113][306] NR_duplex_evo_general”, RAN4#113, Orlando, Florida, USA, 18th-22nd November, 2024