3GPP TSG-RAN WG4 Meeting #114	R4-2501329

# Athens, GR, 17th – 21st Feburary, 2025

Agenda Item:	6.19.1

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN B252 UE RF requirements

Document for:	Approval

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to specify UE RF core requirements for fulfilling the urgent deployment demand.

In the contribution, the consideration for IoT-NTN B252 UE RF requirements is provided for discussion.

- Discussion

2.1		Spectrum WI scope for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new E-UTRA NTN FDD band for NB-IoT/eMTC NTN operation, with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario; - Support UE Categories NB1, NB2, M1; - Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. The specification work of this WI shall leverage the studies and requirements for NR/LTE NTN n256/B256, n255/B255, n254/B254, where applicable.

2.2		Previous Meeting Agreements

In RAN4#113, the approved WF [3] provided the guidance for following works. The partial contents are listed below for reference.

Issue 2-1-1: UE TX-RX Separation Agreement: - Specify the variable Tx-Rx frequency separation for Band B252 - Discuss the applicability of variable Tx-Rx frequency separation for other IoT NTN bands under other agendas.

In RAN4#112bis and RAN4#113, the approved WF [2][3] provided the guidance for following works. The partial contents are listed below for reference.

In RAN4#113 Issue 2-1-2: UE Maximum Output Power for Cat M1 and NB1/NB2 Agreement: - The MOP of this band will be aligned with the agreements in other Rel-19 NTN related WIs. In RAN4#112bis Issue 2-1-10: Out-of-band blocking for Cat-M1 - Agreement - FFS Issue 2-1-11: Out-of-band blocking for Cat-NB1/NB2 - Agreement - Further study whether same issue as B256 applies.

2.3		General

In Rel-19 WID [1], it was agreed to update TR 36.764 LTE IoT NTN operating bands for B252.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TR 36.764 | Support for IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) | RAN#107 (March 2025) | Core |

Because the existed TR 36.764 version is V18.2.0, to add new draft Rel-19 TR 36.764 is needed.

Observation 1: In Rel-19 WID [1], it was agreed to update TR 36.764 LTE IoT NTN operating bands for B252.

Proposal 1: Consider adding Rel-19 TR 36.764 B252 skeleton as provided below.

8 FDD band B252 (IoT-NTN FDD S_band) 8.1 Regulation 8.1.1 Regional Applicability 8.2 UE requirements 8.2.1 Channel arrangement 8.2.2 UE transmitter characteristics 8.2.2.1 Maximum output power 8.2.2.2 Emission requirements and NS values 8.2.2.2.1 Spurious emission 8.2.3 UE receiver characteristics 8.2.3.1 Reference sensitivity 8.2.3.2 Blocking requirements 8.2.4 A-MPR Evaluation 8.3 BS requirements

2.4		UE RF requirements

2.4.1	Tx-Rx frequency separation

Observation 2: In WF [3], the B252 was allowed to support variable TX-RX frequency separation.

Proposal 2: Discuss whether the requirements below could be used for supporting B252 variable TX-RX frequency separation.

NTN Cat-M1 Table 5.4A.3-1: Default UE TX-RX frequency separation E-UTRA Operating Band; TX – RX carrier centre frequency separation. 252; 80 MHz1 161.4 to 198.6 MHz2. NOTE 1: Default TX-RX separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3A.. Table 7.3A-4: TX – RX carrier centre frequency separation for REFSENS verification E-UTRA Operating Band; Channel bandwidth; TX – RX carrier centre frequency separation for REFSENS verification. 252; 1.4 MHz; 161.4 MHz, 198.6 MHz. NTN NB1/NB2 Table 5.4B.3-1: Default UE TX-RX frequency separation E-UTRA Operating Band; TX – RX carrier centre frequency separation. 252; 80 MHz1 160.2 to -199.8 MHz2. NOTE 1: Default Tx-Rx separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3B.. Table 7.3B-2: TX – RX carrier centre frequency separation for REFSENS verification E-UTRA Operating Band; Channel bandwidth; TX – RX carrier centre frequency separation for REFSENS verification. 252; 0.2 MHz; 160.2 MHz, 199.8 MHz.

2.4.2	Out-of-band Blocking for B252 for Cat-M1 and Cat-NB1/NB2

It is observed that B256 has different out-of-band blocking requirements because of using the same duplexer filter to cover B65 and B256.

Table 7.6A.3-2: Out of-band blocking for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 253, 2542, 255 | Finterferer (C`W) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 2: The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

Table 7.6B.3-1: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 253, 2545, 255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2562 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Void. NOTE 2: Band 256 lower frequency ranges are modified to enable specific implementations. NOTE 3: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz.” NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 5: The power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

Currently, the new duplexer filter would be used for B256 and B252.

Observation 3: It is expected that the same duplexer filter would be used for both B256 and B252.

Proposal 3: Discuss whether tables below could be used for band B252 out-of-band blocking requirements.

Table X: Out of-band blocking parameters for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 3: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

Table Y: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 2526 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 6: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

3 Conclusion

In this paper, our views on IoT-NTN B252 UE RF requirements are provided.

Observation 1: In Rel-19 WID [1], it was agreed to update TR 36.764 LTE IoT NTN operating bands for B252.

Proposal 1: Consider adding Rel-19 TR 36.764 B252 skeleton as provided below.

8 FDD band B252 (IoT-NTN FDD S_band) 8.1 Regulation 8.1.1 Regional Applicability 8.2 UE requirements 8.2.1 Channel arrangement 8.2.2 UE transmitter characteristics 8.2.2.1 Maximum output power 8.2.2.2 Emission requirements and NS values 8.2.2.2.1 Spurious emission 8.2.3 UE receiver characteristics 8.2.3.1 Reference sensitivity 8.2.3.2 Blocking requirements 8.2.4 A-MPR Evaluation 8.3 BS requirements

Observation 2: In WF [3], the B252 was allowed to support variable TX-RX frequency separation.

Proposal 2: Discuss whether the requirements below could be used for supporting B252 variable TX-RX frequency separation.

NTN Cat-M1 Table 5.4A.3-1: Default UE TX-RX frequency separation E-UTRA Operating Band; TX – RX carrier centre frequency separation. 252; 80 MHz1 161.4 to 198.6 MHz2. NOTE 1: Default TX-RX separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3A.. Table 7.3A-4: TX – RX carrier centre frequency separation for REFSENS verification E-UTRA Operating Band; Channel bandwidth; TX – RX carrier centre frequency separation for REFSENS verification. 252; 1.4 MHz; 161.4 MHz, 198.6 MHz. NTN NB1/NB2 Table 5.4B.3-1: Default UE TX-RX frequency separation E-UTRA Operating Band; TX – RX carrier centre frequency separation. 252; 80 MHz1 160.2 to -199.8 MHz2. NOTE 1: Default Tx-Rx separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3B.. Table 7.3B-2: TX – RX carrier centre frequency separation for REFSENS verification E-UTRA Operating Band; Channel bandwidth; TX – RX carrier centre frequency separation for REFSENS verification. 252; 0.2 MHz; 160.2 MHz, 199.8 MHz.

Observation 3: It is expected that the same duplexer filter would be used for both B256 and B252.

Proposal 3: Discuss whether tables below could be used for band B252 out-of-band blocking requirements.

Table X: Out of-band blocking parameters for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 3: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

Table Y: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -153 |
| 2526 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 6: Band 252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

5 Reference

- RP-241662, “New WID on Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- R4-2417206, “WF on NTN and IoT-NTN bands”, Inmarsat

- R4-2420465, “WF on requirements for NR IoT NTN bands”, Inmarsat