3GPP TSG-RAN WG4 Meeting #114	R4-2501400

Athens, Greece, 17th to 22nd  February 2025

Agenda Item:	7.10.2.1

Source:	Ericsson

Title: 		On Co-existence simulation assumption considerations for NR-NTN and NB-IoT-NTN HP UE

Document for:	Discussion

# 1	Introduction

In the RAN plenary meeting RAN#104, there was an agreement for a new Rel-19 RAN4 led WI Enhanced requirements and test methodology for NR and IoT NTN [1]. The baseline assumptions agreed in RAN4#113 meeting have been captured in WF [2].

This contribution intends to trigger further discussion on co-existence study conclusions focusing on adjacent channel interference between NTN and TN networks based on the Release – 17 NTN co-existence studies [3].

# 2	Co-existence simulation assumptions

### 2.1	NR NTN UE ACLR

In the previous RAN4 meeting, above observations were made from the co-existence results provided with isolation distance of 1.5km and it was encouraged for more companies to provide results with isolation distance of 0km.

Isolation distance of 1.5km between NTN UE and edge of TN cluster has been agreed as the baseline assumption.

The Table 6.5.2.4.1-2 from TS 38.101-1 [6] below depicts the ACLR values for NR UE for Power Classes 3, 2,1.5 and 1.

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

The NR ACLR value is specified as 31 dB for Power Class 2 and 1.5 and 37 dB for Power Class 1.

In our companion contribution [4], we presented our simulation results for Case 4 when NR-NTN/NB-IoT NTN are interfering TN UL for Power class 2,1.5 and 1 assuming isolation distance of 1.5 km and in [5] assuming isolation distance of 0 km. The ACIR requirements for 5%-tile have been summarized below -

Table 2.2-1: Summary of ACIR requirements for PC2, 1.5 and 1 with isolation distance of 1.5km and 0km

| ACIR (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Case 4 |  | PC 2 | PC 1.5 | PC 1 |
| With isolation distance of 1.5km (based on results presented in R4-2418995 [4]) | With wrap-around | GEO | NR NTN and TN BS with AAS | 11 | 13 | 15 |
|  |  |  | NR NTN and TN BS with Non-AAS | 3 | 6 | 8 |
|  |  |  | NB-IoT NTN | NA | 1 | 3 |
|  |  | LEO1200 | NR NTN and TN BS with AAS | 9 | 13 | 15 |
|  |  |  | NR NTN and TN BS with Non-AAS | 3 | 5 | 7 |
|  |  |  | NB-IoT NTN | 1 | 4 | 5 |
|  | Without wrap-around | GEO | NR NTN and TN BS with AAS | 3 | 7 | 9 |
|  |  |  | NR NTN and TN BS with Non-AAS | NA | 2 | 4 |
|  |  |  | NB-IoT NTN | NA | 2 | 4 |
|  |  | LEO1200 | NR NTN and TN BS with AAS | 6 | 9 | 11 |
|  |  |  | NR NTN and TN BS with Non-AAS | NA | 2 | 4 |
|  |  |  | NB-IoT NTN | NA | 0 | 2 |
| With isolation distance of 0km | With wrap-around | GEO | NR NTN and TN BS with AAS | 30 | 34 | 38 |
|  |  |  | NR NTN and TN BS with Non-AAS | 31 | 35 | 37 |
|  |  |  | NB-IoT NTN | 32 | 35 | 37 |
|  |  | LEO1200 | NR NTN and TN BS with AAS | 31 | 35 | 37 |
|  |  |  | NR NTN and TN BS with Non-AAS | 35 | 37 | 39 |
|  |  |  | NB-IoT NTN | 29 | 31 | 33 |
|  | Without wrap-around | GEO | NR NTN and TN BS with AAS | 33 | 37 | 39 |
|  |  |  | NR NTN and TN BS with Non-AAS | 31 | 34 | 36 |
|  |  |  | NB-IoT NTN | 27 | 31 | 33 |
|  |  | LEO1200 | NR NTN and TN BS with AAS | 34 | 37 | 39 |
|  |  |  | NR NTN and TN BS with Non-AAS | 32 | 35 | 37 |
|  |  |  | NB-IoT NTN | 28 | 32 | 34 |

The ACIR requirements for all combinations within Case 4 are greater than 30 dB for PC2 and PC1.5 and greater than 37 dB for PC1 when isolation distance assumption between the NTN UE and edge of TN cluster is 0km.

Based on the results presented in Table 2.2-1, the ACIR requirements are very stringent when isolation distance assumption is 0 km for both NR-NTN and IoT-NTN but are slightly comparable to TN values.

RAN4 to re-use TN ACLR requirements for PC 2, PC 1.5 and PC 1 even if lower ACIR requirements are observed from co-existence studies for the baseline assumption of 1.5km.

RAN4 to agree to only capture the simulation results with isolation distance of 0 km in TR as information but should not use to specify any ACLR requirements for NTN HPUE in TS.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Isolation distance of 1.5km between NTN UE and edge of TN cluster has been agreed as the baseline assumption.

Observation 2	The NR ACLR value is specified as 31 dB for Power Class 2 and 1.5 and 37 dB for Power Class 1.

Observation 3	The ACIR requirements for all combinations within Case 4 are greater than 30 dB for PC2 and PC1.5 and greater than 37 dB for PC1 when isolation distance assumption between the NTN UE and edge of TN cluster is 0km.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to re-use TN ACLR requirements for PC 2, PC 1.5 and PC 1 even if lower ACIR requirements are observed from co-existence studies for the baseline assumption of 1.5km.

Proposal 2	RAN4 to agree to only capture the simulation results with isolation distance of 0 km in TR as information but should not use to specify any ACLR requirements for NTN HPUE in TS.

# References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei)

- R4-2420403,” WF on NR NTN HPUE”, 3GPP RAN4#113, November 2024

- 3GPP TR 38.863 v18.3.0: “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects”

- R4-2418995,” On Co-existence simulation results for NR-NTN and NB-IoT-NTN HP UE”, Ericsson, 3GPP RAN4#113, November 2024

- R4-2501401, “On Co-existence simulation results for NR-NTN and NB-IoT-NTN HP UE”, Ericsson, 3GPP RAN4#114, February 2025

- 3GPP TS 38.101-1 V18.7.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”