3GPP TSG-RAN WG4 Meeting # 114															R4-2401411

Athens, Greece, 17th – 21st February, 2025

Agenda item:			7.1.1.3.4

Source:	Samsung

Title:	Views on SRS IL imbalance issue

Document for:	Discussion

1	Introduction

As one of objectives, ”6Rx for handheld and FWA UE” under Rel-19 WI on UE RF enhancements for NR FR1/FR2 and EN-DC (Phase 4) [1], the issue of insertion loss imbalance across SRS ports has been discussed to study whether the corresponding solution is required or not.

However, RAN4 is still struggling for the justification. Even though they spent much time to find out the solution for the issue from Rel-18, the same question and outcome have been being repeated during the last three quarters such as whether to solve SRS IL imbalance issue in Rel-19 or not [2-6]. In this contribution, we would like to continue sharing our views while trying to conclude this unproductive discussion.

2 Discussion

In the latest WF [6], it also has two parts as previous ones [2-5], one for seeking its original justification on whether to solve SRS IL imbalance issue in Rel-19, the other is to try a compromised solution with the least impact to RAN1, RAN2, and RAN4 specifications as below.

## Sub-topic 3-1: General considerations for SRS IL imbalance issue Issue 3-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution framework with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 3-2-1. Issue 3-1-2: Initial Considerations for SRS IL imbalance issue Way forward: RAN4 to focus on refining the compromised solution framework in Issue 3-2-1. ## Sub-topic 3-2: SRS IL imbalance issue solutions Issue 3-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: RAN4 to further discuss the candidate solution framework below and present proposals concerning the details. Companies are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed. Candidate solution framework: - UE performs self-compensation of SRS IL up to its configured maximum output power. - FFS on whether condition would introduce any additional RAN4 performance requirement. No RAN1 specification impact. - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting. - Others - Details of SRS IL self-compensation and assistance framework are FFS

However, since the two sub-topics are not mutually independent, which means that RAN4 first has to identify the issue of the IL imbalance across SRS ports clearly from the real network operations, the arguments have repeated without outcomes from Rel-18.

Observation 1:	RAN4 first has to identify the issue of the IL imbalance across SRS ports clearly from the real network operations.

Theoretically, such power imbalance between SRS antenna ports can be an issue as it could affect overall uplink and downlink performance due to the low SRS power of Rx antenna ports if single PA based antenna switching is conducted. This would be one motivation from Rel-18 to strive for the solution to compensate the imbalance between the ports. However, according to our prior study, we found that this kind of assumption or conclusion does not apply to real world as we have provided many times before. In short, we have got following facts from the measurement, which might lead the discussion not to having a solution for the SRS IL imbalance issue from the SRS AS in NR specifications.

Observation 2:	No performance degradation can be shown from SRS IL imbalance up to 16 dB in real networks.

Observation 3:	Throughput loss is negligible even if 26 dB imbalance gap between SRS port (up to 0.5 dB) in real networks.

Proposal 1:		Following facts from the measurement, it is proposed not to strive for a solution for the SRS IL imbalance issue from the SRS AS in NR specifications.

Moreover, even though RAN4 can compromise a solution with the least and simplest method in the end, in our understanding, it is still doubtful that how network can acknowledge the SRS power gap whether it is from IL imbalance or from others, and how UEs can estimate the insertion loss value dynamically as it is not from their intention. This is because the gap between SRS ports is more dependent on the channel fading, hand grip loss, or other issues than IL. These fundamental issues cannot be removed since many other underlying factors like path differences between antennas and LPAF (LNA/PA/Filter/switch) are still there affecting the imbalance even though some IL imbalance related reporting schemes or capabilities are introduced expecting the better network behavior. It should also be noted that the SRS power headroom reporting for an activated serving cell, i.e., Type 3 PHR, can be happened only if the UE is not configured for PUSCH transmissions on the serving cell, which mean that the SRS power headroom reporting with the imbalance information for the network would not happen directly nor dynamically as specified in TS 38.213.

Observation 4:	The gap between SRS ports is more dependent on the channel fading, hand grip loss, or other issues than IL.

Observation 5:	SRS power headroom reporting, i.e., Type 3 PHR, with the imbalance information for the network would not happen directly nor dynamically.

Then, it would give the same question about the purpose of the new capability if it cannot make any change of the SRS imbalance issue. The potential solution with a new UE capability for IL imbalance reporting would not help improving accuracy of the downlink channel estimation. In our view, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

Observation 6:	Introducing new capability for IL imbalance reporting would not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

Nevertheless, we understand that RAN4 is now in a deadlock and it is time to close the time-consuming discussion with a clear conclusion. Even if RAN4 needs a solution for a specification work, the impacts to RAN1, RAN2, and RAN4 specifications should be nothing or marginal given the limited timeline for the WI. In order for that possible, it is recommended to consider setting a deadline for proponent companies to provide expected benefit/performance gains and draft CRs to check any impacts to RAN1, RAN2, and RAN4 specifications as noted in [6].

Proposal 2:		Even if RAN4 needs a solution for a specification work, the impacts to RAN1, RAN2, and RAN4 specifications should be nothing or marginal given the limited timeline for the WI.

Proposal 3:		It is recommended to set a deadline for proponent companies to provide expected benefit/performance gains and draft CRs as noted in the WF.

3	Conclusion

In this contribution, we continued to provide our analysis and view on the proposed solutions. It is also proposed a possible way for RAN4 at this stage. Following observations and proposals are provided to be considered by this meeting.

Observation 1:	RAN4 first has to identify the issue of the IL imbalance across SRS ports clearly from the real network operations.

Observation 2:	No performance degradation can be shown from SRS IL imbalance up to 16 dB in real networks.

Observation 3:	Throughput loss is negligible even if 26 dB imbalance gap between SRS port (up to 0.5 dB) in real networks.

Proposal 1:		Following facts from the measurement, it is proposed not to strive for a solution for the SRS IL imbalance issue from the SRS AS in NR specifications.

Observation 4:	The gap between SRS ports is more dependent on the channel fading, hand grip loss, or other issues than IL.

Observation 5:	SRS power headroom reporting, i.e., Type 3 PHR, with the imbalance information for the network would not happen directly nor dynamically.

Observation 6:	Introducing new capability for IL imbalance reporting would not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

Proposal 2:		Even if RAN4 needs a solution for a specification work, the impacts to RAN1, RAN2, and RAN4 specifications should be nothing or marginal given the limited timeline for the WI.

Proposal 3:		It is recommended to set a deadline for proponent companies to provide expected benefit/performance gains and draft CRs as noted in the WF.

Reference

[1] RP-241656	New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4

[2] R4-2406585	WF on requirements for 6Rx		RAN4#110-bis

[3] R4-2410751	WF on requirements for 6Rx		RAN4#111

[4]	R4-2414285	WF on 6Rx UE requirements		RAN4#112

[5]	R4-2417209	WF on 6Rx UE RF requirements	RAN4#112-bis

[6]	R4-2420376	WF on 6Rx requirements			RAN4#113