3GPP TSG-RAN WG4 Meeting #114	R4-2501416

# Athens, Greece, 17 Feb. – 21 Feb., 2025

Source: 	vivo

Title: 	Further consideration on RRM impacts for SBFD operation

Agenda Item:			7.23.3.2

Document for:	   Discussion

- Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

The updated objective in [1] which are related to RRM are copied below for reference:

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

  - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

    - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - PUSCH resource mapping, i.e., rate-matching around the muted REs

    - UCI resource determination in symbols with muted REs

  - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] (Reuse major principles)

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

    - At least aperiodic reporting

    - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - CLI measurement accuracy requirement

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In this contribution, we provide our considerations on RRM requirements for SBFD.

- Discussion

This WI has been discussed at RAN4 112 and RAN4 112bis meeting. At previous meeting, based on the WF [2] and topic summary [3], the following agreements are available:

Issue 1-2-1: Requirements for SSB based measurement

- Agreements (from online session)

  - For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

  - This can be revisited if it proves to be conflicting with further RAN1 agreements.

Issue 1-2-2: Requirements for CSI-RS based L1 measurement

- Agreements (from online session)

  - For CSI-RS colliding with UL transmission

    - For aperiodic CSI-RS resource, existing measurement requirements apply, i.e. the measurement is prioritized over semi-statically configured UL transmission.

    - For periodic and semi-persistent CSI-RS resource, RAN4 will send RAN1 LS for clarification if needed.

  - For CSI-RS puncturing in frequency domain in SBFD symbols,

    - For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs.

    - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold.

      - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs.

      - FFS for the DUD configuration.

    - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions.

    - For CSI-RS on SBFD symbols, no accuracy requirements apply if the usable number of CSI-RS PRBs after applying the puncturing operation is below the existing threshold.

    - FFS the measurement behavior for DUD configuration.

    - Above bullets apply to L1-RSRP/SINR measurement and also RLM/BFD/CBD measurement.

  - For CSI-RS distributed over SBFD and non-SBFD symbols,

    - Ask RAN1 clarification on whether related RAN1 agreement applies for L1 measurement (L1-RSRP/SINR measurement and RLM/BFD/CBD measurement). Further discuss the wording when drafting the LS.

Issue 1-2-3: Requirements for CSI-RS based L3 measurement

- Agreements (from online session)

  - Companies to check the existing requirement, and discuss whether they can be applied to SBFD operation and discuss any additional aspect needs to be addressed.

Issue 1-2-4: Requirements for scheduling restriction

- Agreements (from Wed AH session)

  - For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction.

  - Further discuss the impact to inter-frequency SSB measurement without gap case.

In this contribution we discuss the following open issues.

#### Issue 1-2-2: Requirements for CSI-RS based L1 measurement

- Agreements (from online session)

  - For CSI-RS puncturing in frequency domain in SBFD symbols,

    - For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (for the DU configuration) is no less than the existing threshold.

      - Note: in existing requirements, CSI-RS based RLM/BFD/CBD requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. CSI-RS based L1-RSRP/L1-SINR requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 48 RBs.

      - FFS for the DUD configuration.

    - For CSI-RS on SBFD symbols, if the CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, FFS whether existing accuracy requirements for CSI-RS based measurements can apply and related additional side conditions.

    - FFS the measurement behavior for DUD configuration.

For issue 1-2-2, for the DUD configuration, whether the existing accuracy requirements for CSI-RS based measurement will apply or not under particular scenarios is still not clear. For the DUD configuration, the following scenarios can be identified:

A. One part of downlink CSI-RS resource bandwidth (consecutive CSI-RS PRBs) satisfies the threshold for the number of consecutive PRBs of requirement and the other part does not satisfy.

B. Both parts of downlink CSI-RS resource bandwidth (consecutive CSI-RS PRBs) are less than the threshold for the number of consecutive PRBs of the requirements however the total bandwidth (number of PRBs) across two downlink satisfy the threshold for CSI-RS accuracy requirements.

To determine whether the accuracy for CSI-RS is applicable for scenario A or B, how to handle the CSI-RS resource across two subbands also needs be clarified.

For the scenario A, since the CSI-RS resources at least within one particular subband satisfies the threshold for the number of consecutive PRBs of the CSI-RS accuracy requirements, the CSI-RS accuracy requirements should apply. Regarding how to handle the CSI-RS resources in the other subband which have less consecutive CSI-RS PRBs compared with the threshold of the CSI-RS accuracy requirements, it is preferred to be determined by UE implementation. To our understanding the accuracy represents the overall impact by the channel and noise and even two groups of PRBs are disjoint in the frequency domain, these PRBs can still be jointly used in accuracy calculation.

Proposal 1: In DUD configuration, when one part of downlink CSI-RS resource bandwidth (consecutive CSI-RS PRBs) satisfies the threshold for the number of consecutive PRBs of requirement and the other part does not satisfy, CSI-RS accuracy will apply for this scenario.

For the scenario B, following former analysis, the CSI-RS accuracy will apply in principle. However considering the scenario where very few consecutive CSI-RS PRBs in one particular subband which may lead deteriorate accuracy, a lower bounds could be defined for this scenario and when the number of consecutive PRBs in one subband is less than this lower bound, the CSI-RS accuracy requirement will not apply.

Proposal 2: When consecutive CSI-RS PRBs of both subbands are less than the threshold of the CSI-RS accuracy requirements however the total number of consecutive PRBs across two downlink subbands satisfy that threshold, a lower bound could be defined and when the number of consecutive PRBs in each of the subband is equal or larger than this lower bound, the CSI-RS accuracy requirement will apply.

In addition when UE is processing two disjoint DL subbands, the UE processing capability should be considered. This issue has been discussed at RAN1 and the following agreement is available.

It is suggested to follow the principle of RAN1’s agreement and define separate UE capabilities to account for the difference of UE processing capability.

Proposal 3: When UE performs CSI-RS measurement across two disjoint DL subbands, suggested to follow the principle of RAN1’s agreement and define separate UE capabilities to account for the difference of UE processing capability.

#### Issue 1-2-3: Requirements for CSI-RS based L3 measurement

- Agreements (from online session)

  - Companies to check the existing requirement, and discuss whether they can be applied to SBFD operation and discuss any additional aspect needs to be addressed.

For the CSI-RS layer 3 measurement, one issue is UE is not aware of whether there is any SBFD configuration on the neighbor cell and what is the exactly SBFD configuration of the neighbor cell. This may impact the CSI-RS L3 measurement requirements.

For the CSI-RS L3 inter-frequency measurement, since it is only performed within the gap, the operations of SBFD, such as DL/UL collision, CSI-RS puncturing in frequency domain in SBFD symbols may not have impact on CSI-RS L3 inter-frequency measurement. For CSI-RS distributed over SBFD and non-SBFD symbols, since the UE cannot differentiate the SBFD symbol or non-SBFD symbol in the neighbour cell, the CSI-RS L3 inter-frequency measurement can be performed as the legacy, i.e., assuming non-SBFD symbol in the gap. Alternatively waiting for more RAN1 input on this issue.

Proposal 4: The CSI-RS L3 inter-frequency measurement can be performed as that of the legacy. Or more RAN1 input is needed for the SBFD operation where CSI-RS are distributed over SBFD and non-SBFD symbols.

The detailed CSI-RS L3 inter-frequency measurement requirements are copied below:

T CSI-RS_identify_inter = (TPSS/SSS_sync + T CSI-RS_measurement_period_inter + TCSI-RS_SFN_inter) ms

TPSS/SSS_sync

T CSI-RS_measurement_period_inter

To our understanding, all factors in the above table could be independent of the SBFD operation. In general the CSI-RS L3 inter-frequency requirements could be reused if the issue in proposal 4 is confirmed.

For the CSI-RS L3 intra-frequency measurement, it will be performed without gap. One of the conditions where the CSI-RS based can be defined as the intra-frequency measurement is the following:

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement

To our understanding, for SBFD operation, no matter with DU or DUD configuration, the center frequency of CSI-RS resource for serving cell is not impacted and the former conditions need not update.

For CSI-RS puncturing in frequency domain in SBFD symbols, it was agreed that “For CSI-RS on SBFD symbols, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs”, to our understanding this applies for the serving cell CSI-RS measurement. For the neighbour cell CSI-RS L3 intra-frequency measurement, if the same principle is used, some of CSI-RS resource for measurement will be punctured as well. The current requirement for CSI-RS based intra-frequency measurement requirement requires PRBs ≥ 48. Hence similar to the serving cell L1 CSI-RS measurement, the applicability of neighbour cell CSI-RS L3 intra-frequency measurement needs be check and related issues in 1-2-2 needs be addressed as well.

Table 9.10.2.5-1: Measurement period for intrafrequency CSI-RS based measurements without gaps(FR1)

| DRX cycle | T CSI-RS_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp_CSI-RS) x CSI-RS period) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp_CSI-RS) x max(CSI-RS period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp_CSI-RS) x DRX cycle x CSSFintra |
| NOTE 1: The requirements apply assuming CSI-RS configuration with {D=3 with PRBs ≥ 48}. D is frequency domain density for the 1-port CSI-RS for L3 mobility defined in clause 7.4.1 of TS38.211 [6]. |  |

Proposal 5: For CSI-RS puncturing in frequency domain in SBFD symbols, the neighbour cell CSI-RS L3 intra-frequency measurement will be impacted. The related issues in 1-2-2 needs be addressed for CSI-RS L3 intra-frequency measurement, i.e., the number required for consecutive CSI-RS PRBs when the requirement is applicable; if the consecutive CSI-RS PRBs is satisfied however it is split across the two subbands.

For CSI-RS distributed over SBFD and non-SBFD symbols, since the UE does not know whether the neighbour cell has any SBFD configuration nor the detail SBFD configuration if it is configured. Hence even with the following RAN1 agreement, it is still unclear how to perform L3 intra-frequency measurement hence it is suggested that the CSI-RS based L3 intra-frequency measurement could be deprioritized.

Agreement #118b

For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

Proposal 6: The CSI-RS based L3 intra-frequency measurement could be deprioritized.

#### Issue 1-2-4: Requirements for scheduling restriction

- Agreements (from Wed AH session)

  - For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction.

  - Further discuss the impact to inter-frequency SSB measurement without gap case.

For the scheduling restriction due to DL/UL transition, the RAN1 116 agreement on collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol for SBFD-aware UEs is sufficient and there is no necessity for RAN4 to update current scheduling restriction. There is also no new impact from the latest RAN1 119 agreements.

Regarding the second bullet of issue 1-2-4, the issue is for the UE support Rel-16 inter-frequency measurement without gap, the neighbour cell SSB can be measured within the DL subband and within the active BWP or outside DL subband however still within the active BWP. For the first scenario where actually the neighbour cell SSB is completely contained within the DL subband, existing scheduling restriction is sufficient and no extra update is needed. However when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed and the scheduling restriction in section 9.3.9.4 can be used as a reference. In addition, UE is also not expected to have reception on data outside DL subband on SBFD symbol.

Proposal 7: For the UE support Rel-16 inter-frequency measurement without gap, when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed and the scheduling restriction in section 9.3.9.4 can be used as a reference.

#### Issue 1-2-5: Requirements for RACH requirements

- Recommended WF

  - Further discuss the issue.

#### Issue 1-2-6: Requirements for UL PC and or spatial relation update

- Recommended WF

  - Interested companies are encouraged to bring proposals on exact RRM impacts, if any

#### Issue 1-2-8: Requirements for UL resource muting

- Recommended WF

  - Interested companies are encouraged to bring proposals on exact RRM impacts, if any

For the impact of the SBFD operation on RACH requirements, UL PC and or spatial relation update, UL resource muting, no impact are identified for these issues.

Proposal 8: For the impact of the SBFD operation on RACH requirements, UL PC and or spatial relation update, UL resource muting, no impact is identified for these issues.

- Conclusions

In this contribution we provided our initial considerations on RRM requirements for SBFD and have the following proposals:

Proposal 1: In DUD configuration, when one part of downlink CSI-RS resource bandwidth (consecutive CSI-RS PRBs) satisfies the threshold for the number of consecutive PRBs of requirement and the other part does not satisfy, CSI-RS accuracy will apply for this scenario.

Proposal 2: When consecutive CSI-RS PRBs of both subbands are less than the threshold of the CSI-RS accuracy requirements however the total number of consecutive PRBs across two downlink subbands satisfy that threshold, a lower bound could be defined and when the number of consecutive PRBs in each of the subband is equal or larger than this lower bound, the CSI-RS accuracy requirement will apply.

Proposal 3: When UE performs CSI-RS measurement across two disjoint DL subbands, suggested to follow the principle of RAN1’s agreement and define separate UE capabilities to account for the difference of UE processing capability.

Proposal 4: The CSI-RS L3 inter-frequency measurement can be performed as that of the legacy. Or more RAN1 input is needed for the SBFD operation where CSI-RS are distributed over SBFD and non-SBFD symbols.

Proposal 5: For CSI-RS puncturing in frequency domain in SBFD symbols, the neighbour cell CSI-RS L3 intra-frequency measurement will be impacted. The related issues in 1-2-2 needs be addressed for CSI-RS L3 intra-frequency measurement, i.e., the number required for consecutive CSI-RS PRBs when the requirement is applicable; if the consecutive CSI-RS PRBs is satisfied however it is split across the two subbands.

Proposal 6: The CSI-RS based L3 intra-frequency measurement could be deprioritized.

Proposal 7: For the UE support Rel-16 inter-frequency measurement without gap, when the neighbor cell SSB is outside DL subband however still within the active BWP, update on existing scheduling restriction is needed and the scheduling restriction in section 9.3.9.4 can be used as a reference.

Proposal 8: For the impact of the SBFD operation on RACH requirements, UL PC and or spatial relation update, UL resource muting, no impact is identified for these issues.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2420097, WF on RRM requirements for NR_duplex_evo, RAN4 113, Huawei

[3] R4-2418281, Topic summary for [113][223] NR_duplex_evo, Huawei, RAN4 113